Code No.210851

OR

## II-B.Tech I-Semester Supplementary Examinations May/June 2004

## MECHANICS OF SOLIDS

(Common to Chemical Engineering and Metallurgy and Material Technology)

# Time: 3 hours							Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Define young’s modulus, shear modulus and Bulk modulus. Derive the relation between them.

b)	The following data refer to a tensile test conducted on a mild steel bar :

diameter of the  specimen = 20 mm

length of the specimen = 200 mm

extension at a load of 40 kN = 0.12mm

Load at yield point = 80kN

Maximum load = 150 kN

Total extension = 50 mm

Neck diameter = 15 mm.

Determine (i)  young’s modulus (ii) yield stress (iii) ultimate stress                            (iv) percentage elongation and (v) percentage reduction in area.

- Draw B.M. diagram, S.F. diagram and axial thrust diagram for the frame shown in the figure below.

3.a)	State the assumptions made in the theory of simple bending.

b)	A simply  supported beam of 10m span is subjected to a central concentrated load P. The section is a T-section  of flange 250mm x 25mm and rib 250mm x 25mm.. If the compressive and tensile stresses in the beam are not to exceed 100 N/mm2   and 120 N/mm2 respectively, determine  the maximum force P that can be applied on the beam.

Contd…2.

Code No.210851				-2-				OR

4.a)	Derive an expression for the intensity of shearing stress at any point in the          cross-section of a loaded beam.

b)	A beam of circular section is 200mm diameter. At a particular section, the beam is subjected to a shear force of 10 kN. Find the maximum shear stress.

5.	A beam of length 6m is simply supported at the ends. It carries a uniformly distributed load of 4 kN/m over a length of 2 metres from the left end. Find the maximum deflection of the beam.

Take E = 2 x 105 N/mm2  and I = 2 x 107 mm4.

6.	A cylindrical shell of 200 mm diameter and 1 metre length is filled with a fluid at atmospheric pressure. The wall thickness is 8mm. If  an additional 2 x 104 mm3 of the fluid is pumped into the cylinder, find the pressure exerted by the fluid on the wall of the cylinder.  Find also the hoop stress induced. E = 2 x 105 N/mm2 ; Poissson’s  ratio = 0.3.

7.a)	Show that in a strained material subjected to two dimensional stress, the sum of  normal components of stress on any two mutually perpendicular planes is constant.

b)	A  point in a strained material is subjected to mutually perpendicular stresses of 600 N/mm2 tensile and 400 N/mm2  compressive. It is also subjected to a shear stress of 100 N/mm2.  Draw Mohr’s circle and find the principal stresses and maximum shear stress.

8.	A hollow shaft of internal diameter 3/8th of external diameter is required to transmit 600 KW at 200 rpm, the maximum torque being 20% greater than the mean. If the maximum shear stress is to be limited to 65 N/mm2 and the twist in a length of 4m is not to exceed 2o , determine the external diameter of the shaft.

Take  G = 0.8 x 105 N/mm2.

-x-