Code No: OR-210452.

OR

II-B.Tech. I-Semester Supplementary Examinations, May/June-2004

NETWORK THEORY

(Common to Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Bio medical Engineering, Electronics and Control Engineering)

# Time: 3 Hours							Max. Marks: 70

## Answer any FIVE questions

## All questions carry equal marks

---

1.a)	Explain the properties of different Voltage and current sources.

b)	Find the power delivered by the 5V Voltage source in the network shown below.

2.a)	Explain  i)  Reciprocity theorem.

ii)  Tellegan’s theorem.

- Calculate the current in the 2 resistor using superposition theorem.

3. a)	Explain why current becomes maximum in series RLC circuit at the condition of resonance.

b)	An inductance of  0.5H, a resistance of 5, and a capacitance of 8F are in series across a 220V a.c. supply.  Calculate the frequency at which the current flowing through the circuit becomes maximum. Also find Band width, half power frequencies, and Voltage across capacitance at resonance.

(Contd…2)

Code No.OR-210452.                            - 2 -                                                              OR

4.a)	Explain the concept of transients in circuits?

- Obtain the node Voltage equations for the network shown below.

5.a)	Define Laplace transformation.

b)	Explain the advantages of Laplace transformation.

c)	Find L-1 10s(s+1)(s+2)

6.a)	Define  i)  Graph  ii)  Subgraph   iii)  A path   iv)  Degree of Vertex   v)  Loop

vi)  Connected graph     vii)  Oriented graph    viii)  Plannar graph

b)	Obtain the incidence matrix for the graph shown below.  Take node 4 as reference node.

7.a)	Obtain the relation between Y and Z parameters.

b)	Obtain ‘h’ parameters of the network shown.

8.	Write short notes on:

a)  Source transformation.

b)  Q-factor on series resonance.

c)  Two-port network.

???