Code No: 220451

OR

II B.Tech II-Semester Supplementary Examinations, May 2004

SWTICHING THEORY AND LOGIC DESIGN

(Common to Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Electronics Control Engineering and Electronics and Telematics)

Time: 3 hours							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1. a)	Devise a single error correcting code for a 11 bit group 01101110101

b)	Test the following Hamming code sequence for 11 bit message and correct it if necessary (1010010111  01011).

2. a)	Simplify the Boolean Expression (i) X[Y + Z[(XY + XZ)] (ii) Obtain the simplified expression in SOP form of x’z’ +y’z’ +yz’ +xyz.

b)	Obtain the simplified expression in product of sums

F(A, B, C,D) =m (5, 6, 7, 8, 9, 12, 13, 14,).

3. 	For the given expression use tabular method f(v,w,x,y,z) =  m(0,4,12,16,19,24,27,28,29,31) in SOP form and draw the circuit using only NAND gates.

4.  a)	Give  the  circuit  implementation of a 4-bit carry look-ahead adder.

b)	Give the implementation of a 1-bit magnitude comparator.

c)	Bring out the differences among a PAL and PLA.

5. a)	Analyze the circuit given and prove it is equivalent to a T flip flop.

b)	Draw the circuit diagram of a mod-10 ripple counter and explain its operation with the aid of output state timing diagram.

6. 	Design a sequential circuit with two D flipflops A and B and one input x. When x=0, the state of the circuit remains the same. When x=1, the circuit goes through the state transitions from 00 to 01 to 11 to 10 back to 00 and repeats.

Contd…2

Code No: 220451				.2.					OR

7.	For the machine given below, find the equivalence partition and a corresponding reduced machine in standard form and also explain the procedure:

| PS | NS, Z |  |
| --- | --- | --- |
|  | X=0 | X=1 |
| A | B, 0 | E, 0 |
| B | E, 0 | D, 0 |
| C | D, 1 | A, 0 |
| D | C, 1 | E, 0 |
| E | B, 0 | D, 0 |

8.	Construct an ASM block that has 3 input variables (A, B, C), 4 output       (W,X,Y,Z) and 2 exit paths.  For this block, output Z is always 1, and W is 1 if   A and B are both 1.  If C=1 and A=0, Y=1 and exit path 1 is taken.  If C=0 or A=1, X=1 and exit path 2 is taken. Realize the above using the multiplier and register.

###