Code No: 221852

OR

## II-B.Tech II-Semester Supplementary Examinations April/May 2004

## THERMODYNAMICS AND KINEMATICS

(Metallurgy and Material Technology)

# Time: 3 hours							Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What are different types of systems? Explain with examples.

- State and explain the Zeroth law of thermodynamics.

- How the Zeroth law is helpful in temperature measurement?

2.a)	Show that the internal energy is function of temperature only?

- How the Joules Thompson’s coefficient can be estimated experimentally?

- Calculate the work transfer, heat transfer and change in internal energy if the gas is changing its state from 1 bar, 27oC to 20 bar, 400oC polytropically.

3.a)	What are the limitations of first law of thermodynamics?

b)	Five moles of an ideal gas whose Cr = 12 kJ/kmA k initially at 15 bar and 300 k expands adiabaticaly to a pressure of 2 bar. Calculate change in entropy.

4.a)	Show that equilance of Kelvin Planck’s statement with Claussius statement of second law of thermodynamics?

b)	A reversible heat engine operates between three heat reservoirs A, B and C. The heat received by the engine from each reservoir A and B is same and at temperature TA and TB respectively and the engine rejects heat to reservoir C at temperature Tc. If the engine efficiency K times of the efficiency of reversible engine operating between reservoirs A and C only, then show that

TBTA=12(1−K)TATC+2k−1

5.a)	Derive the Gibbs free energy equation from first principles?

- Discuss the use of third law of thermodynamics in evaluation of ∆So of a reaction.

6.a)	Define the terms fugacity, activity and equilibrium constant.

- Discuss the importance of Sigma function in evaluation of fugacity?

7.a)	Derive the half life for Zero order and first order reactions.

- Explain unimolecular reactions with examples?

8.	Write short notes on:

- Catalystic reactions.   b) Claussius Clayperon equation.

- Maxwell’s relations. d)  Entropy and disorder. e) Hess law of heat of reaction.

***     ***     ***