Code No.:  NR-221901

Set No.

1

II B.Tech II Semester Supplementary Examinations, April/May 2004

DATA COMMUNICATION

(Electronics and Computer Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	If the input data to differential binary PSK (DBPSK) is 1011 1000 1101 and reference bit is zero compute XNOR output and its output phase and then compute DBPSK input phase and the corresponding recovered bit stream.

2.	What are the disadvantages of star topology?

3.	Explain the different types of error detection methods with suitable examples.

4.a)	What is acoustic coupling?

b)  	What are the virtues and drawbacks of acoustic coupling for modems?

c)	What is a direct connect modem?  How does it work?  What are the advantages?

5.	Briefly explain the following Framing Technique:

- added-digit framing

- robbed-digit framing

- added-channel framing

- statistical framing

(e) unique-line code framing.

6.a)	Define Multiplexing & describe Time Division Multiplexing with Block Diagram.

b)	A PCM-TDM system multiplexes 24 voice band channels.  Each sample is encoded into 7 bits and a framing bit is added to each frame.  The sampling rate is 9000 samples / second.  BPRZAMI encoding format is used.  Determine              (i)  Line speed in BPS (ii)  Min Nyquist Band Width.

7.a)	Draw the block diagram of Data Above Voice and explain.

b)	Draw the frequency response of above system.

8.	Give in detail, the basic principles of polling and selection on a multipoint line.

- - -

Code No.:  NR-221901

Set No.

2

II B.Tech II Semester Supplementary Examinations, April/May 2004

DATA COMMUNICATION

(Electronics and Computer Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	If the input data to differential binary PSK (DBPSK) is 0011 0011 0101 01 and reference bit is zero compute XNOR output and the output phase and then compute DBPSK input phase and the corresponding recovered bit stream.

2.	What are the advantages of star topology?

3.	Draw the block diagram of USART and explain its operation.

4.a)	Explain the operation of Intelligent  modems.

b) 	Discuss about burst error correction.

5.	Write short notes on:

(a) Multi link procedure

(b) information transfer

(c) balanced asynchronous class.

6.a)	Write short notes on frequency corelator.

- Calculate the frequency  range at MG,combining network for the following assignment

GP	SG	MG

3	13	2

5	D25	3

1	15	1

2	17	2

7.a)	Explain about Data Above Video system.

b)	Draw the block diagram of Data Above Video system.

8.	Briefly explain half duplex multipoint operation with suitable examples.

- - -

Code No.:  NR-221901

Set No.

3

II B.Tech II Semester Supplementary Examinations, April/May 2004

DATA COMMUNICATION

(Electronics and Computer Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	If the input data to differential binary PSK (DBPSK) is 1100 1100 1010 10 and the reference bit is a logic 1 compute XNOR output and its output phase and then compute DBPSK input phase and the corresponding recovered bit stream.

2.	What are the advantages of star topology?

3.	Explain the working of UART with neat block diagram.

4.	Define the following terms:

- AT commands in modem

- Signal constellation

- Dial-up line

- Equalization.

5.a)	Describe the history of SDLC.

b)	Discuss the different types and topologies of SDLC.

6.a)	Compare regenerative repeater and analog repeater.

b)   	A binary digital communication system transmits data over a wireless channel of

length 1000 Km.  Repeaters are used every 10 Km to offset the effect of channel attenuation.  Determine the b/N0. Ratio of the energy in the transmitted signal to the output noise that is required to achieve a probability of a bit error of 10-5.  If (a) Analog repeaters are employed.  (b).  If regenerative repeaters are employed

7.a)	What is meant by Data on Voice?

b)	Draw the frequency response of Data Under Voice.

8.	Discuss point –to- point and multi drop configurations with necessary diagrams.

- - -

Code No.:  NR-221901

Set No.

4

II B.Tech II Semester Supplementary Examinations, April/May 2004

DATA COMMUNICATION

(Electronics and Computer Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	For the QPSK modulator in Figure 2 determine the I and Q bits for an input of   a= + sinwct- coswct. Explain the functioning of this QPSK modulator.

2.	What is the purpose of the Hamming code?

3.	Write short notes on the following:

- CRT terminals

- PC terminals

4.a)	What are the characteristics of a serial interface?

b)  	Define Baud  rate.

5.	Give comparison between BISYNC, SDLC,  and HDLC protocols.

6.	Draw the biphase, miller and dicode line encoding format for the data 11010100011 and sumarise their general characteristics.

7.a)	What is the difference between a DUV and a DAV network?

b)	What are the four primary types of hybrid data network?

8.	What are the advantages of a multipoint connection over a point- to- point       connection?

- - -