Code No.OR-220452

OR

## II-B.Tech II-Semester Supplementary Examinations April/May 2004

## ELECTRONIC CIRCUITS

(Common to Electronics & Communication Engineering, Electronics & Instrumentation Engineering, Biomedical Engineering, Electronics and Control Engineering, Electronics and Telematics)

# Time: 3 hours							Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What do you understand by ‘thermal runaway’ in a junction transistor? Is this phenomenon present in JFET? Justify.

b)	A germanium transistor is used in the self-bias circuit, with VCC = 16V,                 RC = 1.5k. The Q-point is chosen to be VCE = 8v and  Ic = 4mA. A stability factor of S=12 is desired. If   = 50, find R1, R2 and RE.

2.a)	Draw the low frequency ‘h’ parameter equivalent circuit of a CE  transistor and explain the significance of each parameter.

b)	A CE  transistor amplifier is biased using collector  to base bias. The values of the components in the circuit are RS = 10 k, RF = 200 k, RC = 10K, and                     RE = 100Ω. Calculate Ai, Av , Avs and Ri. Transistor parameters are as follows:  hie = 1.1 k, hre = 2.5 x 10-4, hfe = 50 , hoe = 25 A/v.

3.a)	Draw the h-parameter equivalent circuit for a typical common emittes  amplifier and derive expressions for Ai, Ri, AV and Ro .

b)	Explain, how h-parameters can be obtained from the transistor characteristics.

4.a)	Derive the expression for the CE short circuit current gain Ai as  a function of frequency.

b)	A BJT has the following parameters measured at Ic = 1 mA, hie = 3 k, hfe = 100  fT = 4 MHz , Cc = 2PF., and Ce= 18PF . Find  r b'e , rbb'  , gm and fH for  RL = 1 k.

5.a)	A common drain amplifier shown in fig. has gm = 6000 S, rd = 70 k. Calculate Zi, Zo and Av

Fig.

contd….2.

Code No.OR-220452				-2-				OR

b)	Why self bias is not suitable for depletion type and enhancement type MOSFET?

6.a)	What are the advantages and disadvantages of negative feedback? Explain .

b)	Derive expressions for Av , Zi and Zo  of voltage series feedback amplifier

7.a)	Explain the principle and working of  Rc phase shift oscillator.

b)	Derive an expression for the frequency of oscillation of wien-bridge oscillator.

8.	Write short notes on any THREE

(a)   Thermal stability	(b)  Emitter follower  (c)  Hybrid –π model

(b)   Barkhassen conditions.

-x-