Code No: OR-220254

OR

II-B.Tech. II-Semester Supplementary Examinations, April/May-2004

ELECTRICAL CIRCUITS-II

(Electrical and Electronics Engineering)

# Time: 3 Hours							Max. Marks: 70

## Answer any FIVE questions

## All questions carry equal marks

---

1.a)	Explain the two waltmeter method of measuring power in 3  S systems (star and delta connected).

b)	Three impedances each of 3-J4 is connected as shown in figure across a 3 s  230 v banced supply.  Calculate the line and phase currents in the  connected load and the power delivered to the load.

2.a)	What do you mean by graph of a network?  Explain using an example.

b)	Explain the method of obtaining Tie-set matrix of a network.

3.a)	State and explain Thevenin’s theorem.

b)	Find the current I in the circuit using Norton’s theorem.

4.a)	Obtain the equivalent inductance of two coupled coils connected in

(i) series aiding (ii) series opposing.

b)	Two coils A and B with negligible resistance have self induction 0.2 H and 0.1 H respectively are connected in series.  The mutual inductance between the coils is 0.1 H.  Find the equivalent inductance of the series circuit for (i) cummulative coupling (ii) differential coupling.

Contd….2.

### Code No: OR-220254				:: 2 ::				OR

5.	Discuss in detail about ABCD and hybrid parameters of two-port networks.

6.	Find the voltage across R in the circuit shown using mesh analysis.

7.a)	Discuss briefly about three phase unbalanced systems.

b)	The unbalanced star connected load shown in figure has balanced voltages of   100 V and acb sequence.  Calculate the line currents and neutral currents.

Take ZA = 15 , ZB = 10 + J5 , ZC = 6-J 8 .

8.a)	State and explain maximum power transfer theorem.

b)	Find the hybrid parameters of the two-port network shown in figure below:

@@@@@