Code No.310551

# OR

## III-B.Tech I-Semester Supplementary Examinations May/June-2004

SIGNALS AND MODULATION THEORY

(Computer Science and Engineering)

### Time: 3 hours							Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the term convolution.

b)	A voltage represented by an impulse train of strength I and period T is filtered by a low-pass RC filter having 3-db frequency fC.

- Find the Fourier series of output voltage across the capacitor.

- If the third harmonic of the output is to be attenuated by 1000, find fC.T.

2.a)	Explain the term cross correlation.

b)	A waveform V(t) has a Fourier transform V(f) = 1 in the range -fM to +fM and

V(f) = 0 else where.  Make a plot of the transform V2(f).

3.a)	Explain about probability density function.

b)	A coin is tossed until a head appears.  Let T be the random variable which identifies the number of tosses t required for the appearance of first head.  Make a plot of the probability P(T  t) as a function of t up to t = 5.  Tabulate the results.

4.a)	With the help of a block schematic explain how a single side band signal can be generated using balanced modulators and phase shifters.

- The Fourier transform of m(t) is  F[m(t)] = M(f).

Show that

F[m(t)cos2πfct]12[M(f+fc)+M(f−fc)]

5.a)	With the help of equations deduce the relation ship between phase and frequency modulation.

- Using necessary equations, explain about frequency spectrum of a signal.

6.a)	Using necessary equations, give the geometrical representation of Binary phase shift keying signals.  Explain about power spectral density of binary PSK.

b)	Using circuit diagram explain how a differential phase shift keying signal is generated, and give the wave forms.

7.a)	What is the need for a Predictor?  Explain.

- Using a block schematic, explain about Delta modulation.

8.	Write notes on any TWO:

(a) Central limit theorem  (b) Synchronous Tome division multiplexing                (c)    Flat top sampling.

***     ***     ***