Code No: NR-311701

Set No:

1

III-B.Tech. I-Semester Supplementary Examinations, May-2004

MICROWAVE ENGINEERING

(Electronics and Telematics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.a)	With the help of Apple gate diagram explain velocity modulation and the 	working of a Reflex Klystron.

b)	Draw the equivalent circuit of a Reflex Klystron.

2.a)	Explain how magnetron allows electron bunching to take place and	prevents favored electrons  from slipping away from their relative

position.

b)	Is strapping advantageous under all conditions?

- Draw the cross section of a magnetron cavity system that does not require      strapping.

3.a) 	Explain the operation cycle of a TRAPATT diode.

b) 	Explain the difference between IMPATT diode and TRAPATT diode with respect 	to their operational characteristics and applications.

4.a)	Explain the physical structure and I-V characteristics of a Tunnel diode. Hence establish its microwave applications.

b)	Draw the equivalent circuit of a crystal diode and explain its features.

5.a)	What is the significance of Q in resonant circuits? Derive a general expression for Q for a series resonant circuit. What happens to Q when a circuit is loaded?

b)	A short-circuited /2 line is used as a resonator. Derive an expression for 	the Q 	of such a resonator.

6.	Explain the operating principles and constructional details of a Faraday rotation 	isolator. Where does one use an isolator in microwave networks? Obtain the 	S-matrix for an ideal isolator.

7.	Explain the microwave power measurement using

Bobmeter and thermo couple Distinguish between the two methods.

8.	State the types of losses in a micro stripline. Explain each of them.

-*-*-*-

Code No: NR-311701

Set No:

2

III-B.Tech. I-Semester Supplementary Examinations, May-2004

MICROWAVE ENGINEERING

(Electronics and Telematics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.a)	What is a re-entrant cavity? Why is such a cavity used in Klystrons?

b)	Explain the functions of a two-cavity Klystron using Apple gate diagram.

2.a)	What are the power capabilities and practical applications of various types of TWTS.  What are the major advantages of CW and pulsed TWTS?

b)	Explain the construction and principle of operation of coupled cavity TWT.

c)	Describe the methods to avoid oscillations in a TWT amplifier.

3.a) 	Compare Gunn diode and IMPATT diode with respect to their characteristics, 	applications and demerits.

b) 	Give equivalent circuit of IMPATT diode and explain how it is used as an

oscillator?

4.a)	Give Manley-Rowe relation equations regarding power flow of a nonlinear

reactance.

b) 	Describe the amplification mechanism of a parametric amplifier with the help of 	its equivalent circuit.

5.a)	A short open-circuited /4 line is used as a resonator. Derive an expression for 	the Q of such a resonator.

b)	Define the terms: Critical coupling and over coupling of a microwave resonator.

6.	Describe , with a neat sketch, the principles of operation of a 3 port circulator. 	Derive the S-matrix for the circulator, when all the ports are matched. Write down 	the S-matrices for a clock-wise and a counter clock wise circulator.

7.a)	Using an appropriate set up compare barretters and thermistors and their applications in the measurement of microwave power.

b) 	Explain the principle and utility of matched load in microwave measurements.

8.a) 	What are the factors upon which the radiation losses of a micro stripline are dependent?

- Derive an expression for the radiation factors of a micro stripline?

-*-*-*-

Code No: NR-311701

Set No:

3

III-B.Tech. I-Semester Supplementary Examinations, May-2004

MICROWAVE ENGINEERING

(Electronics and Telematics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.a)	What is ‘de bunching Effect’.

b)	What is ‘beam coupling coefficient’?

# c)	Explain the mechanism of formation of Bunch in a transit time device such as a Two-cavity Klystron with the help of Applegate diagram.

2.a)	Describe the practical aspects of backward wave oscillator performance, explaining the possibility of wide tuning  range in this device.

b)	Explain clearly the wave modes for a TWT.

c)	List the applications of a TWT.

3.a) 	Explain how IMPATT diode makes use of negative resistance concept.

b) 	Discuss merits and demerits of IMPATT diode.

4.a)	What is meant by parametric up-converter? Explain its features.

b) 	Explain the principle of operation and characteristics of a parametric negative 	resistance amplifier?

5.a)	Derive a general expression for the resonant frequencies of a rectangular 	waveguide cavity.

b)	List out the microwave applications cavity resonators.

6.a)	Define and explain the 3 theorems associated with waveguide T-junctions.

b)	A 4 port 10 dB directional coupler having 40 dB directvity has a transmission loss 	of 1 dB. For an input power of 10 mW  at port 1 of main arm, determine the 	power at the other ports.

7.a)	Explain the operation, the use and the limitations of a bolometer type of power

measurement  bridge.

b)	Distinguish between the different types of Q factors for a cavity, and establish a relation between them.

8.a) 	Which are the factors that limit the Quality factor of micro stripline?

b) 	With respect to the Dielectric attenuation constant, derive an expression for Quality factor.

-*-*-*-

Code No: NR-311701

Set No:

4

III-B.Tech. I-Semester Supplementary Examinations, May-2004

MICROWAVE ENGINEERING

(Electronics and Telematics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.a)	Describe the limitations of conventional tubes at microwave frequencies.

b)	Discuss the applications of microwaves for industrial control and  measurement.

2.a)	What is the purpose   of magnetic field in the cavity   magnetron?

b)	Derive Hull cut off magnetic field in a magnetron.

c)	What is mode jumping in a cavity magnetron? Explain clearly how mode jumping is avoided in a magnetron with neat diagrams.

3.a) 	Explain how Gunn diode can be used as an amplifier.

b) 	Distinguish between transit time domain mode and quenched domain modes of 	operation of a Gum diode.

4.a)	What is a  parametric down converter? Explain its features.

b)	Explain the significance and characteristics of a degenerate parametric amplifier?

5.a)	Find the resonant frequencies of the first three modes of a rectangular waveguide 	cavity of 5 x 4 x 2.5cm, filled with a dielectric of r  =1.210.

b)	Explain how a rectangular waveguide section can be used as an attenuator. What 	are its limitations?

6.a)	A 3 port circulator has an insertion loss of 1 dB, isolation of 20dB, and a VSWR 	of 1.50. Find the output power at ports 2 and 3 for an input power of 100mW at 	port 1.

b)	Obtain the S-matrix for this circulator.

7.a)	Explain how the measurement of medium and high microwave power can be done using calorimeter – wattmeter measuring system

b)	Explain the need for an isolator in a microwave bench.

8.a) 	Distinguish between micro striplines and Parallel striplines.

b) 	Show that a Parallel stripline can support a quasi TEM mode. Derive the required expression.

-*-*-*-