Code No:310452

## OR

III  B.Tech I-Semester Regular Examination, May 2004

# CONTROL SYSTEMS

(Common to Electronics and Communication Engineering, Electronics and Instrumentation Engineering and Electronics and Control Engineering )

Time: 3 hours.							Max.Marks:70

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	A Global Positioning  System (GPS) allows an aircraft to know its position in the airspace landing corridor very precisely.  Sketch a block diagram depicting how an air traffic controller might utilize GPS for aircraft collision avoidance.

- A common example of a two-input control system is a how shower with

separate valves for hot and cold water.  The objective is to obtain (1) a desired

temperature of the shower water and (2) a desired flow of the water.  Sketch a

block diagram of this closed-loop control system.

2.a)	Obtain the transfer function of the bridged – T network shown below.

Sketch the pole-zero diagram when R1=0.5 k, R2=lk, C1=C2=0.5F.

- A rotating load is connected to a field controlled dc motor through a gear system.  A test results in the output load reaching a speed of 1 rad/sec within ½ second when a constant 80V is applied to the motor.  The output steady-state speed is 2.4 rad/sec.  Determine the transfer function θν rad/volts assuming negligible field inductance.

Contd………2

Code No:310452				-2-	OR

3.a)	For the closed-loop control system shown below, determine the steady-state error when the desired input is a step and GL(s) = K and the disturbance d(s)=0.  When the reference input R(s) =0 and the disturbance d(s)=3s, determine the

steady-state error when (i) GC(s) = K1 and (ii) GC (s) = Ks.

- A unity feedback system is shown in the figure below.  Determine the relative stability of the system by locating the poles in the complex s-plane.

4.	The plant transfer function of a unity negative feedback system is given by

G(s) = K(s+20)s(s2+24s+144)

Sketch the root locus for K0.  Find the value of K that results in an oscillatory response.

5.	Given the loop transfer function

G(s) = −200s2s3+14s2+44s+40

Obtain the Bode plot and hence the gain and phase margins (Notice the negative gain).

Contd………3

Code No:310452			                -3-	OR

6.	Given the loop transfer function

G(s)=99(1+0.1s)(1+0.01s)2(1+0.2s)(1+1.5s)

Determine the stability of the closed loop system by applying Nyquist’s criterion.

7.	Given the following control system.

Find K to obtain a phase margin of 600.  Find the rise time, percent overshoot and settling time of this system.

8.	Given the state variable model

A=110−204    B = 00    C = 010   D = 0

With the initial condition

X(0)  = −1−1

Obtain the step response of the system.

***********