Code No: NR: 310102

Set.No:1

III-B.Tech I- Semester Supplementary Examinations May, June 2004

STRUCTURAL ANALYSIS

(Civil Engineering)

Time: 3 hours.							Max.  Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1. 	Draw the bending moment and shear force diagram of a propped cantilever beam of span 6 m due to a point load of 6 kN at the mid span.

2. 	Find the fixed end moments for a fixed beam of span 6 m subjected to a 	concentrated clockwise moment of 10 kNm at 2.5 m from the left end.

3.	A tripod with three 1.5m legs having a common ball-joint is set up on a uniform 	slope. The feet form an equilateral triangle with 1.8m sides, with two of the feet in 	a line of steepest slope and having a difference in level of 0.3m. Find the load 	induced in each leg by a load of 500N hung from the ball-joint. Use the method of 	tension coefficients.

4. 	Analyse the continuous beam shown in Figure by Clapeyron's theorem of three 	moments. Assume EI as constant. Also sketch the BMD and elastic curve.

5	Define Strain energy. Derive an expression for strain energy for a linear elastic system under axial load.

6.	A uniform load of 30kN/m, 5m long, crosses a girder 20m span. Calculate the maximum S.F and B.M at a section 8m from the left support. Also find out the maximum shear and the absolute maximum BM in the beam.

7	A Pratt girder consists of eight panels, each 3.5m square, the loading being on the lower bottom. Draw the influence line for the force in the diagonal third panel from the left and determine the maximum tension and compression in it due to uniformly distributed load of 100kN/m, 10m long.

8. a) 	The resultant of two forces is 225 N and the same are inclined at 250 and 400 with resultant, one on either side. Calculate the magnitude of two forces graphically..

b) 	Write Short notes on concurrent forces and non-concurrent forces.

#$#$#.

Code No: NR: 310102

Set.No:2

III-B.Tech I- Semester Supplementary Examinations May, June 2004

STRUCTURAL ANALYSIS

(Civil Engineering)

Time: 3 hours.							Max.  Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1. 	Find the maximum bending moment and locate the point of inflection for a propped cantilever beam of span 5 m due to a uniformly varying load, whose intensity is 5 kN/m at the fixed support and 2 kN/m at the simple support.

2. 	Draw the bending moment and shear force diagrams for a fixed beam of span 6 m 	due to a UDL of 5 kN/m on the left half of the span and a point load of 15 kN at 	the middle of the right half of the span.

3. 	Four straight wires AO, BO, CO and OD are made of the same material and have 	the same cross section. Their upper ends are connected to a horizontal ceiling, the 	points A, B, C and D forming a rectangle in which AB = 2m and BC = 2.5m. The 	junction O is 3m vertically below a point Q on the ceiling, 1m from AB and 	0.75m 	from AD. The connections are adjusted so that initially there is no 	slackness in 	any wire and a load of W is then suspended from O. Denoting the 	pull in the wire AO as P, find the pull in each of the remaining wires in terms of P 	and W. Use the method of tension coefficients.

4. 	A two span continuous beam is loaded as shown in Figure. During loading 	support B sinks by 10mm. Find the fixed end moments using Clapeyron's 	theorem of three moments. Also draw the BMD. Assume E = 2 x 104 N/mm2 and 	I = 1.5 X 108 mm4.

5	Define Strain energy. Derive an expression for strain energy due to bending moment.

6.	Two equal loads of 80kN each, spaced 4m apart roll over a girder of 12m span. Calculate the maximum B.M anywhere in the girder.

7.	A Warren girder of 30m span has six panels of 5m each. All the triangles are      equilateral. The load on the girder is transmitted to it at the lower panel points            only. Draw the influence line for the force in the bottom chord member in the third panel from left end. Find the maximum force in this member due to a uniform load of   37.5kN/m which is 16m long.

Contd..2

Code.No: 310102				-2-				Set.No: 2

8. a)	The angle between resultant (16.45 kN) and one of the forces (20 kN) is 450 55'. 	Find the other force and its direction graphically.

b)	Define the terms: 	i) Resultant 	and ii) Equilibrant.

#$#$#.

Code No: NR: 310102

Set.No:3

III-B.Tech I- Semester Supplementary Examinations May, June 2004

STRUCTURAL ANALYSIS

(Civil Engineering)

Time: 3 hours.							Max.  Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.	Find the maximum bending moment for a propped cantilever beam of length 8m with the prop at 5 m from the fixed end due to a UDL of 20 kN/m on the entire span.

2. 	Derive the fixed end moments of a fixed beam when one of the supports sinks by 	an amount . The span of the beam is L and the flexural rigidity of the beam is EI.

3.	The braced frame shown in Figure. is simply supported at A and B, and carries 	four 10kN loads at F, G, H and J. Determine the forces in members AG, GH, GD 	and AD using the method of tension coefficients.

4. 	The two span beam fixed at ends is loaded as shown in Figure. It has a uniform 	section throughout. Find the reactions and support moments using Clapeyron's 	theorem of three moments and draw the BMD and elastic curve.

Contd..2

Code.No:310201			-2-					Set.No:3

5	Define Strain energy. Derive an expression for Strain energy for a linear elastic system under shear force.

6.	A tractor engine crosses a plate girder bridge of 16m span from left to right . The load on the leading wheels is 50kN and on the driving wheels is 150kN, the wheel base being 4m.Find the maximum B.M under the driving wheels per main girder. If there are two main girders of same moment of Inertia.

7. 	Draw the influence line diagram for B.M at a point 8m from the left abutment on a bridge girder of span 30m and find the maximum B.M at that point due to a series of wheel loads 80kN, 160kN, 160kN and 160kN at centre to centre distances of 4m, 2.5m, 2.5m and 2.5m respectively. The loads can cross in either directions with the 80kN load leading.

8. a) 	A force acts at the origin in a direction defined by the angles  y = 60° and            	 z = 30°.

Knowing that the x component of force is -100 kN, determine graphically

i) 	The other components and magnitude of force

ii) 	The value of  x.

iii) 	The x, y, z components of a force are 35 kN, -25 kN and 35 kN 				respectively. Find the component of this force along the line joining

A (2,2,-2) and B (-1, -3, 2).

#$#$#.

Code No: NR: 310102

Set.No:4

III-B.Tech I- Semester Supplementary Examinations May, June 2004

STRUCTURAL ANALYSIS

(Civil Engineering)

Time: 3 hours.							Max.  Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1. 	The intensity of uniformly distributed load on the entire span of a propped cantilever beam of length 8 m is 5 kN/m. The prop is at a distance of 2 m from the free end. Draw the bending moment diagram.

2.	A fixed beam of span 8 m carries a concentrated load of 250 kN at 3 m from the                                                         left end. If the left end sinks by 15 mm, find the fixing moments at the supports. For the beam section take I = 4.5 x 107 mm4 and E =200 kN/mm2. Find also the reactions at the supports.

3.	Figure shows the plan of tripod, the feet A,B and C being in the same horizontal 	plane and the apex D being 4.5m above the plane. Horizontal loads of 100kN and 	150kN are applied at D in the directions shown. Using tension coefficients, find 	the forces in the members assuming all joints as pin joints.

4. 	Analyze the continuous beam shown in Figure by Clapeyron's theorem of three 	moments. Sketch the BMD and elastic curve.

- State and explain Castigliano’s first theorem taking any example.

Contd..2

Code.No:310201				-2-				Set.No:4

6.	A system of wheel loads crosses a girder of 21.60m span, which is simply supported at its ends. The loads and their distances are as follows.

Wheel load (kN) : 	100	100	  200	    200	       150

Distance between

centre (meter)    :             1.80       2.70     2.40        2.10

Determine (i) the maximum B.M at quarter span

(ii) the maximum B.M in the girder.

7. 	Determine the maximum S.F and B.M at quarter span from left end when a uniformly distributed load longer than the span of intensity 20Kn/m accompanied by a 120Kn concentrated load crosses the span of 14m. Use influence lines. The concentrated load can occupy any position.

8. 	Forces 40 kN, 25 kN and 110 kN are concurrent at origin and are respectively 	directed through the points whose coordinates are A(2,1,6), B(4,-2,5), C(-3,-2,1) 	and D(5,1,-2). Determine the resultant of the system, graphically

#$#$#.