Code No: NR-311403

Set No:

1

III-B.Tech. I-Semester Supplementary Examinations, May-2004

FINITE ELEMENT ANALYSIS

(Mechatronics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.	With the help of a neat diagram, describe the various components of stress and strains.

2.	Derive the shape function for a Quadratic one dimensional line element in Natural Co-ordinate system.

3.a) 	Compare the characteristics of beam element with the truss element?

b) 	Derive the load vector for the specified uniform distributed load over the plane truss element?

4.	Calculate the deflection at the center of the fixed beam as shown in the figure.

E = 220 Gpa,  	I1 = 1000 mm4	       20 kN	 I2 = 2000 mm4

A2 = 400 mm2

A1 = 200 mm2

750 mm	       1000 mm

5.	An axisymmetric ring element is shown in figure. Derive the matrices, [B] and [D]. Take E = 2 X 105 N / mm2 and μ = 0.33.

2				2

50

3

50    1      100			1	  3

6.	One side of the brick wall of width 5 m, height 4 m and thickness 0.5 m is exposed to a temperature of - 250C while the other surface is maintained at 320C. If the thermal conductivity is 0.75 W/m K and the heat transfer coefficient on the colder side is 50 W/m2 K. Determine (1) the temperature distribution in the wall and (2) heat loss from the wall.					Contd…2

Code No. NR-311403				    -2-				     Set No.1

7.	Determine the natural frequencies and mode shapes of a stepped bar as shown in figure using the characteristic polynomial technique. Assume E = 250 Gpa and density is 7850 kg/m3.

400 mm2

300 mm2

10 kN

0.3 m  			0.3 m

8. 	What are the advantages and limitations of NISA Package over ANSYS, 	CAEFEM and NASTRAN?

# ^^^

# Code No: NR-311403

III-B.Tech. I-Semester Supplementary Examinations, May-2004

Set No:

2

FINITE ELEMENT ANALYSIS

(Mechatronics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.	Derive the various equations for static equilibrium of an elastic body.

2.	Derive the shape functions for a one dimensional linear line element in global Co-ordinate system.

3.	For the pin jointed truss  shown in figure. If E = 200 Gpa,  determine

a)	Element stiffness  matrices		(b)   Global stiffness matrix

c)	Stress in the element 1		(d)  Strain in the element 2

A1 = 1000 mm2

500mm						15 kN

A2 = 1250 mm2		10 kN

750mm

4.	Calculate the deflection at the end of the stepped beam as shown in the figure.

E = 200 Gpa,  I1 = 4200 mm4;  I2 = 4200 mm4

25 kN

A1 = 200 mm2

A2 = 400 mm2

750 mm	       1000 mm

5.	Establish the Jacobian operator [J] of the two dimensional element shown in figure also find the Jacobian determinant.

3 (4, 5)

(2, 4)	4

2

1			(5,2)

(1,1)			Contd…2

Code No. NR-311403				    -2-				     Set No.2

# 6.	Derive the conductivity matrix and vector for the 2-D element when one of the faces is exposed to a heat transfer coefficient of h at T and with internal heat generation of q W/m3.

7.	Consider axial vibrations of the steel stepped bar as shown in figure:

(a) develop global stiffness matrix and mass matrix, (b) natural frequencies and (c) mode shapes.

600 mm2

10 kN

350 mm2

0.275 m  		0.275 m

8.a)	What are the limitations of NASTRAN in stress analysis?

b) 	What are the drawbacks of ANSYS in modeling?

# ^^^

# Code No: NR-311403

III-B.Tech. I-Semester Supplementary Examinations, May-2004

Set No:

3

FINITE ELEMENT ANALYSIS

(Mechatronics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.	Using the general approach of displacement function, derive the force-displacement relationship and element stiffness matrix.

2.	Find the displacement at the free end and the reaction force for the following problem.  Assume E = 2 x 105 N/mm2.

3.	Starting from the first principles derive the stiffness matrix for a 1- d bar element and extend it for the plane truss element?

4.	Consider a cantilever beam with uniform distributed load as shown in the figure. Estimate the deflection at the end of the beam.

E = 200 Gpa ; A = 25 mm X 25 mm.

10 kN/m

1 m		 	1 m

5.	Calculate the nodal forces of the four node axisymmetric finite element shown in figure when the element is subjected to centrifugal loading.

z

4		       3

η

ξ	1  cm

1

R1	    		         2

R2

ω

Contd…2

Code No. NR-311403				    -2-				     Set No.3

6.	A composite wall consists of 4 cm thick wood, 10 cm glass fiber insulation, and   1 cm thick plaster. If the temperature on wood and plaster faces are 200C and -200C respectively. Determine the temperature distribution in the wall. Assume the thermal conductivity of wood, glass fiber and plaster are 0.17,0.035and 0.5W/m K respectively and colder side heat transfer coefficient is 25 W/m2 K.

7.	Determine the natural frequencies of a simply supported beam of length 800 mm with the cross sectional area of 75 cm X 25 cm. Take E= 200 Gpa and density of 7850 kg/m3.

75cmX25cm

800 mm

8.	Explain and compare the different features of the following FEM software  packages. NISA, ANSYS, NASTRON.

^^^

Code No: NR-311403

III-B.Tech. I-Semester Supplementary Examinations, May-2004

Set No:

4

FINITE ELEMENT ANALYSIS

(Mechatronics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.	Derive a Property matrix (D)6 x 6 for isotropic materials, using Hook's relations.

2.	Derive the finite element equations from the one dimensional second order equation by variational approach.

3.	Estimate the stresses and reactions in truss structure shown in the following figure:					     15 kN

(3)		       (2)		E = 200 GPa

300 mm						A = 1000 mm2

(1)		      20 kN

400 mm

4.	Calculate the deflection at the center of the beam as shown in figure.

Take E = 220 Gpa; A = 40 mm X 40 mm	30 kN

300

1 m		 	1 m

5.a)	27. a)	Calculate the nodal load vector due to surface traction for the four nodded rectangular element shown in Figure

Py4

Py3

4		         3

Y			s

r	 2b

1		         2

2a

b)	Using Gaussian three point formula evaluate	13dxx

Contd…2

Code No. NR-311403				    -2-				     Set No.4

6.	Consider a brick wall thickness 0.3m with the thermal conductivity of 0.7 W/mK. The inner surface is at 280C and the outer surface is exposed to cold air with the heat transfer coefficient of 40 W/m2 K at -150C. Determine the steady state temperature distribution and also the heat flux through the wall. Use two elements and obtain the solution.

7.	Find the natural frequencies of a stepped bar as shown in the figure.

5 cm X 5 cm

10 kN

3.5 X 3.5 cm

E = 2 x105 N/mm2

Density

7850 kg/m3

12 cm		    24 cm

8.	Explain the terms (i) conformal element, (ii) Complete element, (iii) C continuity, 	(iv) p and r refinements.

^^^