Code No: 321351

### OR

III-B.Tech. II-Semester Supplementary Examinations, April/May-2004

CONTROL SYSTEM COMPONENTS

(Electronics and Control Engineering)

# Time: 3 Hours							Max. Marks: 70

## Answer any FIVE questions

## All questions carry equal marks

---

1.	Show that scaling of load parameters on a drive motor is achieved through employing gear trains.  Derive expression for the equivalent load and inertia and equivalent friction, control through a gear ratio of n:1 (motor-load).

2.a)	What are observational errors?  How does a linear scale reduce observational errors.

b)	What is an expanded scale?  Given an example of an indicating instrument having expanded scale.

3.a)	Describe the structure of a triac and describe function gate operation.

b)	List out advantages of FET over BJT (transistor).

4.a)	List out some of the exclusive application of FET.

b)	The N-channel E-MOSFET used in the common-source amplifier shown in the figure has the following parameters.

ID(ON) = 4mA at VDS = 10V;  VGS(th) = 4 V; Im = 5000 s.

Calculate VGS, ID, VDS, and Vo.

5.	Describe the torque-speed and control voltage Vs torque characteristics of an a.c. servo motor. Obtain an expression for the developed torque as a function of speed and the control winding voltage.

Contd…..2.

#### Code No: OR-321351				:: 2 ::				OR

6.a)	Give the pin diagram of IC 741 and describe one application of IC 741.

b)	A 5mV, 1 KHz sinusoidal signal is applied to the input of an OPAMP integrator shown in the figure below:

Given R = 100 k  and c = 1 F, find out e0.

7.a)	Define the following opto coupler parameters: (i) Isolation Voltage (ii) VCE(max) (iii) IE(max) and (iv) band width.

b)	Distinguish between pulse transformers and Auto winding conventional transformers.

8.a)	With the help of a neat diagram describe the working of a self-exclude magnetic amplifier.

- Each reactor of a series connected magnetic amplifier has a control winding of 2500 turns and a load winding of 500 turns and a feed back winding of 250 turns.  Calculate the power gain if the load resistance is 1500 ohm and control circuit resistance is 50 ohm.

@@@@@