Code No: NR-320402

Set No:

1

III-B.Tech. II-Semester Regular Examinations, April/May-2004

DIGITAL SIGNAL PROCESSING

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics and Control Engineering and

Electronics and Telematics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.a)	Define “discrete-time signal sequence”.

b) 	Write a note on “sampling theorem”.

c) 	Determine whether the DSP system described by the following equations is linear time invariant.

(i)  y(n) = anx(n)	(ii) y(n) = ax(n-1) + ax(n-1) + bx(n-2)

2.	Explain the geometrical construction method to determine magnitude and phase response of second order digital systems.

3.a)	What is “ padding with Zeros ”? With an example, explain the effect of padding a sequence of length N with L Zeros or frequency resolution.

b)	Compute the DFT of the three point sequence x(n) = { 2, 1, 2 }. Using the same sequence, compute the 6 point DFT and compare the two DFTs.

4.a)	Give the steps involved in implementing Radix –2, DIT FFT algorithm.

b)	Compute the 16-point DFT of  x(n)=Cosπ2n  0  n  15 using Radix – 2 , DIT algorithm.

5.a)	Check for the stability of the following system:

H(z)=2.2z3−4.57028z2−3.06z−0.593z4−1.84z3+1.2294z2+0.23z−0.354 .

b)	State and prove the necessary and sufficient condition for stability of a system.

6.a)	Explain the relation between analog and digital filters poles in IIM of 	transformation.

b)	Discuss the Aliasing effect due to impulse invariance transformation (IIM)

c)	Explain the method of determination of pole locations on an ellipse with major axis ‘R’ and minor axis ‘r’ for Chebyshev filter.

# Contd…2

Code No. NR-320402				   -2-				    Set No.1

7.a)	Explain briefly the method of designing FIR filter using Fourier series method

b) 	Design a FIR filter approximating the ideal frequency response

Hd(ejω) =  e –jαω, for | ω | ≤  π / 6

0,       for π / 6  ≤ | ω | ≤  π

Determine the filter coefficients for N=13.

8.a)	Explain the factors that influence the choice of structure for realisation of a LTI

system.

y(n)=a1y(n-1)+x(n)+b1x(n-1)   b)	An LTI system is described by the difference equation

Realize it in direct form I structure and convert it to direct form II structure.

# ^^^

Code No: NR-320402

Set No:

2

III-B.Tech. II-Semester Regular Examinations, April/May-2004

DIGITAL SIGNAL PROCESSING

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics and Control Engineering and

Electronics and Telematics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.a) 	What is a causal system? Why are non-causal systems unrealizable?

b) 	Check whether the DSP systems described by the following equations are causal.

(i) y(n)=3x(n−2)+3x(n+2)  (ii)y(n)=x(n−1)+ax(n−2)  (iii) y(n)=x(−n)

c) 	Define the terms “impulse response” and “unit step response” and give the relationship between them.

2.a)	Explain magnitude and phase spectrum with reference to frequency response of

system.

y(n)-y(n-1)+316y(n-2)=x(n)−12 x(n-1)   b)	Find the impulse response, frequency, magnitude response and phase response

of the given second order system

3.a)	Prove the following properties

(i)   arg[X(K)]  = -arg[X((-K)N]RN(K)     (ii)   Im[X(K)]   = -Im [X((-K))NRN(K)

b)	If X(K) denotes the N-point DFT of N-Point sequence x(n), show that with N even and if x(n) = x(N-1-n) then X(N/2)=0

4.a)	Develop a Radix – 2, DIF/FFT algorithm for an 8-Point sequence.

b)	Let X(k) be the N-point DFT of the sequence x(n), for 0 n N-1. Define a 2N-point sequence y(n) as

x(n/2)   n even

y(n) =

0 	       n odd.

express the 2N- point DFT of y(n) in terms of X(R).

5.a)	Consider the system:

H(z)=1−2z−1+2z−2−z−3(1−z−1)(1−0.5z−1)(1−0.2−1) 		0.5 < | z | < 1

- Sketch the pole Zero pattern. Is the system stable?

- Determine the impulse response of the system.

b)	Show that  | h(n) | <  is the necessary condition for stability of a system.

Contd…2

Code No. NR-320402				   -2-				    Set No.2

6.	Determine the system function H(Z) of the lowest order Chebyshev and Butterworth digital filter with the following specification

  - 3db ripple in pass band 0 w  0.2 

  - 25db attenuation in stop band 0.45   w  

7.a) 	What is the principle of designing FIR filters using windows.

b) 	Using a rectangular window technique design a low pass filter with pass band gain of unity, cut-off frequency of 1kHz and working at a sampling frequency of 5 kHz. The length of the impulse response should be 7.

8.a)	 Describe how targets can be decided using RADAR.

b) 	Give an expression for the following parameters relative to RADAR.

i) Beam width       ii) Maximum unambiguous range

c) 	Discuss signal processing in a RADAR system.

^^^

Code No: NR-320402

Set No:

3

III-B.Tech. II-Semester Regular Examinations, April/May-2004

DIGITAL SIGNAL PROCESSING

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics and Control Engineering and

Electronics and Telematics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.	The discrete-time systems are represented by the following difference equations in which x(n) is input and y(n) is output.

i) y(n)=3y2(n−1)−nx(n)+4x(n−1)−2x(n+1) and

ii) y(n)=x(n+1)−3x(n)+x(n−1) ; n ≥ 0

Are these systems linear? Shift-invariant? Causal? In each case, justify your answer.

2.a)	Prove the modulation and time shifting properties of distribute time Fourier transform.

b) 	A discrete system is given by following difference equation

y(n)-5y(n-1) = x(n) + 4x(n-1)

Where x(n) is the input and y(n) is the out put. Determine its magnitude and phase

response as a function of frequency.

3.a)	State and prove the circular time shifting and frequency shifting properties of the DFT.

b)	Compute the circular convolution of the sequences

x1(n)	=  { 1, 2, 0, 1 } and

x2(n)   	=  { 2, 2, 1, 1 } Using DFT approach.

4.	Develop a radix –2 DIF / FFT algorithm for evaluating the DFT for N=16 and hence determine the 16-point DFT of the sequence x(n) = { 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1 } .

5.a)	5.a)	How will you test the stability of a digital filter? Discuss the stability of the system described by

H(Z)=Z-11−Z−1−Z−2   b)	Determine the frequency, magnitude and phase responses and time delay for the

y(n)+14 y(n-1)=x(n)-x(n-1)	  system.

Contd…2

Code No. NR-320402				   -2-				    Set No.3

6.a)	What is warping effect?  Discuss influence of warping effect on amplitude response and phase response of a derived digital filter from a corresponding analog filter.

b)	Discuss impulse invariance method.

7.	Design a band pass Finite Impulse Response filter that approximate the following frequency response:

1;  160 ≤ f ≤ 200 Hz.

H(f) =

0; elsewhere in the range  0 ≤ f  ≤ fs / 2

# when the sampling frequency is 8000 sps. Limit the duration of impulse response to 2 msec. Draw the filter structure.

8.a)	Explain in detail the short time Fourier analysis for speech signals

b) 	What is a vocoder? Explain with a block diagram.

# ^^^

Code No: NR-320402

Set No:

4

III-B.Tech. II-Semester Regular Examinations, April/May-2004

DIGITAL SIGNAL PROCESSING

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics and Control Engineering and

Electronics and Telematics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.a)	Determine the impulse response for the systems given by the following difference equations.

i)  y(n)+3y(n−1)+2y(n−2)=2x(n)−x(n−1)

ii) y(n)=x(n)+3x(n−1)−4x(n−2)+2x(n−3)

b) 	Obtain condition for stability?

2.a)	Prove that the convolution in time domain leads to multiplication in frequency domain for discrete time signals

b)	The out put y(n) for a linear shit invariant system, with the input x(n) is given by

Y(n) =  x(n)-2x(n-1)+x(n-2)

Compute and sketch the magnitude and phase response of the system |w|≤ п.

3.a)	Define DFT of a sequence. Compute the N – point DFT of the sequence.

X(n) = Cos ( 2  r n / N), 0  n  N-1  and  0  r  N-1

b)	Explain how DFT can be obtained by sampling DFS for a given sequence.

4.a)	What is the computational efficiency of FFT Radix – 2 DIT algorithm for an N-point sequence? Compare it with that of direct DFT computation?

b)	A designer has available a number of 8 point FFT chips. Show explicitly how he should connect three such chips in order to compute a 24 – point DFT.

5.a)	With reference to Z-transform, state the initial and final value theorem.

b)   Determine the causal signal x(n) having the Z-transform.

































4

1

2

1

)

(

2

2

Z

Z

Z

Z

Z

X

6.a)	Discuss Bilinear transformation method of deriving IIR digital filter from

corresponding analog filter.

- Convert the following analog filter with transfer function      HA(S)=S+0.2/(S+0.2)2+16 using Bilinear transformation method

Contd…2

Code No. NR-320402				   -2-				    Set No.4

7.a)	Design a low pass filter by the Fourier series method for a seven stage with cut-off frequency at 300 Hz if ts= 1 msec. Use hanning window.

b) 	Explain in detail, the linear phase response and frequency response properties of Finite Impulse Response filters.

8.a)	Obtain the cascade and parallel form realisation of the LTI system governed by

the equation.

b)	Compare cascade and performance of direct and canonic forms.

^^^