Code No: NR-320104

Set No:

1

III-B.Tech. II-Semester Regular Examinations, April/May-2004

ESTIMATING AND CONTRACTS

(Civil Engineering)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

---

1.	List out the various items of work involved in the construction of class ‘A’ residential building.

2.a)	You have to prepare an estimate for providing a door in place of a window in a wall.  List the items involved in preparing such estimate.

b)	Show the forms used in preparing (i) abstract of cost and (ii) material statement.

3.	List out the items required for preparing detailed estimate of a simple flat roof two	roomed residential building.

4.a)	What is meant by Analysis of Rate? Mention various purposes of Rate Analysis.

b)	List out the factors affecting Rate Analysis. State the requirements of Rate Analysis.

5.	Prepare rate analysis for:

a)	2.5cm thick D.P.C. of cement concrete of 1:1.5:3 proportion with coarse sand and 13mm stone ballast including waterproof compound – for 100 sq.m.

b)	Random rubble stone masonry in superstructure in cement sand mortar in 1:6 proportion for cement work – for 10 cu.m.

Adopt the following quantities and rates of materials:

For 2.5cm thick D.P.C.

Stone ballast 2.25 cu.m.	Rs.500.00/cu.m.

Cement 			Rs.2580/cu.m.

Sand coarse			Rs.200.00/cu.m

Water proofing compound 30kg  Rs.30/kg

2N Mistri			Rs.80.00/head

2N of mason		Rs.65.00/head

14N of beldar		Rs.45.00/head

10N of coolie		Rs.20.00/head

T&P	L.S.			Rs.40.00

For random rubble stone masonry

Cement      0.7cu.m.	Rs.2050/cu.m.

Sand coarse		Rs.110/cu.m.

Stone including wastage 14 cu.m.Rs.250/cu.m.

0.5N Mistri			Rs.80.00/head

12N of mason		Rs.70.00/head

13N of beldar		Rs.40.00/head

11N of coolie		Rs.30.00/head

Scaffolding 		L.S.	Rs.31.00

T&P			L.S.	Rs.31.00

The quantities of the remaining materials have to be worked out

by adopting the above mentioned materials.

Contd….2.

### Code No: NR-320104			:: 2 ::				Set No:1

6.	Define the term contract.  What are the essential requirements of a valid contract? Write in brief about each essential requirement.

7.	Mention the various aims of specifications.  Explain in brief about various types of specifications.

8.	Write short notes on detailed specifications for

- Cement mortar

- Kankar lime mortar

- White lime

- Sand.

# @@@@@

# Code No: NR-320104

III-B.Tech. II-Semester Regular Examinations, April/May-2004

Set No:

2

ESTIMATING AND CONTRACTS

(Civil Engineering)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

---

1.	Write down the various items of work involved in the construction of

class ‘B’ residential building.

2.a)	Explain in detail about proforma estimate.

b)	How do you estimate the total quantity of steel in a preliminary estimate? Give an example.

3.	Mention the items required for preparing detailed estimate of a sloped roof 	single room residential building having a front verandah.

4. a)	“The Analysis of Rate is usually worked out for the unit of payment of a particular item of work under two heads”.  What are they?

b)	List out the factors on which rates of a particular item of work depend.

5.	Analyse the rates for:

a)	Ashlar masonry in superstructure in 1:6 cement sand mortar for completion of work.

b)	2.5cm cement concrete floor 1:2:4 – Take 100 sq.m. Adopt the following quantities and rates of materials:

For Ashlar masonry

Stone undressed 14cu.m.	Rs.240.00/cu.m.

Cement 0.4 cu.m.				Rs.1000/cu.m.

Sand coarse				Rs.200.00/cu.m

1N Mistri					Rs.80.00/head

16N of mason				Rs.70.00/head

7N of stone cutter				Rs.70.00/head

21N of beldar				Rs.40.00/head

21N of coolie				Rs.35.00/head

T&P					L.S.	Rs.40.00

Scaffolding					L.S.	Rs.37

For 2.5cm cement concrete

Stone ballast 20mm gauge 3 cu.m.	Rs.550/cu.m.

Sand coarse				Rs.220/cu.m

Cement 					Rs.2580/cu.m.

Cement for surface finishing		0.4 cu.m.

1N Mistri					Rs.55/head

11N of mason				Rs.50.00/head

6N of beldar				Rs.25.00/head

8N of coolie				Rs.25.00/head

Side forms 					L.S.	Rs.35.00

T&P					L.S.	Rs.40.00

The quantities of the remaining materials have to be worked out y adopting the above mentioned materials.

Contd…..2.

### Code No: NR-320104			:: 2 ::					Set No:2

6.	Differentiate between lumpsum and item rate contracts.  State the advantages and disadvantages of each.

7.	Write down the detailed specifications of Lime concrete in foundation.

8.	Write specifications for:

a)	Laying and joining of G.I. pipes for external work.

b)	Polished stone flooring over a bed of concrete.

# @@@@@

# Code No: NR-320104

III-B.Tech. II-Semester Regular Examinations, April/May-2004

Set No:

3

ESTIMATING AND CONTRACTS

(Civil Engineering)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

---

1.	Write down the various items of work involved in the construction of class ‘C’ residential building.

2.a)	Briefly explain the various methods by which volume of earthwork is computed.

b)	Explain the method of preparation of complete estimate of class ‘C’ residential building.

3.	What are the items required for preparing detailed estimate of a sloped roof two roomed building with front verandah.

4.a)	Write short notes on (i) Overhead Charges (ii) Contingent charges.

b)	Explain the procedure of analysis of rate of a particular item with the help of an example.

5.	Work out the analysis of rates for:

(i) Mosaic or Terrazo tile floor – Take 100 sq.m. (ii) Water proof cement pointing of approved make and quality, two coats on plastered surface – Take 100 sq.m.

Adopt the following quantities and rates of materials:

For Mosaic or Terrazo tile floor

Mosaic tiles	100 sq.m.	Rs.70/sq.m.

White lime.	2 cu.m.		Rs.350/cu.m.

Surkhi	4 cu.m.		Rs.190.00/cu.m

Cement	0.4 cu.m.	Rs.2700/cu.m.

1N Mistri			Rs.65.00/head

17N of mason		Rs.60.00/head

17N of mazdoor		Rs.30.00/head

2N of beldar		Rs.30.00/head

Polishing stone	L.S.	Rs.200

Oxalic acid powder	L.S.	Rs.50

T&P			L.S.	Rs.50.00

For Water proof cement pointing First Coat

Snow cem 30 kg		Rs.16/kg

3N of expert white washer	Rs.80/head

4N of coolies		Rs.30.00/head

T&P			L.S.	Rs.15.00

Second Coat

Snow cem 20 kg		Rs.16/kg

2N of expert white washer	Rs.80/head

4N of coolies		Rs.20.00/head

T&P			L.S.	Rs.20.00

The quantities of the remaining materials have to be worked out by adopting the above mentioned materials.

Contd….2.

### Code No: NR-320104			:: 2 ::					Set No:3

6.	What is the main object of making conditions of contract?  Group various conditions on which contract can be prepared.

7.	Write the detailed specifications of cement concrete 1:2:4.

8.	Write specifications for:

a)	Brick on edge flooring

b)	Painting

c)	Distempering.

@@@@@

# Code No: NR-320104

Set No:

4

III-B.Tech. II-Semester Regular Examinations, April/May-2004

ESTIMATING AND CONTRACTS

(Civil Engineering)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

---

1.	Write down the various items of work involved in the construction of class ‘D’ residential building.

2.a)	What are the different types of rough cost estimates?

b)	List out the factors to be considered during the preparation of a detailed estimate.

3.a)	Differentiate between abstract estimate and detailed estimate.

b)	Explain the advantage of centre line method over long walls and short walls method by means of a small example.

4.	List out the materials required (per unit rate for the following items of work):             (a) I class brickwork in cement mortar (b) Brick ballast cement concrete in 1:4:8

(c) 2.5cm stone ballast cement concrete 1:2:4 over 7.5cm brick ballast lime concrete floor.  (d) Course rubble masonry in cement mortar 1:6.

5.	Prepare rate analysis for:

a)	Half brick wall (10cm thick partition wall) with 1:3 cement mortar – take 100 sq.m.

b)	2.5cm thick cement concrete 1:2:4 over 7.5cm lime concrete floor Adopt the following quantities and rates of materials:

For half brick wall

Bricks I class (5000 no.’s)		Rs.1000.00 per % no’s

Cement 	0.75 cu.m.		Rs.2780/cu.m.

Sand coarse				Rs.200.00/cu.m

Mild steel bars 6mm dia every 4th layer

or hoop iron			40 kg	Rs.10.00/kg

0.5N Mistri				Rs.60.00/head

12N of mason			Rs.50.00/head

8N of beldar			Rs.25.00/head

10N of coolie			Rs.24.00/head

Scaffolding 			L.S.	Rs.80.00

T&P				L.S.	Rs.20.00

For 2.5cm thick cement concrete

Stone ballast      2.4cu.m.		Rs.120/cu.m.

Sand coarse			Rs.200/cu.m.

Cement				Rs.2500/cu.m.

0.75N Mistri			Rs.65.00/head

1N of mason			Rs.50.00/head

5N of beldar			Rs.30.00/head

5N of coolie				Rs.28.00/head

Side forms 				L.S.	Rs.80.00

T&P				L.S.	Rs.20.00

Contd…….2.

### Code No: NR-320104			:: 2 ::					Set No:4

For 7.5cm thick lime concrete

Brick ballast      7.5cu.m.		Rs.310/cu.m.

White lime slaked	1.2 cu.m.	Rs.300/cu.m.

Surkhi	2.4 cu.m.		Rs.185/cu.m.

0.75N Mistri			Rs.60.00/head

1N of mason			Rs.50.00/head

10N of beldar			Rs.30.00/head

10N of coolie			Rs.28.00/head

Side forms 				L.S.	Rs.80.00

T&P				L.S.	Rs.20.00

The quantities of the remaining materials have to be worked out by adopting the above mentioned materials.

6.	Write detailed notes on different types of contracts.

7.	Write the detailed specifications of Reinforced Cement Concrete.

8.	Write specifications for the following items of work:

a)	2.5cm cement concrete flooring

b)	Cement pointing.

@@@@@