Code No: NR-410101

Set No:

1

IV-B.Tech I-Semester Supplementary Examinations, May 2004

COMPUTER AIDED ANALYSIS

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.	Explain various fundamental reasons for implementing a Computer Aided Design system.

2.	Explain various types of graphics devices used in Computer Aided Design.

3.a)	Write about two dimensional geometry.

b) 	What is the difference between clipping and windowing?

4.	Find the forces in the members of the truss shown in figure by the stiffness method. Area of cross section and Young’s modulus are the same for all members.

A

3 m

5 kN

D

300

B

C

600

20 kN

5.	Derive expressions for first order, second order, third order and fourth order differentiation expressions using concepts of finite difference method.

6.	The differential equation governing the buckling of simply supported beam with both ends hinged with flexural rigidity EI is given by

d4y/ dx4 + (P/EI) d2y/dx2 = 0

Where P is the axial  load over its length L. Considering beam to be divided into 2 equal parts show that the error in calculated value of Pcr (Crippling load) is given by 19%. Use Finite Difference method.

Contd…2

Code No. NR-410101				-2-				Set No. 1

7.	Find the maximum and minimum value of the objective function z = 4x+5y.

Subject to 2x+y    6,     x+2y    5,           x-2y    2

-x+y     2,     x+y      1    and   x, y    0.

8.a)	Prove the following theorem.

“If both  primal  and dual problems have feasible solutions, then both have optional solutions and minimum f = maximum v.

b)  	construct the dual of the primal problem

Maximize       z = 2x1+x2+x3

Subject to              x1+x2+x3      6

3x1-2x2+3x3 = 3

-4x1+3x2-6x3 = 1

x1, x2, x3    0.

^^^

Code No: NR-410101

Set No:

2

IV-B.Tech I-Semester Supplementary Examinations, May 2004

COMPUTER AIDED ANALYSIS

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.	Describe automated drafting in Computer Aided system

2.	Explain various types of graphic terminals used in Computer Aided Design.

3. 	Write about the following in three dimensional transformation

a) Rotation		b) Translation 		c) Scaling.

4.

500 kN

C

300 kN

20

20

40

30

6 m

A

8 m

B

D

By stiffness method determine vertical and horizontal displacements of joint C and axial forces in the members of the truss shown in figure.

The cross sectional areas (mm2 ×103) are shown in each bar. E=200 GPa.

5.	Derive expressions for differentiation expressions connecting deflection, slope, bending moments shear force and load using concepts of forward difference terms in finite difference method.

6.	The differential equation governing the buckling of simply supported beam with both ends hinged with flexural rigidity EI is given by

d4y/ dx4 + (P/EI) d2y/dx2 = 0

Where P is the axial  load over its length L. Considering beam to be divided into 3 equal parts show that the Pcr (Crippling load) is given by  9 EI/L2. Use Finite Difference method.

Contd…2

Code No. NR-410101				-2-				Set No. 2

7.	Reduce the system of equations

2x1+3x2-2x3-7x4 = 1

x1+x2+x3+3x4 = 6

x1-x2+x3+5x4 = 4

into a canonical form with x1, x2 and x3 as the basic variables.

8.	Given the linear programming problem :

Maximize  z = 3x1+5x2

Subject to the constraints 	3x1+2x2   18,  x1     4

x2     6,    x1, x2   0

a)  Determine an optimum solution to the L.P. problem

b)  Discuss the effect on the optimality of the solution when the objective function is changed to z = 3x1+x2.

^^^

Code No: NR-410101

Set No:

3

IV-B.Tech I-Semester Supplementary Examinations, May 2004

COMPUTER AIDED ANALYSIS

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.	Explain typical features of a central processing unit used in Computer Aided Design.

2.	Explain raster scan terminals used in Computer Aided Design systems.

3.	Write about the following with examples.

a) Translation		b) Rotation		c) Scaling

4.	In the plane rigid frame shown in figure, calculate the displacements due to uniformly distributed vertical load of 2 kN/m acting over the beam. E= 200 GPa.

I=200x106mm4

D

C

2kN/m

5m

I=200x106 mm4

8m

B

A

5.	Derive expressions for differentiation expressions connecting deflection, slope, bending moments shear force and load using concepts of backward difference terms in finite difference method.

6.	The differential equation governing the buckling of simply supported beam with both ends hinged with flexural rigidity EI is given by

d4y/ dx4 + (P/EI) d2y/dx2 = 0

Where P is the axial  load over its length L. Considering beam to be divided into 3 equal parts show that the error in the value of Pcr (Crippling load) is given by 9%. Use Finite Difference method.

7.	Minimize      f  = -3x1-2x2                 subject to

x1-x2     1,    3x1-2x2     6,        x1     0,    x2    0.

Contd…2

Code No. NR-410101				-2-				Set No. 3

8.	Given the following linear programming problem :

Maximize z = -x1+2x2-x3

# Subject to       3x1+x2-x3    10,  -x1+4x2+x3   6

x2+x3     4,     x1, x2, x3     0

a)  Determine an optimal solution to the problem

b)  Determine the ranges for discrete changes in the components b2 and b3 of the required vector so as to maintain the optimality of the current optimum solution.

^^^

Code No: NR-410101

Set No:

4

IV-B.Tech I-Semester Supplementary Examinations, May 2004

COMPUTER AIDED ANALYSIS

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.	Describe some common editing features available in Computer Aided Design system.

2.	List out various functions of graphics software.

3.	Explain about the following.

(a) 	Windowing transformation		(b) 	Viewing Transformation.

4.	Analyse the frame shown in figure by matrix stiffness method.

P          B                                  C

2I

I                              I                 L

A                       2L                   D

5.	Derive expressions for differentiation expressions connecting deflection, slope, bending moments, shear force load using concepts of central difference terms in finite difference method.

6.	The differential equation governing the buckling of simply supported beam with both ends hinged with flexural rigidity EI is given by

d4y/ dx4 + (P/EI) d2y/dx2 = 0

where P is the axial  load over its length L. Considering beam to be divided into 4 equal parts show that the Pcr (Crippling load) is given by 9.376EI/L2.

7.	Minimize     f = -40x1-100x2       subject to

10x1+5x2     2500,        4x1+10x2     2000,

2x1+3x2      900,           x1    0,        x2    0.

Contd…2

Code No. NR-410101				-2-				Set No. 4

8.	Determine an initial basic feasible solution to the following transportation problem using

(a).  North-West corner rule        (b).  Vogel’s Approximation method.

Destination

|  |  | A1 | B1 | C1 | D1 | E1 | Supply |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | 2 | 11 | 10 | 3 | 7 | 4 |
| ### Origin | B | 1 | 4 | 7 | 2 | 1 | 8 |
|  | C | 3 | 9 | 4 | 8 | 12 | 9 |
|  | Demand | 3 | 3 | 4 | 5 | 6 |  |

^^^