Code No. 411352

### OR

## IV-B.Tech I-Semester Supplementary Examinations, May-2004

## PROCESS CONTROL

(Electronics and Control Engineering)

# Time: 3 hours							Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the following terms:

(i) Process Variables.		(ii) Process degree of freedom.

(iii) Fluid Resistance.		(iv) Liquid Capacitance.

b)	Explain the following with neat sketches:

(i) Thermal Process.		(ii) Liquid Process.

2.a)	Show that the time constant of a liquid vessel with turbulent resistance is              T = 2 HA/Q  .

b)	Calculate the gas capacitance of a 10 cu ft vessel containing air at room temperature.

3.a)	Discuss the Ziegler-Nichols method to tune the optimum parameters of PID controller.

b)	In an application of the Ziegler-Nichols method, a process begins oscillation with a 30% proportional band in an 11.5 minimum period.  Find the nominal          three-mode controller settings.

4.a)	Derive an expression for second-order response of fluid manometers.

b)	A mercury U-tube has 2 ft total length of mercury in a 1/8 cm.  I-D diameter tube.  What is the period of oscillation and damping ratio.

5.a)	Draw PI controller circuit, explain the working principle and operation of the circuit.

- Explain the principle of operation of pneumatic actuators.

6.	Explain the feed forward control scheme with neat sketch.  Give one example with block diagram representation.

7.	Discuss the following terms:

(i) Control-Valve sizing.		(ii) Control-Valve types

(iii) Cavitation.			(iv) Valve materials and services.

8.	Draw the control scheme of a distillation column for constant bottom product rate and constant reflux rate.  What assumptions are made in developing such a scheme?  What are its disadvantages?

- - -