Code No.:  NR-412001

Set No

1

IV B.Tech I Semester Supplementary Examinations, May 2004

TOOL DESIGN

(Production Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What are the important properties any cutting tool material should possess? 	Discuss the suitability of different cutting tool materials based on those properties.

b)	Give the necessary features of L-M-N system of single point cutting tool 	nomenclature. What is tool signature according to this system?

2.a)	What are the problems faced with continuous chips? How can they be overcome?

b)	Discuss how the rake and clearance angles vary radially in a drill bit?

3.a)	In the design of broach, what is meant by constant load per tooth? How is it 	obtained?

b)	Sketch a plain milling cutter and briefly describe its elements.

4.a)	The following contour has to be punched out from an aluminum alloy [ultimate 	strength 20,000 psi sheet of 0.1 inch thick. Find press tonnage and centre of 	pressure.

Note:- All dimensions are in inches.

b)	What are the different methods of bending sheet metal? What type of die – sets 	does each require? What is the press tonnage required for each method?

5.a)	With a neat sketch, explain the metal flow in a cup drawing process.

b)	What are the different types of drawing dies? What are the considerations in 	drawing die design?

(Contd…….2)

Code No.:  NR-412001			::2::			 	      Set.No:1

6.a)	What are Jigs and fixtures? What are their basic elements?

b)	Explain the 3-2-1 principle of pin location. What is redundant location?

7.a)	State Taylor’s  principles of gauge design. Why is it necessary to take deviation 	from Taylor’s principles?

b)	Give the procedure in cam design for any one component machined on a single 	spindle automatic.

8.	Write Short notes on

a)	Scrap strip layout

b)	Shear spinning

c)	Fool proofing

d)	Gilbert’s models.

$$$

Code No.:  NR-412001

Set No

2

IV B.Tech I Semester Supplementary Examinations, May 2004

TOOL DESIGN

(Production Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Sketch a single point cutting tool and name its elements. What is the importance 	of each of the elements?

b)	What are the problems encountered in high speed machining? What changes were 	made in the conventional single point cutting tools to over cone those problems?

2.a)	What is the need of chip breakers? What are the different types of chip breakers?

b)	Discuss about the grinding of land and rake surface of a plain milling cutter.

3.a)	Draw a neat sketch of  any one type of reamer and name its elements.

b)	What are thinned web drills?

c)	Give the design procedure for circular form tools?

4.a)	Draw a neat sketch of a progressive die to obtain a washer.

b)	What do you mean by providing shear on punch or die? What are its advantages 	and drawbacks?

5.a)	What is spring back? What are the methods of combating spring back in bending?

b)	Sketch any three quick acting clamps and explain in brief how they work?

6.a)	What are the important principles of location? Explain about each in brief.

b)	Obtain an expression for the tool life of a single point cutting tool with maximum 	production rate criterion.

7.a)	A fit is describe by 28Hf8. What is the type of fit? Design gauges to check this 	type of fit. 28mm lies in the diameter step 18 and 30mm. Fundamental tolerance 	unit i = 0.45(3D) + 0.001D microns. Fundamental deviation for                       	f-shaft = -5.5D0.41 and I T = 16i.

b)	State Taylor’s principles of gauge design.

8.	Write short notes on

a)	Plastics as tooling materials

b)	Forging die design

c)	Limitations of broaching

d)	Drill jig bushings.

$$$

Code No.:  NR-412001

Set No

3

IV B.Tech I Semester Supplementary Examinations, May 2004

TOOL DESIGN

(Production Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Describe the single point cutting tool geometry in N R S system. What is the tool 	signature according to this system?

b)	What are the materials suitable as cutting tool materials? What are the latest 	developments in this area?

2.a)	Design a circular form tool and sketch its geometry, for machining a semicircular 	groove of 5mm radius on a M.S rod of 40mm die. Take rake angle as 100 and 	clearance angle as 60.

b)	How is the number of teeth on a milling cutter decided?

3.a)	What are the different types of broaches? Sketch a pull type broach and describe 	its elements.

b)	Considering the design of a reamer, explain how you would design

i)	Reamer diameter		ii)	Back taper

iii)	Body length 			iv)	Number of teeth.

4.a)	Explain the shearing action in die – cutting operations. What is the importance of 	die – punch clearance?

b)	What is spring back in bending? How can it be overcome?

5.a)	What are the different types of drawing dies? What are the important drawing die 	considerations?

b)	What are the problems associated with drilling? Explain how they are over come 	with use of drill Jigs?

6.a)	Discuss about various principles of clamping. What are unconventional clamping 	methods?

b)	What are different costs associated with machining? Obtain an expression for tool 	life which gives minimum costs.

7.a)	Bring out the steps which would result in the lead cam for a turret type single 	spindle automatic.

b)	What are the different methods of constructing tools from plastics? Discuss about 	each in brief.

8.	Write short notes on:-

a)	Indexing Jigs		b)	Parting line design

c)	Limit gauging		d)	Redundant location.

$$$

Code No.:  NR-412001

Set No

4

IV B.Tech I Semester Supplementary Examinations, May 2004

TOOL DESIGN

(Production Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What are the general characteristics of carbide tools? What are coated carbides.

b)	Explain the features of high production cutting tools.

2.a)	During the machining of C-40 steel , a double carbide cutting tool of

0-10-6-8-6-750- 1 mm ORS shape has been used. What is the amount of

i)	Back rake 	ii)	Side rake and 	iii)	Front clearance

b)	What are the elements of drill point geometry? What are the common types of 	drill points?

3.a)	What is the classification of milling cutters? When is each types of milling cutter 	preferred?

b)	Considering the design of a broach, discuss how you will decide

i)	Total length of broach

ii)	Number of teeth

- Shear angle

- Tooth height

4.a)	Discuss about the design of punch for die – cutting operations. What are the 	methods of supporting slender punches?

b)	Draw the diagram of a simple die – set. What are the functions performed by each 	of its elements?

5.a)	Briefly discuss about the variables affecting the metal flow during drawing 	process.

b)	Sketch any one type of drill jig and recognize the basic elements.

6.a)	What is the use of drill jig bushes? Sketch and explain about different types drill 	bushes.

b)	What are the general principles of jigs and fixtures design?

7.a)	What are the important considerations in forging die design?

b)	Discuss about the various applications where plastics can be used as tooling 	materials.

8.	Write short notes on:

a)	Thinned web drills	b)	Shear & Pinning

c)	Nesting location	d)	Economics of machining.

$$$