Code No: NR-420403

Set No:

1

IV-B.Tech. II-Semester Regular Examinations, April/May-2004

INDUSTRIAL ELECTRONICS

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Mechatronics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.a)	What is the need for DC amplification? Give reasons.

b)	Explain the simple Direct-coupled Amplifier and derive its dc output voltage and 	gain expressions with the help of its equivalent circuit.

2.	Draw and explain the half bridge SMPS and derive expression for its output 	power.

3.a)	Define the short-period and long-period accuracy of a stabilizer.

b)	Draw the basic voltage stabilizer circuit using nonlinear voltage divider and 	explain it.

4.a)	Explain the operation of a SCR using its characteristics.

b)	Define all the ratings of SCR and give its significance.

5.	Explain the half wave thyristor phase-control circuits with inductive load using 	their waveforms and derive the expressions for average load voltage, average load		current and RMS load voltage.

# 6.	Draw and explain the simple SCR series inverter circuit employing Class A type commutation. Draw and explain the relevant waveforms. Explain its limitations.

# 7.	Explain the SCR sequential flasher used for automobile turn signals.

8.a)	Explain the theory behind the induction heating.

b)	List and explain the applications of induction heating.

# ^^^

Code No: NR-420403

Set No:

2

IV-B.Tech. II-Semester Regular Examinations, April/May-2004

INDUSTRIAL ELECTRONICS

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Mechatronics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.a)	What is the main trouble in DC amplifiers and give the main causes and remedies		 for it?

b)	What is the principle behind saturable core reactors with the help of voltage and 		current waveforms?

# 2.	Draw and explain the full bridge SMPS and derive expression for its output power.

3.a)	Explain the principle of operation of shunt  voltage regulator using integrated 	circuit.

b)	List out fabrication methods of integrated circuits.

4.a)	What is meant by catastrophic failure and explain the protection techniques for 		that in thyristors.

b)	Define the gate power dissipation and explain its importance in SCr.

# 5.	Explain the full wave thyristor phase-control circuit using its waveforms when the load is resistive and derive the expressions for average load voltage, average load current and RMS load voltage.

# 6.	Explain the operation of self-commutated inverter circuit and give all the voltage and current waveforms and give its applications.

# 7.	Design a simple and inexpensive burglar alarm to give an audio and visual indication of a door opening. Design for battery operation and low standby power.

8.a)	Explain the Synchronous welding control circuit.

b)	Draw and explain the energy storage welding circuit.

# ^^^

Code No: NR-420403

Set No:

3

IV-B.Tech. II-Semester Regular Examinations, April/May-2004

INDUSTRIAL ELECTRONICS

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Mechatronics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.a)	Explain the DC amplifier using cathode follower as the 1st stage and derive the 	expression for its gain using its equivalent circuit.

b)	What is meant by chopper and give its principle.

2.	Draw and explain the feed-forward SMPS with output filter and derive expression		 for its output power.

3.a)	Draw the circuit of monolithic regulator connected as a current regulator and 		explain it. Also obtain the expression for its load current.

b)	Differentiate between series and parallel voltage regulators.

4.a)	Explain the various types of triggering methods of an SCR briefly. Which is the 	universal method and why?

b)	List any four Triac applications.

# 5.	Explain the full wave thyristor phase-control circuit using its waveforms when the load is inductive and derive the expressions for average load voltage, average load current and RMS load voltage.

# 6.	Explain the operation of a single-phase parallel commutated inverter without 	freewheeling diodes, with the help of voltage and current waveforms.

# 7.	Explain the light dimmer control for incandescent lamps using traics.

8.a)	List and explain different electronic welding controls.

# b)	Explain the concept of heat control in resistance welding.

# ^^^

Code No: NR-420403

Set No:

4

IV-B.Tech. II-Semester Regular Examinations, April/May-2004

INDUSTRIAL ELECTRONICS

(Common to Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Mechatronics)

# Time: 3 Hours							Max. Marks: 80

## Answer any FIVE questions

## All questions carry equal marks

- - -

1.a)	Explain the principle of balanced push-pull DC amplifier with the help of its 	circuit diagram and give its advantages.

b)	Give the principle behind single transistor chopper.

2.	Draw and explain the single transistor fly back SMPS and derive expression for		its output power.

3.a)	Explain how to improve the performance of a regulator.

b)	Explain the LM 340 fixed voltage regulator.

4.a)	Explain the theory and operation of Triac.

b)	Explain the characteristics, ratings of Triac.

5.	Explain the half wave thyristors  phase-control  circuits  with  resistive load using their waveforms and derive the expressions for average load voltage, average load current and RMS load voltage.

6.	Design a snubber circuit and explain its operation and give its applications.

7.	Draw and explain the triac time delay relay circuit.

8.a)	Explain the resistance welding process.

b)	Draw the basic circuit of ac resistance welding and explain it.

^^^