Code No.:420857

OR

IV B.Tech II Semester Supplementary Examinations, April/May 2004

COMPUTER APPLICATIONS IN CHEMICAL ENGINEERING

(Chemical Engineering)

Time: 3 hours							Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.	A polymer blend is to be formed from three feed streams A, B and C having the 	following compositions in weight percent.

__________________________________________________________________

Component						Stream

______________________________________

A		B		C	Blend

1				25		35		55	30

2				35		20		40	30

3				40		45		 5	40

Deter mine the masses of A, B and C to produce 1000Kg of the blend having 	the desired composition.

2.	The molal heat capacities VS temperature for oxygen gas at zero pressure are 	given in the following table:

T(k)					Cp(Kcal/Kmol K)

100						6.95

200						6.96

400						7.194

600						7.67

800						8.06

1000						8.335

Fit a second degree polynomial to the data.

3.	A tank contains 2m3 of water. A stream of brine containing 20Kg/m3 of salt 	is added to the tank at a rate of 0.02 m3/s. Liquid flows from the tank at a rate 	of 0.01 m3/s. If  the tank is well agitated, determine the salt concentration in 	the tank, when the tank contains 4m3 of  brine. And also write a “C” program 	for the solution.

(Contd……….2)

Code No.:420857				::2::					OR

4.	Find the root of the equation x sinx + cosx = 0 corrected to three decimal 	places 	by “Newton Raphson” technique.

5.	Find the root of the following equation using BISECTION method correct to 	two decimal places: x3 + x2 + x + 7 = 0

6.	Find the minimum and the range of optimum for the function Y = (2x – 90)2 	using direct search with appropriate accelerating step size.

7.	The total annual cost of operating a pump and motor (C) in a particular piece 	of equipment is a function of x, the size of the motor namely,

C = $ 500 + $0.9x +$0.03x (150,000)($:dollar) Find the motor size that minimizes 	the total annual cost.

8.	Given that dydx = x +y2 and y = 1 at x = 0, find an approximate value of y at         	x = 0.5, using modified Euler’s method.

$$$