Code No.:421051

OR

IV B.Tech II Semester Supplementary Examinations, April/May 2004

DIGITAL SIGNAL PROCESSING

(Electronics and Instrumentation Engineering, Electronics and Control Engineering)

Time: 3 hours							Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define the following terms: related to LTI systems

i)	Linearity		ii)	Stability

iii)	Causality		iv)	Time invariance.

b)	Prove that the complex exponentials and sinusoidal sequences are Eigen 	sequences of LTI DT systems.

2.a)	State and prove circular shift and circular conduction properties of  DFT.

b)	Determine the circular convolution of x1(m) = 1,1,1,1 and x2(m) = 4,3,2,1 .

3.a)	Develop an 8 – point DIT FFT algorithm.

b)	Determine the DFT of the given sequence using DIF FFT algorithm 	x(m) = i,j,i,j,i,j,i,j

4.	Realize the following system using all possible realization methods:

H(z) = 1−12 Z−1(1−14 Z−1) (1+14 Z−1).

5.	Compare and contrast the design result of a Butter worth and Chebyshev  filters 	that are employed to realize a CPF to satisfy a flat response within 3dB from 	d.c to 10 KHZ and attenuation ≥30dB for f ≥ 2o KHZ.

6.a)	Explain why ideal filters are not physically realizable.

b)	Derive the expression for bilinear transformation method.

7.a)	Explain the Fourier series method of designing FIR filters.

b)	Design an FIR LPF using hanning window to meet the following specification

H(w) = 1 forW≤Π4. H(w) = 0,for Π/4< W≤Π  Assume impulse length 	of the 	filter = 11.

8.	Write short notes on any two:

a)	Applications of DSP in speech processing.

b)	In – Place computation.

c)	Impulse – invariance method.

$$$