Code No. 210255

OR

II B.Tech.  I-Semester Supplementary Examinations, November-2003

MECHANICS OF FLUIDS AND HYDRAULIC MEACHINES

(Electrical and Electronics Engineering)

Time: 3 hours							         Max. Marks:70

Answer any FIVE questions

All question carry equal marks

- - -

1.a)	Distinguish between solids, liquids and gases. Is there any analogy of Hooke’s law in fluids? If so state the parallel law is fluids.

b)	In a stream of glycerine in motion at a certain point the velocity gradient is 0.25 per second. The mean density of fluid is 1268 kg/m3 and the kinematic viscosity is 6.30 x 10-4 m2/s. Calculate the shear stress at the point.

2.a)	Study of pressure variation around objects on a wind tunnel requires measurement of very small  pressures.  What measuring fluid and what type of manometer would you suggest?

b)	A vertical gate is a tunnel 6.0 m in diameter has water in the one side and air on the other. The water surface is 12 m above the invert and the air pressure is    122.5 kN/m2 abs. Determine the magnitude and location of the resultant force acting on the gate.

3.	Sketch the stream lines represented by  = x2 + y2 and   = x2 - y2 showing the direction of flow in each case. Also find out the velocity and its direction at (1,1).

4.a)	A pipe carries a liquid of specific weight , at a flow rate of Q m3/s. If at a section, the datum head, the pressure and velocity are Z, p and v respectively, what is the total energy passing that section? Why does the concept of fluid head has no significance in gas flow?

b)	0.91 m3/s of water flows up through a vertical  conical enlargement, one meter high, 30 cm diameter at bottom and 60 cm diameter at top. The pressure at bottom is  981 bar. Calculate the magnitude and direction of the force exerted by water on the enlargement.

5.a)	What arrangements would you suggest to increase the discharge without increasing the size of the orifice for the same head? What are the various methods of flow measurement in open channels?

b)	The head up stream of a rectangular weir 2 m long is 10 cm. If cd = 0.6 and the cross sectional area of the up stream flow is 0.3 m2, estimate to a first approximation the discharge allowing for the velocity of approach.

Contd…2

Code No. 210255				-2-					OR

6.a)	Obtain an expression for the force exerted by a jet of water on inclined flat plane.

b)	A jet of water 50 mm  in diameter has a velocity of 40 m/s. The jet strikes a vane which deflects it through 1350. The vane moves through a velocity if 10 m/s in the direction of jet. Determine the force, work done per second and the efficiency of the vane.

7.a)	What are important compinents of a reaction turbine and what are their functions?

b)	A turbine operates under a head of 25 m at 240 rpm and uses a discharge of

10 m3/s. The over all efficiency of the turbine is 85%. Determine,

(i)	Power developed by the turbine

(ii)	The specific speed of the turbine.

- Type of turbine and

- Performance of turbine under a head of 20 m.

8.a)	What is the meaning of the term NPSH? What is required NPSH and available NPSH? What is its importance?

b)	A centrifugal pump, has a 300 mm diameter impeller and an outlet width of 50 mm. It runs at 1000 rpm and deliver water against a head of 15 m.  The vanes of the impeller are curved backwards at an angle of 300 with the periphery at outlet. The manometric efficiency of the pump is 92%. Calculate the discharge.

^^^