Code No.RR/NR211001

Set No.

1

II B.Tech. I-Semester Supplementary Examinations, November-2003

ELECTRICAL TECHNOLOGY

(Common to Electronics and Instrumentation Engineering, Bio-Medical Engineering and Electronics Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	State the reasons for a self exited DC shunt generator to fail to build up voltage, and suggest the necessary modifications.

b)	Explain the importance of critical field resistance and how it can be determined

c)	A 4-pole generator having wave-wound armature winding has 51 slots each slot 	contains 20 conductors. What will be the voltage generated in the machine when 	driven at 1,500rpm assuming the flux per pole to be 7.0 mWb?

2.a)	From the first principles, derive an equation for torque developed in a D.C. Motor.

b)	A 20 kw, 250 V d.c. shunt generator has armature and field resistances of 	0.04and 200 respectively. Determine the total armature power developed 	when working    (i) as generator delivering 20 kw output and

(ii) as a motor taking 20 kw input.

3.a) 	Derive the e. m. f. equation of a single-phase transformer and draw the no-load

phaserdiagram.

b) 	A 40 kVA transformer with ratio of 2000 / 250 V has a primary resistance of

1.15  and a secondary resistance of 0.0155 . Calculate (i) the total resistance 	interms of secondary winding, (ii) the total resistance drop on full load, and

(iii)  the total copper loss on full load.

4.a) 	Obtain the equivalent circuit of a single-phase transformer.  Explain how to evaluate the equivalent circuit of a transformer from open circuit & short circuit tests.

b) 	A 5 kVA, 220 / 110 volts, 1-phase transformer has a maximum efficiency of 96.97 % at 0.8 p.f. lagging. It has a core loss of 50 watts and the full load regulation at 0.8 p.f. lagging is 5 %. Find the efficiency and regulation at full load 0.9 p.f. lagging.

5.a) 	Explain the principle of rotating magnetic field and hence prove that it is of constant magnitude and rotates at synchronous speed.

b) 	A 3-phase, 50 Hz induction motor has a full-load speed of 1440 r.p.m. For this 	motor, calculate the following (i) number of poles (ii) full-load slip and rotor 	frequency (iii) speed of stator field with respect to (a) stator structure and (b) rotor 	structure and (iv) speed of rotor field with respect to (a) rotor structure (b) stator 	structure and (c) stator field.

(Contd…2)

Code No.RR/NR211001				-2-		Set No.1

6.a) 	Derive e.m.f equation for an alternator and explain distribution factor and pitch factor used in e.m.f. Equation.

b)	Write the expression showing the relationship between speed frequency and no. of 	poles of a synchronous machine. The speed of rotation of the turbine driving an 	alternator is 166.7 r.p.m. What should be the no. of  poles of the alternator if it is 	to generate voltage 50HZ.

7.a)	Explain the principle of working of synchronous motor.

b) 	A 3 phase, 1385 V star connected synchronous motor having synchronous reactance of 2ohm per phase and negligible resistance takes an input of 207.8 kw with an induced e.m.f of 916.5 V per phase. Calculate the motor line current and its power factor.

8.a) 	Describe the construction and working principle of shaded pole induction motor.

b) 	Discuss the difference of capacitor start, split phase, permanent capacitor, shaded 	Pole induction motors.

***     ***     ***

Code No.RR/NR211001

Set No.

2

II B.Tech. I-Semester Supplementary Examinations, November-2003

ELECTRICAL TECHNOLOGY

(Common to Electronics and Instrumentation Engineering, Bio-Medical Engineering and Electronics Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the constructional features of a D.C. Machine with the help of a neat sketch.

b)	Name the main parts of a D.C. Machine and state the materials of which each part is made.

c)	Explain different methods of excitation of D.C. Generators with suitable 	diagrams.

2.a)	With the help of neat sketches, explain torque-speed characteristics of the

(i) D.C. Series  (ii) D.C. Shuntmotor

b)	Find the torque in N-m exerted by a 4-pole series motor whose armature has 1200 	conductors connected up in a 2-circuit winding. The motor current is 10A and the 	flux per pole is 0.02Wb.

3.a) 	Sketch the phasor diagrams of a single-phase transformer for (i) resistor

(ii) inductor and (iii) capacitor loads.

b) 	A 220/110V, 50Hz, 1.5KVA, for has by & sec winding resistance of 1 and 2, 	and reactance of 3 and 5 respectively

find (i) the total. Example resistance equilibrium reactance and equilibrium impedance referred to Py. and Secondary.

4.a) 	Obtain an expression for the regulation of a single-phase transformer from its equivalent circuit / phasor diagram.

b) 	A 20 kVA, 2500 / 250 volts, 50 Hz, 1-phase transformer gave the following test results:                   O.C. test (L.V. side): 250 V, 1.4 A, 105 W;

S.C. test (H.V. side): 104 V, 8 A, 320 W.

Compute the parameters of the approximate equivalent circuit referred to L.V.

5.a)	Explain why the rotor of polyphase induction motor can never attain synchronous speed.

b) 	A 10 kW, 400 V, 3-phase, 4-pole, 50 Hz delta connected induction motor is 	running at no load with a line current of 8 A and an input power of 660 watts. At 	full load, the line current is 18 A and the input power is 11.20 kW. Stator 	effective resistance per phase is 1.2  and friction, windage loss is 420 watts. For 	negligible rotor ohmic losses at no load, calculate, (i) stator core loss ; (ii) total 	rotor losses at full load ; (iii) total rotor ohmic losses at full load ; (iv) full load 	speed; (v) internal torque, shaft torque and motor, efficiency.

(Contd…2)

Code No.RR/NR211001				-2-		Set No.2

6.a)  	Derive an equation for emf of a three phase alternator.

b)	A 400 V, 10KVA, 3 phase alternator with star connected stator winding has an 	effective armature resistance per phase of 1.0 ohm. The alternator generates an 	open circuit voltage per phase of 90 V with a field current of 1.0 A. During the 	short circuit test, with 1.0 A of field current, the short circuit current flowing in 	the armature is 15A. Calculate (i) the synchronous impendence (ii) If the 	alternator is supplying a load current of 15A at 0.8 p.f lagging, to what value 	would the terminal voltage rise if the load is through off.

7.a)	Explain clearly the difference between the working of an induction motor and synchronous motor.

b)	The input to an 1100 V, 3 phase star connected synchronous motor is 60 A.The 	effective resistance and synchronous reactance per phase is 1 ohm and 30 ohm 	respectively. Find the power supplied to the motor and the induced e.m.f for a 	power factor of 0.95 leading.

8.a) 	Draw a diagram showing the construction of a stepper motor and discuss  its 	operation.

b) 	Discuss the function of an ac tacho meter. Explain its construction and operations.

***     ***     ***

Code No.RR/NR211001

Set No.

3

II B.Tech. I-Semester Supplementary Examinations, November-2003

ELECTRICAL TECHNOLOGY

(Common to Electronics and Instrumentation Engineering, Bio-Medical Engineering and Electronics Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Why brushes and commutator are necessary for operation of a D.C. Machine.

b)	How D.C. Generators are classified?

The armature of a 6-pole d.c. generator has a wave winding containing 664 	conductors. Calculate the generated e.m.f. when flux per pole is 0.06 weber and 	the speed is 250rpm. At what speed must the armature be driven to generate an 	e.m.f. of 250V if the flux per pole is reduced to 0.058 weber?

2.a)	What are all the various losses in a D.C. Machine?

b)	A series motor of resistance 1 ohm between terminals runs at 1,000rpm at 250Vwith a current of 20A. Find the speed at which it will run when connected in series with a 6 resistance and taking the same current at the same supply voltage.

c)	Derive an expression for efficiency of a D.C. Machine.

3.a) 	Develop the equivalent circuit of a single-phase transformer and draw its phasor diagram for a leading power factor load.

b) 	A single phase 230/115V, 50Hz, 2.5KVA, transformer has 110 turns as Primary as HV side.  Determine the emf/turn, number of terms of LV side, full load currents as hv/Lv sides.

4.a) 	Define efficiency of a transformer. Obtain the condition for maximum efficiency.

b) 	A 25 kVA, 2500 / 250 V, single-phase transformer gave the following test results.      O.C. test (LV side): 250 V, 1.4 A, 105 Watts.

S.C. test (HV side): 105 V, 8 A, 320 Watts.

Compute the equivalent circuit parameters referred to LV side and HV side. Also obtain percentage regulation at full load with 0.8 power factor lagging.

5.a)	With a neat sketch, explain the construction of a 3-phase induction motor.

b) 	3-phase, 4-pole, 1440 rpm, 50 Hz induction motor has star-connected rotor winding, having a resistance of 0.2  per phase and a standstill leakage reactance of 1  per phase. When the stator is energised at rated voltage and frequency, the rotor-induced e.m.f at standstill is 120 V per phase.

(i) calculate the rotor current, rotor power factor and torque both at starting and at full load and compare these results

(ii) If an external resistance of 1 ohm per phase is inserted in rotor circuit, 	calculate rotor current, rotor power factor and torque at the time of starting.

(Contd…2)

Code No.RR/NR211001				-2-		Set No.3

6.a)	Define voltage regulation of an alternator. Explain synchronous impedance method of determining regulation of an alternator.

b) 	Calculate the voltage induced per phase in a 3phase 50 Hz, alternator having a 	flux per pole of 0.1515 wb. The no. of conductors in series are 360. Assume full 	pitch coil with a distribution factor of 0.96.

7.a)	Explain the principle of operation of synchronous motors.

b) 	A 3-phase alternator is rated at 5 KVA, 110V, 26.3A, 50 Hz and 1200 r.p.m. The 	stator resistance between terminals as measured with dc is 0.2 ohm. With no load 	and rated speed the stator line voltage is 160V for a field current of 4A.At rated 	speed, the short circuit stator current per terminal is 50A for a field current of 	4A.compute voltage regulation of alternator at 0.8 p.f. Lagging. Using 	synchronous impedance method.

8.a) 	What is a synchro? Draw a connection diagram of a synchro transmitter and 	explain its working.

b) 	Explain the working principle of split-phase capacitor-run single phase Induction 	Motor.

***     ***     ***

Code No.RR/NR211001

Set No.

4

II B.Tech. I-Semester Supplementary Examinations, November-2003

ELECTRICAL TECHNOLOGY

(Common to Electronics and Instrumentation Engineering, Bio-Medical Engineering and Electronics Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Sketch the symobilic representation following types of D.C. Generators

(i) Shunt    (ii) Series and        (iii) Compound

b)	State with reason the area of application of various D.C. Generator is used?

c)	A 4-pole machine running at 1000rpm has an armature with 90 slots having 6 	conductors per slot. The flux per pole is 6X10-2 Wb. Determine the induced emf 	as a D.C. Generator if the coils are lap connected. If the current per conductor is 	50 amperes, determine the electrical power output of the machine.

2.a)	With  a neat sketch, explain the working of a 3-point starter for d.c. shunt motor.

b)	A 6-pole, 250V series motor is wave-connected. There are 240 slots and each slot has 4 conductors. The flux per pole is 1.75X10-2 Wb when the motor is taking 80A. The filed resistance is 0.05, the armature resistance is 0.1 and the iron and frictional loss is 0.1kw. Calculate (i) Speed, (ii) bhp and (iii) shaft torque.

3.a) 	Draw the phasor diagram of a 1-phase transformer with lagging loads. Explain.

b) 	A 1-Q, 440/220V, 5 KVA, 50Hz transformer, draws a no-load current of 0.8A at 	a.p.f of 0.25 lag on h.v side.  Determine the magnitising component, careless 	component, emf / turn as either side and the primary current when the secondary 	current is 18A at a p.f. of 0.9 lag.

4.a) 	Explain how the equivalent circuit parameters can be obtained from open circuit and short circuit tests.

b) 	A 300 kVA, 11000 / 440 V, single phase, 50 Hz, transformer gave the following test results. Open circuit test on LV side a normal voltage and frequency input 1.3 kW, 21.1 amp; short circuit test HV side with voltage 600 V, input 2.80 kW, 15.0 amps. Calculate the efficiency and regulation for full load at 0.8 p f lagging.

5.a)	Explain the principle of operation of a 3-phase induction motor.

b) 	A 10 kW, 400 V, 3-phase, 4-pole, 50 Hz slip-ring induction motor develops rated 	output (i.e. 10 kW) at rated voltage and frequency and with its slip rings short-	circuited. The maximum torque equal to twice the full-load torque, occurs at a slip 	of 10% with zero external resistance in the rotor circuit. Stator resistance and 	rotational loss are neglected. Determine (i) slip and rotor speed at full-load torque, 	(ii) rotor ohmic loss at full-load torque, (iii) starting torque at rated voltage and 	frequency, (iv) starting current in terms of full-load current, (v) stator current at 	maximum torque, in terms of full-load current and  (vi) full-load efficiency.

(Contd…2)

Code No.RR/NR211001				-2-		Set No.4

6.a)	A 2-pole 3-phase alternator stator has a total of 270 armature conductors on it. The total flux per pole is 1.8 x 106 lines. If the frequency of the generated voltage is 50HZ, find the induced e.m.f per phase.

b) 	A 3-phase 4-pole synchronous generator has a double layer winding having four 	turns per coil placed in a total of 48 slots. If the flux per pole of the generator is 	2 x106 lines and speed of the rotor is 1500 r.p.m., calculate the magnitude of 	generated voltage per phase.

7.a)	Draw the phasor diagram of the synchronous generator on load. Explain the meaning synchronous reactance.

b)	A 4 pole, 400V, 50HZ, 3 phase star connected synchronous motor has a 	synchronous reactance of 2.0 ohm per phase. The resistance of the stator winding 	is negligible. The field excitation is so adjusted that the e.m.f induced is equal to 	the supply voltage. On application of a certain load the rotor is retarded by 4 	mechanical degrees. Find the armature current drawn by the motor.

8.a)  	What is a stepper motor?  Enumerate its advantages.

b) 	With neat sketch, explain the working principle of shaded-pole single-phase 	induction motor.

***     ***     ***