Code No: RR-210303

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003.

BASIC ELECTRONICS

(Common to Mechanical Engineering, Production Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	With the help of neat diagrams explain the two biasing techniques of a diode.

Draw the V-I characteristics and indicate Vγ.

b)	Obtain the static and dynamic resistances of the p-n junction germanium diode, if

the temperature is 270 C and Io=1μA for an applied forward bias of 0.2 V.  	Assume k=1.3810-23 J/ok.

2.a)	What is “Early effect” phenomenon in a transistor and explain its consequences.

b)	Draw and explain the input and output characteristics of n-p-n transistor in C- B configuration.

3.a)	Write short notes on “Frequency stability in Oscillator”.

b)	Draw the circuit of RC phase shift Oscillator using transistor. Derive an expression for frequency of Oscillation.

4.a)	Classify the timers according to the function and the technique used to achieve the

industrial timing.

- List the electronic welding controls used in resistance welding.

5.a)	Explain the principle of dielectric heating.

b)	Explain the application of dielectric heating for

(i)	Pre-heating of plastic preforms and 	(ii)    Food processing.

6.a)	Draw the block diagram of CRO and explain the function of each block.

b)	Derive the expression for the electro-static deflection sensitivity of a cathode ray

tube.

7.a)	What is an interrupt? What are various interrupts? Explain them.

b)	Discuss about various fields in an instruction.

8.a) 	List various characteristics of A-to-D converter.

b) 	Compare three A-to-D conversion techniques.

@@@@@

Code No: RR-210303

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003.

BASIC ELECTRONICS

(Common to Mechanical Engineering, Production Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Find the concentration of holes and of electrons in p-type germanium at 300oK, if

the conductivity is 100 (Ω-cm)-1.

b)	Define diffusion and transition capacitance of p-n junction diode. Prove that diffusion capacitance is proportional to current I.

2.a)	Discuss the potential variation and variation of minority carrier concentration in      p-n-p transistor biased in the active region.

b)	Draw and explain a simple SCR circuit for speed control of a motor.

3.a)	An amplifier with open loop voltage gain 2000 ± 150 is available. It is necessary to have an amplifier whose voltage gain varies by no more than ± 0.2 percent. Find the average transmission factor ß of the feedback network and gain with feedback Af.

b)	List five characteristics of an amplifier which are modified by negative feed back. Explain in detail.

4.a)	Classify timers according to the function and briefly explain them.

b)	Draw the circuit for A.C. resistance welding and explain.

5.a)	Discuss frequency stability considerations in induction heating.

b)	Discuss the industrial application of dielectric heating.

6.a)	Discuss briefly (i)Eletrostatic focusing and 	(ii) Magnetostatic focusing methods used in  cathode ray tube and compare their merits.

b)	Explain the need of ion trap in cathode ray tube

7.	Explain about stack, Input-output and machine control group instructions with examples.

8.a) 	List various A-to-D conversion techniques.

b) 	Explain the operation of flash A-to D converter.

@@@@@

Code No: RR-210303

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003.

BASIC ELECTRONICS

(Common to Mechanical Engineering, Production Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Draw and explain the graph indicating the variation of minority carrier density 	with distance in a p-n junction diode under forward biased condition.

b)	Derive the expression for intrinsic conductivity of intrinsic semiconductor. Find the conductivity and resistivity of an intrinsic semiconductor at temperature of 3000k.

Assume 	ni   = 2.5 x 1013/cm3		i   = 3800 cm2/ sec- volt

p  = 1800 cm2/ sec- volt	q =1.6x10-19 coulomb.

2.a)	Draw the output characteristics of junction transistor in C B configuration and indicate the active, cut off and saturation regions and explain the shape of the these curves.

b)	What are the different methods of triggering SCR. Explain one of these methods.

3.a)	Derive an expression to illustrate that the voltage gain in an amplifier circuit with negative feedback is some what stable even if the ß of the transistor changes due to aging or due to its replacement.

b)	Draw the circuit diagram of a phase shift oscillator using BJT. Explain why we cannot have phase shift oscillator which uses only two RC networks instead of three for obtaining a phase shift of 1800.

4.a)	Compare and contrast the following timers.

(i) 	Delay Timers 			(ii) 	Interval Timers

(iii) 	Repeat cycle timers 		(iv) 	Reset Timers

b)	Discuss the process of resistance welding.

5.a)	Explain the application of induction heating for

(i)	Brazing and		(ii)	Annealing of Brass and Bronze items.

b)	Discuss the selection of suitable electrodes for dielectric heating.

6.a)	Give main constituents of cathode ray oscilloscope.

b)	What are the defects of deflection in C R O? Explain them in detail.

7.a) 	Distinguish between assembly language and machine languages. What are their advantages and disadvantages?

b) 	What is subroutine? Explain about subroutine call instructors.

8.a) 	List the drawbacks of binary weighted resistor technique of D-to-A conversion.

b) 	Explain the R-2R ladder technique of D-to-A conversion.

@@@@@

Code No: RR-210303

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003.

BASIC ELECTRONICS

(Common to Mechanical Engineering, Production Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What is the law of mass action. A p-type germanium at 3000k has conductivity of

300 mho/cm. Calculate the concentration of impurity atoms, holes and electrons.

Assume that p = 1800 cm2/sec – volt and ni = 2.5 x 1013 cm-3.

b)	Define the following for a rectifier.

(i) RMS value	(ii) Ripple factor  (iii)  Regulation  (iv)  PIV

2.a)	Define the large signal current gain  (in C B configuration) and                         (in CE configuration ) and obtain the relationship between them.

b)	How the SCR is used as a controlled rectifier. Explain with the help of necessary circuit diagram and wave forms.

3.a)	Draw the voltage series, voltage shunt, current series and current shunt feed back amplifiers with discrete components. What is the effect of feed back on input and output impedances on each of the above topologies?

b)	Draw a transistor amplifier using self bias (CE configuration) and explain the operation. What is the type of feed back in the presence of emitter resistance?

4.a)	Briefly explain the operation of electromechanical timers.

b)	Discuss important controls used in resistance welding.

5.a)	Give the merits of induction heating as compared to conventional methods of heating in industry. What are the drawbacks of induction heating.

b)	Explain the principle of dielectric heating. List the applications of dielectric heating.

6.a)	Give the applications of C R O.

b)	Describe in detail the method of measurement of frequency using Lissajous figures.

7.a)	Write an assembly program to add two 8-bit numbers.

b)	Explain the functions of an ALU.

8.a) 	Which is the fastest ADC and why?

b) 	What do you mean by quantization error in A-to-D converters?

c) 	Define the terms:

(i) Stability (ii) Accuracy (iii) Conversion time (iv) Settling time.

@@@@@