Code No: 211051

OR

II B.Tech. I-Semester Supplementary Examinations, November-2003

ELECTRICAL AND ELECTRONCIS MEASUREMENTS

(Electronics and Instrumentation Engineering)

Time: 3 hours							      Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	With a neat diagram explain the principle of operation of a electrodynamometer. What are its advantages and disadvantages over PMMC.

b)	With neat circuit diagrams explain the operation of a series type and a shunt type ohmmeter. Also discuss in detail about the calibration of both types.

2.a)	With a neat diagram explain how a D’Arsonval  movement can be used for the measurement of DC voltages. Derive the expression for series resistance Rse (multiplier resistance).

b)	Convert a basic D’Arsonval movement with an internal resistance of 50 Ω and a full seale deflection current of 2 mA into a multirange DC voltmeter with voltage ranges of 0-10V, 0-50V, 0-100V and 0-250V.

3.a)	With neat circuit diagrams explain the principle of operation of d.c. coupled and a.c. coupled peak responding voltmeter. What are their advantages and disadvantages.

b)	With neat circuit diagram, explain the principle of operation of True RMS voltmeter. Discuss about its advantages and disadvantages over the other types.

4.a)	With the help of a neat construction diagram explain the operation of a cathode ray tube (CRT).

b)	Explain about the following units in a CRO.

(i)   Vertical deflection system (ii)  Delay line (iii) Horizontal deflection system.

5.a)	With a neat block diagram explain the principle of operation of a storage oscilloscope.

b)	Discuss in detail about various types of probes used in the CRO measurements.

6.a)	With neat block diagram explain the principle of operation of AF sine and square wave generator.

b)	Give typical front panel details of a signal generator.

7.a)	With a neat block diagram explain the principle of operation of an universal counter.

b)	Discuss about the applications, merits and demerits of an universal counter.

8.a)	Write short notes on any TWO of the following:

(i) Instrument transformers	(ii) Phase angle meters (iii) RF signal generators.

-*-*-*-