Code No. RR212201

Set No.

1

II B.Tech. I Semester  Regular Examinations November, 2003

PRIME MOVERS AND MECHANICAL MEASUREMENTS

(Instrumentation and Control Engineering)

Time: 3 hours							         Max. Marks:80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	How do you determine the impact of jet when it strikes a fixed flat plate held

normal to the direction of jet?

b)	A jet of water of diameter 5 cm strikes a flat plate held normal to the direction of jet with a velocity of 28 m / sec. What is the force exerted on the plate if it is moving at a velocity of 10 m /sec in the direction of the jet.

2.a)	Draw  inlet velocity triangle for radial entry of a centrifugal pump

b)	The impeller of a centrifugal pump has outer diameter of 40cm and inner diameter of 20 cm. The blade angle at outlet is  30. The speed of the impeller is 1450 rpm. The velocity of  flow at inlet and out let is same at 2.2 m/ sec. Find head  developed if manometric  efficiency is 75%, absolute  velocity at out let and blade angle at inlet.

3.	Explain the principle of operation of Babcock and Wilcox boiler with the help of neat sketch showing various mountings and accessories.

4.a)  	Explain various methods of compounding of steam turbine stages.

b) 	Steam issues from the nozzle of a single stage impulse turbine with a velocity of 850 m/sec. The blades move at 350 m/sec. The blades are equiangular (1= 2=36). Steam enters the blade without shock. Neglecting friction in the blade passages, find

- the angle at which the nozzles are inclined  to the direction of motion of the blades  and

ii)	The diagram efficiency

5.a) 	Discuss the importance of intercooling and reheating of a gas turbine power         cycle to improve the performance?

- A gas turbine plant is supplied with air at a pressure of 1 bar and 300 K. The air is then compressed to a pressure of 5 bar and then heated to 850 K. The efficiency of compressor and turbine is 82% and 85 % respectively. Calculate overall efficiency of the cycle?

6.      List the various operational errors of the bourdon pressure tube and explain how the errors can be reduced?

(Contd…2)

Code No:RR212201				:: 2 ::				Set No.1

7.a)	Explain the principle of working and salient features of Piezo-electric type accelerometer.

b)	Explain the difference between Vibrometers and accelerometers.

8.a)	Suggest and explain a device to measure torque delivered by an electric motor.

b)	What is cradling? Explain with neat sketches.

-*-*-*-

Code No. RR212201

Set No.

2

II B.Tech. I Semester  Regular Examinations November, 2003

PRIME MOVERS AND MECHANICAL MEASUREMENTS

(Instrumentation and Control Engineering)

Time: 3 hours							         Max. Marks:80

Answer any Five questions

All questions carry equal marks

1.a)	      How do you determine the force exerted by a water jet on an inclined and stationary

plate ?

b)      A 10 cm diameter water jet having a velocity of 12 m / sec impinges on a plane

plate at an angle of 60 degrees to the normal to the plate. What will be the impact

when the plate  is moving at 5 m /sec in the direction of the jet. What is the work

done per unit time.

2.a)	What is an indicator diagram? Explain with sketches

b)	A single stage centrifugal pump has an impeller of 30 cm diameter  rotating at 1500 rpm. It delivers 2 m3 / sec to a height of 20 m at an efficiency of 70%. What is the number of stages of a similar multistage pump to lift 3 m3/ sec to 40 m running at 1000 rpm.

3.	Explain the different aspects upon which boilers are classified.

4.a)	Explain briefly why in multistage impulse turbines the first stage is often compounded for velocity and remaining having single row wheels.

b) 	For a stage of impulse turbine with single acting wheel and equiangular blades, the nozzle angle is 20. The velocity coefficient for the blades is 0.83. What is the maximum blade efficiency?

If the blade efficiency is 90% of maximum values, what are the possible ratios of blade speed to steam speed.

5.a) 	State the assumptions made for the estimation of thermal efficiency of gas       turbine power cycle?

- A gas turbine plant receives air at 1 bar and 293 K. Find the thermal efficiency of the plant if the compression ratio is 4 and the ratio of specific heats is 1.4

6.         List the various methods of flow measurement. What is the significance of term

’Inferential’ as applied to flow meters?

7.a)	Describe the need and importance of Vibration measurement.

b)	Desribe the basic concept of the Seismic instrument. Under what conditions is a Seismic instrument suitable for acceleration measurement?

8.a)	Suggest and explain a device to measure torque delivered to a generator.

b)	List the different means normally used for torque measurement.

-*-*-*-

Code No. RR212201

Set No.

3

II B.Tech. I Semester  Regular Examinations November, 2003

PRIME MOVERS AND MECHANICAL MEASUREMENTS

(Instrumentation and Control Engineering)

Time: 3 hours							         Max. Marks:80

Answer any Five questions

All questions carry equal marks

- - -

1.a)      Determine the force exerted by a jet of fluid on a normal plate when it is moving

in the direction of the jet.

b)  	    A 24 cm diameter jet of oil of relative density 0.87 strikes a stationary flat plate at

an angle of 35 degrees to the normal. Calculate the force exerted on the plate

when velocity of the jet is 16m / sec. Also calculate the force when the plate is

moving with a velocity of 8m /sec  in the direction of the jet.

2.a)	What is the effect of air vessel on the indicator diagram?

b)	A centrifugal pump runs at 800 rpm and delivers 5 m3/ sec against a head of        7 m. The impeller has an outer diameter of  25 cm and width of 5 cm at outlet. If the vane angle at outlet is 50, determine the manometric efficiency .

3.	Explain the working principle of Benson boiler with the help of neat sketch. Also list out its advantages and disadvantages over other boiler plants.

4.a) 	Explain the essential differences in the functions of moving blades in the impulse and reaction types of the turbines.

b) 	The speed of a single stage impulse turbine is 300 rev/min and the blade diameter is 1 meter. The nozzle of the turbine is inclined at 20 to the plane of the wheel and the moving blade inlet and exit angles are 35 and 30. Assuming a friction factor (blade velocity coefficient) of 0.8, determine

- The power developed for a steam consumption rate of 9000 kg/hr

- The blade or diagram efficiency

- The axial thrust

Assume that the steam enters the blades without shock.

5.a) 	Explain the reheating gas turbine power cycle?

- The minimum and maximum temperature limits in a gas turbine plant are 288 K     and 1100 K. The pressure limits are 1 bar and 8 bar. Determine (a) optimum pressure ratio for maximum work (b) optimum pressure ratio for maximum thermal efficiency?

6.      Compare and contrast the use of venturimeter, flow nozzle and orifice meter as  primary element for flow measurement.

(Contd…2)

Code No:RR212201				:: 2 ::				Set No.3

7.a)	Explain the construction and working of variable Reluctance accelerometer.

b)	What is proving ring? How is it used to measure force?

8.a)	Suggest and explain a device to measure torque transmitted by a rotating Shaft.

b)	Suggest and explain a device to measure torque for hydraulic pump or an air Compressor.

-*-*-*-

Code No. RR212201

Set No.

4

II B.Tech. I Semester  Regular Examinations November, 2003

PRIME MOVERS AND MECHANICAL MEASUREMENTS

(Instrumentation and Control Engineering)

Time: 3 hours							         Max. Marks:80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	How do you employ momentum equation to find the force exerted by a jet of

fluid on  an inclined plate which is moving in the direction of fluid jet.

b)     A 15 cm diameter jet of water with a velocity of 15m/sec strikes a plane normally.

If the plate is moving with a velocity of 6m/sec in the direction of the jet, calculate

the work done per second and the efficiency of energy transfer.

2.a)	Derive the equation for the work done by the impeller of a centrifugal pump

b)	A centrifugal pump delivers water against a net head of 10 m at a  speed of 1000 rpm. The vanes are curved backward and make an angle of 30 degrees. The impeller outside diameter is 30 cm and  has a width of 5 cm at the outlet. Determine the discharge if manometric efficiency is 95%.

3.	Explain the terms equivalent evaporation, boiler efficiency, economiser efficiency    and boiler horse power.

4.a) 	Explain briefly, various methods used to improve the performance of  Rankine cycle

b) 	The steam from nozzles of a single wheel impulse turbine discharges with a velocity of  600 m/sec and at 20 to the plane of the wheel. The blade wheel rotates at 3000 rev/min and the mean blade radius is 590 mm. The axial velocity of the steam at exit from the blades is 164 m/sec and the blades are symmetrical. Calculate:

- The blade angles,

- The diagram work per unit mass flow rate of steam

- The diagram efficiency

- The blade velocity coefficient

5.a) 	What are the advantages and disadvantages of closed cycle gas turbine power cycle over open cycle gas turbine cycle?

- A simple closed cycle gas turbine installation works between the temperature limits of 300 K and 1000 K and pressure limits of 1 bar and 7 bar. If 1.25 kg/s of air circulated, then calculate power developed by the turbine. Take cp = 1.005 kJ/kg K and γ = 1.4.

(Contd…2)

Code No:RR212201                                                :: 2 ::				Set No.4

6.        Compare the advantages and limitations of using venturimeter and Orifice Meter

in fluid flow measurements. Comment on the conclusions you draw from the differences.

7.a)	Explain the working of a hydraulic load cell for the measurement of force.

b)	What are the quantities involved in Vibration measurements? Describe them and give relationships between them.

8.a)	Suggest and explain a device to measure torque for hydraulic motor or an engine.

b)	Describe a procedure you would follow to mount for strain gauges on a mild steel circular shaft for torque measurement.

-*-*-*-