Code No: RR-212102

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

## Fundamentals of Aeronautical Engineering

(Aeronautical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Classify the various aerospace vehicles based upon design features, wing configuration and roles to be accomplished.

b)	What are the various types of tail configurations of a fixed wing aircraft? Explain their functioning with the help of sketches.

2.a)	Distinguish between nose wheel type and tail wheel type under carriages. What is the need to have a retractable under carriage for high-speed aircraft?

b)	Write a short note on VTOL aircrafts.

3.	Establish the relationship between pressure, temperature and altitude for isothermal and gradient layers of standard atmosphere.

4.	Distinguish between conventional and powered controls. Explain the functioning of a typical powered control system.

5.a)	Define the terms: camber, chord, leading edge, slat, single slotted flap, trim tab, taper ratio and washin.

b)	Distinguish between monoplane and biplane

c)	Define static and dynamic stability.

6.	Distinguish between monocoque and geodesic construction. Sketch a typical wing with semi-monocoque construction showing various parts.

7.	Differentiate between piston engine and turboprop engine. Explain the working of a turboprop engine with the help of a sketch.

8.a)	Define Kepler’s laws of planetary motion.

## b)	Explain the principle of operation of a rocket. Explain briefly the various types of rockets.

@@@@@

Code No: RR-212102

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

## Fundamentals of Aeronautical Engineering

(Aeronautical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Distinguish between fixed wing aircraft, tilt rotor aircraft and helicopter with respect to its design features.

b)	How composite aircrafts are superior to metallic aircraft?

c)	Define and explain the importance of aerodynamic center and center of pressure.

2.a)	How do you classify the various flight vehicles based on load factor, wing location and undercarriage?

b)	What are the basic flight instruments required for safe flight of an aircraft? Explain the importance of these flight instruments.

3.	What do you understand by fixed and moveable surfaces of an aircraft?

Explain their significance and show them with the help neat and labeled sketch. Explain the functioning of moveable surfaces.

4.a)	Calculate the standard atmosphere values of temperature, pressure and density at the geo-potential altitude of 18 km.

b)	Explain the terms mach number, critical mach number and drag divergence mach number.

5.a)	Explain the evolution of lift, drag and moment of an aircraft.

b)	Sketch the lift curve slope for symmetrical, cambered and combination of cambered and leading edge slat aerofoil on the same plot. Explain the difference between the three briefly.

6.a)	Write a short note on truss type structures.

b)	Explain the following terms along with their function:

(i) bulkhead  (ii)stringers (iii)spars (iv) ribs (v) turnbuckle and  (vi) fixed tab

7.a)	Distinguish between ballistic, skip and glide re-entry.

b)	Explain the principle of rocket. Explain the major components of a rocket.

8.a)	Explain the various components of a piston engine. What are the various types of piston engine? Explain the functioning of a piston engine.

b)	Explain the situations when either propeller thrust or jet thrust is preferred. Explain their merits.

@@@@@

Code No: RR-212102

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

## Fundamentals of Aeronautical Engineering

(Aeronautical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain briefly the development in the field of aircraft propulsion over the years.

b)	Explain the following terms and their functions:

i) Flaperons (ii) Stabilators (iii) Rudddervators (iv) Elevons (v) Spoilers (vi) Winglets

2.a)	Distinguish between primary and secondary control surfaces of a fixed wing aircraft. With the help of   a sketch show primary and secondary control surfaces.

b)	Classify the various flight vehicles based upon the load factor and wing configuration.

3.a)	Consider an airplane flying at some real altitude. The outside pressure and temperature are 2.65 x 104  N/m2  and 220 K respectively. What are the pressure and density altitude?

b)	Establish the relationship between geo-potential and geo-metric altitude.

4.a)	Explain with the help of a sketch the phenomenon of flow separation over an aerofoil section.

b)	Distinguish between swept back, swept forward and rectangular wings.

5.a)	Differentiate semi-monocoque and geodesic construction. Sketch a typical monocoque fuselage construction.

b)	Write a short note on main rotor of a helicopter.

6.a)	Define aspect ratio. What are the merits and the demerits of high and low aspect ratio wings?

b)	Explain the principle of operation of a propeller. Differentiate between fixed pitch and variable pitch propeller.

7.a)	Explain the working of a turbojet engine with the help of a sketch.

b)	What are the advantages and the disadvantages of low wing, mid wing and high wing aircraft?

8.	Consider a rocket engine burning hydrogen and oxygen; the combustion chamber pressure and temperature are 25 atm and 3517 K respectively. The area of rocket nozzle throat is 0.1 m2. The area of exit is designed so that the exit pressure exactly equals ambient pressure at a standard altitude of 30 km. For the gas mixtures assume that =1.22 and the molecular weight M=16. at a standard altitude of 30 km calculate (a) specific impulse (b) thrust (c) area of exit (d) flow mach number at exit.

@@@@@

Code No: RR-212102

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

## Fundamentals of Aeronautical Engineering

(Aeronautical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	How do you classify the various flight vehicles.

b)	Describe the modern techniques used on an aircraft for

(i) reduction of induced drag (ii) reduction of profile drag

2.a)	How do the primary controls of a helicopter differ from the fixed wing aircraft?

b)	Write a short note on space shuttle.

3.a)	What is stalling? What are the various methods to delay stalling?

b)	At 12 km in standard atmosphere, the pressure density and temperature are         1.9399 x 104 N/m2, 0.31194 kg/m3 and 216.66K respectively. Using these values calculate the standard atmospheric values of pressure density and temperature at an altitude of 18 km and check with the standard altitude table.

4.a)	What are the advantages and disadvantages of (i) stressed skin construction              (ii) monocoque construction

b)	Write short notes on ‘Fail safe’ and ‘Safe life structures’.

5.a)	Explain the typical system for control actuation of basic instruments for flying.

b)	Differentiate between conventional and powered controls.

6.a)	Explain briefly the 4-digit, 5-digit and 6-digit series of NACA airfoils.

b)	Strength to weight ratio is the main criteria in the design of airplane structure. Explain the importance of the statement.

c)	Write short notes on VTOL and STOL aircrafts.

7.a)	Differentiate between turboprop and turbofan engines. Explain the working of a turbofan engine with the help of a neat sketch.

b)	Explain the terms:

(i) stagger and decalage (ii) washin and washout (iii) slat and slot

8.a)	What are the salient features of re-entry vehicles?

b)	Derive the expression for specific impulse of a rocket engine.

c)	Explain briefly “flying wing”.

@@@@@