Code No:RR-212202

Set No.

1

II B.Tech. I-Semester Regular Examinations, November- 2003.

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Instrumentation and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Explain in detail the Permanent magnet moving coil mechanism with construction and    temperature compensation.

2.	Explain the (a) Electrodynamometer type (b) Rectifier Type of AC measuring instruments.

3.a)	Explain two types of AC voltage measurements using rectifier circuits.

b)	The symmetrical square wave voltage shown in figure below is applied to an 	average responding AC volt meter with a scale calibrated in terms of the r.m.s 	voltage of a sine wave. Calculate (i) The form factor of the square wave voltage 	(ii) error in the meter indication.

e

Em

t

T

4.	Explain the working of capacitance measuring meter using Phase shift characteristic of RC circuit. What are its applications?

5.a)	Explain the block diagram of the vertical section of the oscilloscope.

b)	Explain working function of uncompensated and simple compensated attenuator used for vertical deflection system.

6.a)	Explain the working operation of a storage CRT with multiple targets and two electron guns with secondary emission curves.

b)	With neat figure, explain schematic view of a bistable storage tube.

Contd….2

Code No:RR-212202				-2-				Set No:1

7.a)	Mention different types of signal generators and explain their working principles.

b) 	Explain in detail about conventional standard signal generator.

8.a)	Draw and explain the logic diagram of a time base used for a frequency     counter.

b)	Draw and explain the input signal processing circuit for the frequency          counter.

c)	Define gating error.

- - -

Code No:RR-212202

Set No:

2

II B.Tech. I-Semester Regular Examinations, November- 2003.

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Instrumentation and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Derive the expression for Rs Source Resistance for both DC ammeter and DC voltmeter and also discuss their types in detail.

2.a)	Explain in detail the measurement of  power using electrodynamometer.

b) 	Write short notes on watt hour meter.

3.a)	Explain i) R.M.S voltage ii) Form Factor for a sinusoid

- A saw tooth voltage is applied to an average responding AC voltmeter with a scale calibrated in terms of the R.M.S value of a sine wave. Calculate (i) The form factor of the saw tooth voltage   (ii) Error in the meter indication.

e

150 v

3s                t

T

4.a)	Explain Q meter measurement of a low impedance component in the series connection.

b)	Explain residual inductance of a Q meter.

5.a)	Explain the working operation of differential deflection amplifier for an oscilloscope.

b)	Give the specifications of CRO.

6.a)	Explain the working functions of each block of a sampling oscilloscope in full detail.

b)	Compare the merits and demerits of sampling oscilloscope.

Contd….2

Code No:RR-212202				-2-				Set No:2

7.a)	Explain how you generate sine waves from function generator. Draw the circuit and describe its working.

b) 	What is the principle employed in the operation of a function generator. Mention the applications of function generation.

8.a)	Explain with the help of a block diagram how the period can be               measured?

b)	What is meant by time base error and explain a calibration method to improve the accuracy of it.

- - -

Code No:RR-212202

Set No:

3

II B.Tech. I-Semester Regular Examinations, November- 2003.

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Instrumentation and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Explain the voltmeter sensitivity and measuring method with loading effect.

2.a)	Explain the thermo instrument for measuring both AC and DC.

b) 	Write short notes on crossed-coil PF meter.

3.a)	Give a block diagram of a true reading voltmeter. Explain the use of thermo couples used

b)	A 25 Milliamps full-scale current meter with an internal resistance of 100 Ohms is available for constructing an AC voltmeter with a voltage range of 200 V rms. Using 4 diodes I a bridge arrangement where each diode has a forward resistance of 500 ohms and infinite reverse resistance, calculate the necessary series limiting resistance for the 200 V rms range

4.	Explain in detail a vector impedance meter with a block diagram. Give its specifications.

5.a)	Explain the block diagram of a dual trace oscilloscope.

b)	Why is a delay line used in the vertical section of the oscilloscope?

6.a)	Explain the working function of each blocks of a digital storage oscilloscope.

b)	How does the digital storage oscilloscope differ from the conventional storage oscilloscope using a storage CRT? What are the advantages of each?

7.a)	What are the important characteristics of a signal generator? Explain the same.

b) 	Explain briefly about radio frequency synthesized signal generators.

8.a)	Draw and explain the temperature compensated crystal oscillator circuit.

b)	List the suggestions to be followed to attain maximum accuracy in a frequency counter.

c)	Explain the basic principle behind the extension of frequency range of        counter.

- - -

Code No:RR-212202

Set No:

4

II B.Tech. I-Semester Regular Examinations, November- 2003.

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Instrumentation and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Derive the expression for Rh in shunt type ohm-meter. Also prove with an example its suitability for very low resistance measurement.

2.	Explain in detail the principle, working, and constructional details of current transformer and potential transformer.

3.a)	Give the schematic diagram of a balanced bridge DC amplifier used in electronic analog voltmeters and explain its working.

b)	What would true RMS reading meter indicate if a pulse wave from of 5 volts peak and a 25% duty cycle applied? What would the meter indicate if a 5volt DC input were applied (assume the meter has DC capability.

4.	Explain the working of a vector voltmeter in detail. Draw the schematic. Mention its applications.

5.a)	Explain the working operation of triggered time base for an oscilloscope.

b)	Why horizontal and vertical deflections are required for an oscilloscope.

6.a)	Briefly explain the working operation of storage and sampling oscilloscope.

b)	What precautions must be taken when using a sampling oscilloscope?

7.a)	Distinguish between spectrum analyzer and wave analyzer.

b) 	Explain in detail about different characteristics of a signal generator.

8.a)	Explain an automated frequency counter using block diagram.

b)	If the internal time base of a frequency counter is 15MHz, what frequency range is best measured by a period measurement and why.

- - -