OR

Code No: 210454

II B.Tech. I-Semester Supplementary Examinations, November-2003

ELECTRICAL ENGINEERING

(Common to Electronics & Communication Engg., Chemical Engineering, Metallurgy and Material Technology, Comp Science & Information Tech., Electronics & Instrumentation Engineering, Electronics & Control  Engineering and Biomedical Engineering)

Time: 3 hours							      Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the coefficient of Mutual Inductance.

b)	Two identical coils X and Y of 1000 turns each lie is parallel planes such that 80%.  If flux produced by one coil links with the other.  If a current of 5A flowing in X produces a flux of 0.5m wb is it, find the mutual inductance between X and Y.

2.a)	Define peak factor, form factor.

b)	Calculate form factor for the sinusoidal wave form.

3.a)	 In a d.c machine the total iron loss is 8KW at its rated speed and excitation. If  the excitation remains the same, but speed is reduced by 25% the total iron loss  is found to 5KW. Calculate the hysteresis and eddy current losses at (i) Full  speed (ii)half the rated speed.

b) 	Describe in detail the various losses occurring in a d.c. generator.

4.a)	A dc motor takes an armature current of 110A at 480V.  The armature circuit resistance is 0.22 Ω .  The machine has 6poles and the armature is lap- connected with 864 conductors.  The flux/pole is 0.05 wb.  Calculate the gross torque developed by the motor.

b)	A 100H.P 500v shunt motor has 4 poles and a 2 circuit wave winding with 492 armature conductors, the flux is 50 mwb/pole and the full load efficiency 92%. The armature and commutating field winding have  a total resistance of 0.1ohm.  The shunt field resistance is 250 Ω.  Calculate for full load (i) the speed (ii) the useful torque.

5.a)	Distinguish between the efficiency and Regulation of a Transformer. Show how power affects both of them with necessary equations

b)	The efficiency of a 250 KVA, Single phase Transformer is 96% when delivering full-load at 0.8 power factor lagging and 97.2% when delivering half full load at unity power factor. Determine the efficiency at 75% of full load at 0.8 power factor lagging.

Contd.2

Code No: 210454				-2-				OR

6..a)	Derive the Torque equation of an Induction motor.

b)	A 3 phase 50Hz, Induction motor is wound for 8 poles. If the full load slip is

2.5% , calculate

i) Synchronous speed               ii) Slip speed

iii) Rotor speed                        iv) Rotor frequency.

7.	The following test results are obtained on a 6600V alternator.

OC Voltage in volts       3100	4900	6400	7500	8300

Field current in Amps     16	  25	  37.5	   50	  70

A field current of 20 Amps is required to circulate full load current as short circuit of armature. Calculate full load   regulation at 0.8 PF lag by synchronous impedance method.

8.		A moving coil instrument which given full scale deflection with 15mA, has a copper coil having a resistance of 1.5 at 150C, and a temperature coefficient of 1/234.5 at 00C in series with a resistor of 3.5 having a negligible temp coefficient. Determine i) the resistance of shunt required for a full scale deflection of 20A and ii) the resistance required for a full scale deflection of 250v. If the instrument reads correctly at 150C, determine the percentage error in each case when the temperature is 250C.

&&&