Code No:RR-210801

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

FLUID MECHANICS

(Chemical Engineering)

Time:3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What do you mean by fluid statics? And how it is applied in practice.

b)	With a neat sketch show that the pressure at a point in a static fluid is equal in all 	directions.

2.	Define the equation of continuity. Obtain an expression for continuity equation for a three dimensional flow.

3. a)	What are the minor losses ? Under what circumstances will they be negligible?

b)	The diameter of a water pipe is suddenly enlarged from 350mm to 700 mm .  The

rate of flow  is 0.25 m3/sec. And the pressure in the smaller pipe is 7.5 N/m2.

Calculate. 	(i)	The power lost due to enlargement

- The pressure in the larger pipe if the pipeline is horizontal

4.a) 	Distinguish between compressible and incompressible fluid flows.

b) 	What is the relation between pressure and density of a compressible fluid for         isothermal process and adiabatic process.

c) 	Air has a velocity of 1000km/hr at a pressure of 9.81kN/m2 vacuum and a temperature of 47oC. Compute its stagnation properties, and the local Mach number. Take atmospheric pressure=9.81kN/m2, R=287Nm/kg oK.

5.	Distinguish clearly between wall drag and form drag. What is drag coefficient? Discuss the effect of Reynold’s number on drag coefficients of different shapes.

6.a)	Explain how the frictional losses in pipe fittings and valves can be accounted for 	design calculations.

b) 	How do you classify pumps?

7.	Discuss the function and application of different types of compressors, with details on compressor selection.

8.	What are the principles behind the two major classes of fluid moving machinery?  Explain with examples.

&&&

Code No:RR-210801

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

FLUID MECHANICS

(Chemical Engineering)

Time:3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Show that the center of pressure always acts below the center of gravity of an immersed surface in a static fluid.

b)	State and prove Pascal’s law.

2.	Define the following:

(a) Steady and Unsteady flow		(b) Uniform and Non-Uniform flow

(c) Laminar and Turbulent flow	(d) Rotational and Irrotational flow

3.a)	Define Hydraulic gradient and total energy line.

b)	A horizontal pipe line 35m long is connected to a water tank at one end and discharges freely in to the atmosphere at the other end.  For the first 20m of its length from the tank,  the pipe is 15 cm diameter and its diameter is suddenly enlarged to 30 cm. The height of water level in the tank is 7.5 m above the centre of the pipe.  Considering all losses, determine the rate of flow.

4.a) 	Derive the continuity equation for one dimensional compressible flow in differential form.

b) 	At a certain section of a duct in which air is flowing at a temperature of 32oC and pressure of 79.95kN/m2 with a velocity of 365m/s. Assuming isentropic flow

(reversible adiabatic) determine:

(i)  the velocity and temperature at a section where the pressure is 122.63kN/m2

(ii)  the Mach number at both the sections.

Take R=287 Nm/kg oK and k for air =1.4.

5.	Explain about fluidization . A column having 36 cm diameter and 1.9 m height is packed with spherical particles of diameter 0.2 cm. The bed is having a voidage of 0.30. Through packed bed an aqueous sucrose solution is to be pumped at the rate of 110 kg/min. The viscosity of the solution is 56 Centi Poise and its density is 1.2889 gm/c.c What would be pressure drop across the bed.

6.	Explain the following in brief.

(a) Gate valves and globe valves  (b) Plug cocks and ball valves  (c) Check valves

7	Write short notes on

(a)	Impellers		(b)  Centrifugal  pump characteristics

8.	Explain full bore meters and insertion meters in detail.

&&&

Code No:RR-210801

Set No.

3

II B.Tech. II-Semester Regular Examinations, November-2003

FLUID MECHANICS

(Chemical Engineering)

Time:3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What is meant by hydrostatic pressure distribution? Derive the equation for the same.

b)	What is Pascal’s Law.  Explain.

2.a) 	Derive the Bernoullis equation.  State the assumptions made.  Write any one application.

b) 	State the uses of Flow-Net and its limitations.

3.a)	What is a syphon ?

b)	A syphon of diameter 200 mm connects two reservoirs having a difference in

elevation of 20 m. The total length of syphon is 800 m and the summit is 5m  above 	the water level in the upper reservoir.  The separation takes place at 2.8m of water absolute .  Find the maximum length of the syphon from upper reservoir to summit. Take f=0.004 and atmospheric pressure = 10.3 m  of water.

4.a) 	Explain momentum equation for compressible fluids.

b) 	Obtain an expression for the sound wave in a compressible fluid in terms of change of pressure and change of density.

c) 	A larger container holds carbon dioxide at a temperature of 15oC and pressure of 6 atmosphere. The gas is discharged through an orifice 10 mm in diameter. Determine the velocity of jet and the mass flowrate. Take k= 1.30 for carbon dioxide and atmospheric pressure =101.24kN/m2, R= 189Nm/kg oK.

5.	Explain the term fluidization. Briefly discuss the different types of fluidization. How do you calculate velocity at on set of fluidization?

6.	Give a diagrammatic representation of a typical pump application.  Applying Bernoulli’s equation find the pump work.  Also write the expressions for power supplied to the pump and power delivered to the fluid.

7.	What do you understand by:

(a) NPSH		(b) Cavitation		(c)  Suction lift

8.	Write short notes on:

a) Pressure Recovery

b) Polytrophic compression.

&&&

Code No:RR-210801

Set No.

4

II B.Tech. II-Semester Regular Examinations, November-2003

FLUID MECHANICS

(Chemical Engineering)

Time:3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Derive an expression for the force exerted on a submerged vertical plane surface in a static liquid and locate the position of center of pressure.

b)	What is pressure diagram?

2.a)	State the Bernoullis equation?  Explain the significance of each term.

b)	Write any two applications of the Bernoullis equation.

3.a)	Explain the terms

(i) Pipes in parallel  (ii) Equivalent pipe

b)	A single pipe 30 cm. diameter, 300m. long conveys a discharge of 0.1m3/sec.

What 	length of another 40 cm. diameter pipe is required to be placed in parallel

with  the exiting pipe in order to augment the discharge by 30% ? Take f=0.015

and neglect the minor losses.

4.a)	Prove that the velocity of sound  in a  compressible fluid  is given by c = kp where k = Bulk modulus of fluid and ρ is density of fluid.

b)  	A normal shock wave occurs in a duct in which air is flowing at a Mach number

of 1.50. The static pressure and temperature upstream of the shock wave are 1.5 bar and 27oC. Determine the pressure, temperature and the Mach number downstream of the shock. Also calculate the strength of shock. Take k=1.4.

5.	What is drag force? How is it estimated for the flow of a spherical particle traveling in a stagnant fluid? Derive an equation for the estimation of the terminal velocity of particle for such a flow.

6.	Write short notes on the following

(a)   Valves	(b)   Fluid moving machinery.    (c)   Positive displacement pumps.

7.	Explain Positive displacement compressors with figures.

8.	Explain Rota meters in detail.

&&&