Code No: 210256

OR

II B.Tech. I-Semester Supplementary Examinations, November-2003

ELECTRO MECHANICS -I

(Electrical and Electronics Engineering)

Time: 3 hours							      Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Show how mechanical energy output can be determined in the singly excited relay in the non-linear case.

- A doubly excited magnetic field system has coil self and mutual inductances of L11 = L22 = 2, L12 = L21  = cos2θ, where θ is the angle between the axes of the coils. If coil 2 is shorted while coil 1 carries a current of Im = √2 sin 314t, derive expressions for instantaneous and time average torques. If the rotor is allowed to move, at what angle will it come to rest? Plot the average torque variation with respect to θ and explain your result.

2.	Draw the developed armature-winding diagram of a d. c. machine showing commutator segments and position of brushes. The particulars of the windings are given below: Type = simplex retrogressive wave winding; no. of slots = 21; no. of poles = 4;  no. of conductors per slot = 2; no. of commutation segments = 21;  coil span  = 5 slot pitches.

3.a) 	Explain the purpose of laminating the armature core of a d. c. machine.

b)	The diameter of a commutator ring of a lap wound d. c. motor is 25 cms. The brush width is 1.8 cm and width of mica insulation is 0.2 cm.  If the speed of the motor is 200 rpm, determine the time of commutation.

4.a)	What is critical speed? How do you calculate the critical speed in the laboratory?

b)	What are the conditions to build up of emf in a shunt generator?

5.a)	How does a DC motor automatically adjust input to match the mechanical load on the motor?

b)	Explain the armature reaction in DC shunt motors, indicating also a few remedies to its adverse effects.

6.a)	What are the different methods of speed control of DC motor? Explain.

b)	A 220V DC shunt motor draws a no load armature current of 2.5A when running at 1400rpm. Determine its speed when taking an armature current of 60A if armature reaction weakens the flux by 3%.

7.a)	Explain Swinburne’s test of finding the performance of a DC machine working as a motor. What are the advantages and disadvantages?

b)	Write a note on Brake Test on D.C. Motor.

Contd..2

Code No:210256					-2-				OR

8.a)	What are the advantages of the conventional efficiency determination over direct loading? And why direct loading is relatively impossible for large machines?

b)	Two 10 HP motors have each a full load efficiency of 0.83 the running light losses of motor A are 530 W, and of motor B are 420 W. Find the half load efficiency of each motor.

&&&