Code No: RR/NR-210204

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

ELECTROMAGNETIC FIELDS

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	State and explain coulomb’s law.

b)	Two small identical conducting spheres have charge of 2nC and –0.5nC respectively. When they are placed 4 cm apart what is the force between them. If they are brought into contact and then separated by 4cms what is the force between them?

2.	Find the electric field at any point between two concentric spherical shells, inner 	spherical shell has Q1 charge and outer spherical shell has Q2 charge.

3. a)  	State and prove the conditions at the boundary between two dielectrics.

b)	Determine the resistance of a insulation in length ‘L’ of co-axial cable as inner and outer radii are ‘a’ and ‘b’ respectively.

4.	State and prove Ampere’s circuital law. Discuss few applications for the same.

5. a)	What is the force experienced by a charge in a magnetic field?  Obtain Lorentz force equation.

b)	What is the force experience by a differential current element place in steady magnetic field?

6.	Derive formula for self-inductance of a solenoid. use this formula and find self-inductance of a solenoid having 500 turns, mean diameter equal to 10 cm and length equal to 5 cm. Assume medium to be air.

7. a)	State and explain the Faraday’s laws in Electro magnetic induction?

b)	A stationary 10turns square coil of 1-meter side is situated with its lower left corner coincident with the origin and with side’s x1 and y1 along x-axis and        y-axis. If the field B is normal to the plane of the coil and has its amplitude given by B0=sinΠxx1sinΠyy1 Tesla.  Determine the r.m.s value of e.m.f. induced in the coil if B varies harmonically at a frequency of 1 KHz.

8.	Write the Maxwell’s equations in free space.

!!!!!

Code No: RR/NR-210204

II B.Tech. I-Semester Regular Examinations, November-2003

Set No.

2

ELECTROMAGNETIC FIELDS

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Define the terms (i) Electric field intensity (ii) Electric potential.

b)	Derive the relation between electric field and electric potential in rectangular                 co-ordinates.

c)	Find the electric field strength at the point (1,-2,1)m given the potential

V = 3x2y + 2yz2+2xyz.

2. a)	Find potential function at any point between spherical shell in terms of applied 	potential using laplace equation.

b)	A parallel plate capacitor consists of two square metal plates with 500mm side and separated by 10 mm. A slab of sulphur (r = 4) 6 mm thick is placed on the lower plate and air gap of 4mm. Find capacitance of capacitor.

3. a)  	Derive ohms law in point form from first principles.

b) 	In a cylindrical conductor of radius 2mm, the current density varies with the distance from the axis according to

J=103 e-400r A/ m 2

Find the total current.

4. a) 	Derive the boundary conditions at the interface of two different magnetic media.

b) 	Describe any two applications of Ampere’s law.

5.	Two long straight parallel conductors are situated in space and spaced D meters apart.  Determine the force between them if the two conductors carry currents in                          (i) Same direction and (ii) Opposite direction.

6.	A torroid is made if closed iron ring wound with 300 turns of insulated copper wire. The cross sectional area of the ring in 5 sq. cm. The mean radius of the ring in 10 cm. Relative permeability of iron is 1000. Find self-inductance and derive formula used.

7. a)	Write down Maxwell’s equations in their general integral form.  Derive the corresponding equations for fields varying harmonically with time.

b)	Show that the ration of the amplitude of the conduction current and displacement current density isσωεfor the applied field E=Emax Cos wt V/m.

8.	Explain the Faradays disc generator and derive an expression for finding the unknown magnetic field.

!!!!!!

Code No: RR/NR-210204

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

ELECTROMAGNETIC FIELDS

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Find E at origin due to a point charge of 65nC located at (-4,3,2)m in Cartesian              co-ordinates.

b)	It is required to hold three point charges +Q each in equilibrium at the corners of an equilateral triangle. Find the point charge which will do this if placed at the centroid of a triangle.

2. a)	Find electric potential due to electric dipole.

b)	The potential difference between two concentric sphere of radii r1 and r2 (r2>r1) show that electric field E at the surface of inner sphere is minimum of 2V/r1 for r1 = r2/2.

3. a) 	Derive the boundary conditions for the electric field intensity at the interface of a        	dielectric medium and conduction  medium.

- The Polarization with in a region having Єr=2.7 has the uniform value                                p= -0.2ax+0.7ay+ 0.3az C/m2. Find (i)E (ii)D (iii) the magnitude of voltage gradient.

4. a) 	Derive an expression for the force between two current carrying conductors in the same direction.

b) 	Derive the boundary conditions at the magnetic interfaces and show that                         tanθ1θ2 = μr1μr2.

5. a)	What is the direction of force between the two conductors if the direction of current in the conductors is same.

b)	A magnetic field Β = 3.5 x 10−2az wb/m2 exerts a force on a 0.30m conductor along the  x-axis.  If the conductor current is 5 amps in the -ax direction, what force must be applied to hold the conductor in position.

6.	Derive an expression for energy density in a magnetic field and use this formula for computing energy density in a magnetic field having flux density equal to 1 Tesla.

7.	A conductor of length 100cm moves at right angles to a uniform field of strength 10,000 Lines per cm2, with a velocity of 50 metres/sec. calculate the EMF induced in it.  Find also the value of the induced EMF when the conductor moves at an angle of 300 to the direction of the field.

8.	Write the Maxwell’s equations for harmonically varying fields?

!!!!!

Code No: RR/NR-210204

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

ELECTROMAGNETIC FIELDS

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	A point charge Q1 = 300C located at (1 ,-1,-3) m Experience a force F1 =  8i –8j + 4k Newton due to point charge at (3,-3,-2)m determine Q2.

b)	A charge Q2 = 10C is located in a air medium at P2 (-3,1,4) m. Find the force on Q2 due to Q1 = 33C at  P1(3,8,-2)m.

2. a)	Find the electric field at any point due to electric dipole.

b)	A line charge is ‘2a’ meter long and has a uniform charge  C/m. Find the 	potential at a point ‘r’ meters from line and located on the plane which bisects the line.

3. a) 	Three point charges 1nC, 3nC and 4nC are located (0,0,0),(0,0,1) and (1,0,0)           respectively. Find the energy in the system.

b) 	Find the expression for the energy per unit volume of the dielectric due to electric field in a charged capacitor.

4.	Obtain ampere’s circuital law in differential form.

5. a)	Find the torque about the Y-axis for the two conductors of length L separated by a distance D in the uniform field B.  The conductors are placed parallel to and symmetrical with the y-axis, carrying a current I in opposite directions.  The flux density is along ax direction.

b)	A conductor located at x=0.4m, y=0 and O<Z<2.0m carries a current of 5.0A in the azdirection.  Along the length of the conductor Β=2.5az Tesla.  Find the torque about the z-axis.

6.	A torroid is made up of closed steel ring of mean diameter equal to 75 cm. The area of cross section of steel ring in 5 sq. cm. and the number of turns of insulated copper wire wound around the ring material is 500. The relative permeability of steel is 2000 and current is the coil in 5 Amps. Find energy density and total stored energy in the ring.

7.	State Maxwell’s equation for free space and prove that they are satisfied by

E=∂A∂T;H= Curl A		provided that DivA=0   	2A=1∂2Ac2∂t2

Drive E and H, when A= [ a cos 2k(Z-ct)b sin 2k(z-ct)] verify that E and Hare perpendicular and their directions rotate about the X-axis with frequency kc.

Contd….2

Code No. RR/NR-210204					.2.			      Set No. 4

8.a)	Distinguish between conduction  and displacement currents.

b)	A Faradays copper disc 0.3m diameter is rotated at 60 rps on a horizontal axis perpendicular to and though the center of the disc, the axis lying in a horizontal field of 20 micro Tesla.  Determine the EMF measured between the brushes.

!!!!!