Code No: RR211803

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003.

CERAMIC SCIENCE AND TECHNOLOGY

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - - -

1.a)	Define the term ceramics.  Give examples for different traditional ceramics.

b)	Give examples and applications for structural and fine ceramics.

2.a)	Give two examples each for:

(i) bio ceramics (ii) superconducting ceramics and (iii) electronic ceramics.

b)	What are bio-ceramics? What are the important characteristics required of a

bioceramic? Give typical applications for bioceramics.

3.a) 	Define nature of ceramic materials.

b) 	Describe spinel structure and graphite diamond structures of ceramics.

4.a)	Define the terms Stochiometric and Phase Boundary.

b)	Draw a neat sketch of Al2O3 and SiO2 Equilibrium Diagram and explain.

5.	List the powder preparation techniques for advanced ceramics and explain them 	briefly.

6.	Critically discuss the high performance ceramic coatings by plasma spray 	technology.

7. a)	Explain the effect of process variables in dry pressing.

b)     Describe powder flow during die filling and pressing.

8.a) 	Explain microwave processing of ceramics.

b) 	What are the advantages and limitations of microwave processing?

-*-*-*-

Code No: RR211803

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003.

CERAMIC SCIENCE AND TECHNOLOGY

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - - -

1.a)	What are ceramics? How are ceramics classified based on composition? Give examples.

b)	Give the characteristics of the following ceramics with applications:			(i)	Porcelains 	 (ii)	Refractories	(iii)	Glass

2.a)	What are the different electronic ceramics? Discuss their important properties with examples and applications.

b)	Classify the following ceramics as conductors, super conductors, bioceramics and semiconductors:

(i) -alumina 	(ii) YbaCuO 	(iii)  hydroxy apatite	(iv) silicon

3.a) 	What are the developments of ceramics?

b) 	Describe frame structure and vitreous structure of ceramics.

4. 	Phase Diagrams are graphical representations of the regions of distinct chemical and structural behavior of materials in thermodynamic equilibrium. Justify.

5.a)	Explain the process of precipitation and co-precipitation of  ceramic compounds 	with suitable examples.

b)	Give the classification of advanced ceramic powders produced by solution route.

6.	Write short notes on the following:

a)	Laser synthesis.

b)	Vapour phase synthesis.

c)	Reaction synthesis.

7.        Discuss a processing flow diagram for producing dry pressed high performance

structural alumina parts.

8.a) 	Describe calendaring. What is its importance?

b) 	Discuss the merits and demerits of calendaring.

-*-*-*-

Code No: RR211803

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003.

CERAMIC SCIENCE AND TECHNOLOGY

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - - -

1.a)	What are the important characteristics of a ceramic?

b)	What are structural ceramics? Discuss the important structural ceramics with

examples and applications.

2.a)	What are ferroelectric ceramics? Give their characteristic properties with examples.

b)	Name the ceramics that are used for coatings for use at high temperature. What are the important characteristics of these ceramics?

3.a) 	How do you compare ceramics and non-ceramic phases?

b) 	Describe silicon-oxygen tetrahedron (SiO4) structure and double poly-tetrahedron                                   	structures of ceramics.

4.a)	Define the terms Phase and Isotherms.

b)	Draw a neat sketch of BaO –TiO2 Equilibrium Diagram and explain.

5.a)	Explain the formation of glassy silica by Sol- gel processing.

b)	Discuss the preparation of advanced ceramics by Sol-gel technique.

6. a)	What are the two important powder production routes. Explain important features 	of the above techniques.

b)	Explain carbothermic and vacuum synthesis processes in detail.

7. a)  	Explain the behavior of  powder during compaction.

b) 	Describe the die wall effects on compaction.

8.a) 	Explain ultrasonic machining.

b) 	Compare ultrasonic machining with conventional methods.

-*-*-*-

Code No: RR211803

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003.

CERAMIC SCIENCE AND TECHNOLOGY

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - - -

1.a)	Give examples for crystalline and non-crystalline ceramics.

b)	Compare the mechanical properties of ceramics with metals.

c)	What are structural ceramics? Give examples.

2.a)	List three important bioceramics and their applications.

b)	How are magnetic ceramics classified? Discuss the different classes with examples. Mention one application for each class.

3.a) 	Explain mechanical behavior of ceramics.

b) 	Discuss various ceramic forming techniques.

4. 	Draw a neat sketch of MgO-Al2O3-SiO2 three component system Phase Diagram

and explain.

5.a)	Explain the procedure for BaTiO3 powder by solid state reaction method.

b)	Explain the importance of precursor powders for the production of fine ceramics.

6. a)	Explain the manufactures of glass fibres by melting technique.

b)	Explain the advantages of solution/chemical route for the production of ceramic 	powders.

7.       	Describe the different stages in Hot Isostatic Pressing (HIPing) for producing

ceramic products. What are its merits and demerits?

8.a) 	Describe Doctor Blade method.

b) 	What are the advantages and draw backs of Doctor Blade method ?

-*-*-*-