Code No: NR-210303

Set No.

1

II B.Tech. I-Semester Supplementary Examinations, November-2003

ELECTRICAL ENGINEERING

(Common to Mechanical Engineering, Chemical Engineering, Mechatronics, Metallurgy and Material Technology, Aeronautical Engineering and Production Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	An iron ring of mean length 50cm has an air gap of 1mm and a winding of 200 turns.  If the permeability of the iron ring is 400 when a current of 1.25 amperes flow through the coil, find the flux density.

2.a)	Briefly explain how alternating voltages are generated.

b)	A square coil of 10cm side and 100 turns is rotated at a uniform speed of 1000 revolutions per minute, about an axis at right angles to a uniform magnetic field of 0.5wb/m2.  Calculate the instantaneous value at induced electromotive force, when the plane at the coil in (i) at right angles to the field (ii) in the plane of the field.

3.(a)	What purpose is served by the pole shoe in a dock. machine?

(b)	What are the advantages and disadvantages of carbon brushes?

(c)  	Why do we use slotted armature in a d.c. machine?

(d)	Why is armature winding placed on the rotor of a d.c. machine?

4.a)	Explain the significance of back emf in a d.c. motor and explain how it controls the value of armature current in d.c. motor?

b)	A 250V shunt motor takes a total current of 20A.  The shunt field and armature resistances are 200Ω and 0.3 Ω respectively.   Determine i) Value of back emf    ii) gross mechanical power in the armature.

5.a)	On what factors the induced EMF  in the transformer windings depends. Justify

the answer with appropriate derivation.

b) 	A double wound 1- phase transformer is required to step down from 1900V to 240V,

50Hz . It is to have 1.5V per turn . Calculate the required number of turns on the primary and secondary windings respectively. The peak value of flux density is required to be not more than 1.5 wb/m2 . Calculate the required cross sectional area of the steel core. If the output is 10KVA . Calculate the secondary current.

Contd..2

Code No:NR-210303			-2-				Set No.1

6.a)	Explain the principle of operation of a 3 phase induction motor.

b)	A 3 phase, 4 pole, delta connected induction motor has a full load slip of 5%.             If the supply frequency is 50Hz. Find the full load speed, synchronous speed            and rotor frequency.

7.a) 	What is an alternator and what is its principle of operation?

b)	A 600V , 60 KVA , 1-phase alternator has an effective resistance of 0.2  Ohms . A field current of 10 Amps  produces  an armature current of 210  Amps on short circuit and an EMF of 480 V on open circuit. Calculate i) Synchronous impedance  ii) Synchronous reactance   iii) Full load regulation at 0.8 P.F  lagging.

8.a)	Sketch and describe the construction of a moving coil ammeter and give the principle of operation.

b)	A moving coil instrument gives full scale deflection with 15mA and has a resistance of 5. Calculate the resistance of the necessary components in order that the instrument may be used as i) a 2A – Ammeter ii) a 100V voltmeter

&&&

Code No: NR-210303

Set No.

2

II B.Tech. I-Semester Supplementary Examinations, November-2003

ELECTRICAL ENGINEERING

(Common to Mechanical Engineering, Chemical Engineering, Mechatronics, Metallurgy and Material Technology, Aeronautical Engineering and production Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the “Faraday’s laws of Electromagnetic Induction”.

b)	A coil of resistance 100 is placed in a magnetic field of 1mwb.  The coil has 100 turns and a galvanometer of 400 resistance is connected in series with it.  Find the average e.m.f and the current on the coil is measured in 1/10th second from the given field with a field of 0.2m wb.

2.a)	Define cycle, frequency, phase.

b)	An alternating current at frequency 60Hz has a maximum value of 120A.  Write down the equation for its instantaneous value.  Reckoning time from the instant  the current zero and is becoming positive find (i) the instantaneous value after 1/360 second and (ii) the time taken to reach 96A for the first time.

3.a)	A 4 pole d.c. generator has a wave wound armature with 792 conductors. The flux/pole is 0.0121Wb. Determine the speed at which it should be run to generate  240V on no load.

b)	A d.c. generator generates an emf of 520V. It has 2000 armature conductors, flux/pole of 0.013Wb, speed of 1200rpm and the armature winding has 4 parallel paths. Find the number of poles.

c)  	When driven at 1000rpm with a flux/pole of 0.02Wb, a d.c. generator has an emf

of 200V. If the speed is increased to 1100rpm and at the same time the flux/pole is reduced to 0.019Wb/pole, what is the induced emf?

4.a)	Give the voltage equation and power equation of a dc motor.

b)	A 20KW, 250V dc shunt generator has armature and field resistance of 0.1 and 125 	respectively. Calculate the total armature power developed when running (i)  as a generator delivering 20KW output. (ii) as a motor taking 20 KW input.

Contd..2

Code No:NR-210303			-2-					Set No.2

5.a) 	In a test for the determination of the losses of a 440V, 50Hz transformer, the total           iron losses were found to be 2500W at normal voltage and frequency. When the         applied voltage and frequency were 220V, 25Hz, the iron loss were found to be 850W.

Calculate the hysterisis and eddy current losses at normal voltage and frequency.

(b) 	The following readings were obtained from OC and SC tests on 8KVA, 400/120V,

50Hz, transformer.

OC Test     on LV side       120V            4A             75W

SC Test     on HV side       9.5V           20A          110W

Calculate the voltage regulation and efficiency at full load  0.8 P.F lagging.

6.a)	Explain the rotating magnetic field developed in an Induction Motor.

b)	A12 pole 3- phase alternator coupled to an engine running at 500rpm. It        supplies an induction motor which has a full load speed of 1440rpm. Find the % slip and the number of poles of the motor.

7.a)	Why do we prefer field winding of an alternator as a rotating element and   armature as a stationary element, give reasons.

b) 	A 3- phase star connected alternator is rated at 1500KVA, 12KV. The armature effective resistance and synchronous reactance are 2 Ohms and Ohms  per phase respectively. Calculate % regulation for a load of 1200KW at 0.8 PF lag and 0.707 PF lead.

8.a)	Why is spring control to be preferred to gravity control in an electrical measuring instrument?

b)	The coil of a moving coil meter has resistance of 5 and given full scale deflection when a current of 15mA passes through it. What modification must be made to the instrument to convert it into

i) an ammeter reading to 15A   ii) a voltmeter reading to 15V?

&&&

Code No: NR-210303

Set No.

3

II B.Tech. I-Semester Supplementary Examinations, November-2003

ELECTRICAL ENGINEERING

(Common to Mechanical Engineering, Chemical Engineering, Mechatronics, Metallurgy and Material Technology, Aeronautical Engineering and production Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1. a)	State and explain “Lenz’s law”.

b)	Explain how the force in a current carrying conductor is calculated?

2.a)	Define root mean square value, average value.

b)	For the sinusoidal wave calculate R.M.S., Average Values.

3.a)	A four pole d.c generator runs at 750rpm and generates an emf of 240V. The armature is wave wound and has 792 conductors. If the total flux per pole is

0.0145wb, what is the leakage coefficient.

b)	A 1500KW, 550V, 10pole generator runs at 150rpm. There are 2500 lap connected conductors and the full load copper losses are 25KW. The air gap flux density has a uniform value of 0.9wb/m2. Calculate (i) The no load terminal  voltage (ii) The area of pole shoe

4.a)	Derive the condition for maximum power in a dc motor

b)	A 4-pole, wave-connected shunt motor gives 11.19KW when running at a 1000rpm and drawing armature and field current of 50A and 1A respectively.  It has 540 conductors.  Its resistance is 0.1 Ω.  Assuming a drop of 1 V per brush,  find i)total torque  ii) useful torque iii) useful flux/pole iv) rotational losses e)efficiency.

5.a)	A 1-phase transformer working at 0.8 power factor has an efficiency of 94% at  both  three forth full load and at full load of  600 KW. Determine the efficiency at

half full load

b)	10 KVA transformer  has  a turns ratio of 500/250V. The primary winding is 	connected to 1500V, 50Hz, supply. Calculate

i) The secondary voltage  on  open circuit.

ii) Primary and secondary full load currents

iii) The maximum flux in the core.

6.a)	 How do you classify the induction motors, give the constructional details of them.

b)	Define slip and give an account of the quantities which vary with slip.

Contd..2

Code No:NR-210303			-3					Set No.3

7.a) 	Define % regulation and how do you find regulation of an alternator experimentally.

b) 	A 3-phase, 4 pole star connected turbo alternator has a cylindrical rotor. The reactance and resistance per phase of winding is 2.5 Ohms and 0.15 Ohms. The alternator has terminal potential difference of 6600V when delivering a current of 250 Amps. Calculate

- The generated EMF at 0.6 PF lagging.

ii)	 The regulation at 0.6 PF lagging.

8..a)	Draw a diagram to show the essential parts of a modern moving coil instrument. Label each part and state its function.

b)	A moving coil milliammeter has a coil of resistance 15 and full scale deflection is given by a current of 5mA. This instrument is to be adapted to operate i) as a voltmeter with a full scale deflection of 100A ii) as an ammeter with a full scale deflection of 2A. Sketch the circuit in each case.

&&&

Code No: NR-210303

Set No.

4

II B.Tech. I-Semester Supplementary Examinations, November-2003

ELECTRICAL ENGINEERING

(Common to Mechanical Engineering, Chemical Engineering, Mechatronics, Metallurgy and Material Technology, Aeronautical Engineering and production Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the coefficient of Mutual Inductance.

b)	Two identical coils X and Y of 1000 turns each lie is parallel planes such that 80%.  If flux produced by one coil links with the other.  If a current of 5A flowing in X produces a flux of 0.5m wb is it, find the mutual inductance between X and Y.

2.a)	Define peak factor, form factor.

b)	Calculate form factor for the sinusoidal wave form.

3.a)	 In a d.c machine the total iron loss is 8KW at its rated speed and excitation. If  the excitation remains the same, but speed is reduced by 25% the total iron loss  is found to 5KW. Calculate the hysteresis and eddy current losses at (i) Full  speed (ii)half the rated speed.

b) 	Describe in detail the various losses occurring in a d.c. generator.

4.a)	A dc motor takes an armature current of 110A at 480V.  The armature circuit resistance is 0.22 Ω .  The machine has 6poles and the armature is lap- connected with 864 conductors.  The flux/pole is 0.05 wb.  Calculate the gross torque developed by the motor.

b)	A 100H.P 500v shunt motor has 4 poles and a 2 circuit wave winding with 492 armature conductors, the flux is 50 mwb/pole and the full load efficiency 92%. The armature and commutating field winding have  a total resistance of 0.1ohm.  The shunt field resistance is 250 Ω.  Calculate for full load (i) the speed (ii) the useful torque.

5.a)	Distinguish between the efficiency and Regulation of a Transformer. Show how power affects both of them with necessary equations

b)	The efficiency of a 250 KVA, Single phase Transformer is 96% when delivering full-load at 0.8 power factor lagging and 97.2% when delivering half full load at unity power factor. Determine the efficiency at 75% of full load at 0.8 power factor lagging.

Contd.2

Code No:NR-210303				-2-				Set No.4

6..a)	Derive the Torque equation of an Induction motor.

b)	A 3 phase 50Hz, Induction motor is wound for 8 poles. If the full load slip is

2.5% , calculate

i) Synchronous speed               ii) Slip speed

iii) Rotor speed                        iv) Rotor frequency.

7.	The following test results are obtained on a 6600V alternator.

OC Voltage in volts       3100	4900	6400	7500	8300

Field current in Amps     16	  25	  37.5	   50	  70

A field current of 20 Amps is required to circulate full load current as short circuit of armature. Calculate full load   regulation at 0.8 PF lag by synchronous impedance method.

8.		A moving coil instrument which given full scale deflection with 15mA, has a copper coil having a resistance of 1.5 at 150C, and a temperature coefficient of 1/234.5 at 00C in series with a resistor of 3.5 having a negligible temp coefficient. Determine i) the resistance of shunt required for a full scale deflection of 20A and ii) the resistance required for a full scale deflection of 250v. If the instrument reads correctly at 150C, determine the percentage error in each case when the temperature is 250C.

&&&