Code No. 210451

OR

II B.Tech. I-Semester Supplementary Examinations, November-2003

# ELECTRONIC DEVICES

(Common to Electronics and Communication Engineering, Bio-Medical Engineering, Electronics and Instrumentation Engineering, Electronics and Control Engineering)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Derive the semi conductor diode current equation.  Draw its V-I characteristic and discuss its temperature dependence.

b)	Distinguish between zener and avalanche breakdown in diodes.

2.a)	What is meant by P-type semiconductor?  Explain how free charge carriers exist in P-type material.

b)	Determine the concentration of electrons with a current density of 100  A/m2 is flowing in the circuit.  Assume mobility of electrons is 1500 m2/V  sec and electric field is 104 V/m.  charge of the electron is 1.6 x 10-19 coulombs.

3.a)	Sketch the CE output characteristics of a transistor and indicate the active, cutoff and saturation regions.

b)	Prove that in the CB configuration the collector current in  active region is given by Ic = IB + ( + 1) ICO.

4.a)	Plot the drain characteristics in common source configuration and explain its operation.

b)	Explain the depletion mode and enhancement mode of operation of mospet.

5.a)	Explain Fermi level and derive an equation for intrinsic semiconductor.

b)	Explain donor and acceptor impurities.  How they can be added to intrinsic semiconductor?

6.a)	Consider instrinsic Ge at room temperature (300 K).  By what percent does the conductivity increase per degree rise in temperature?

b)	With the help of energy band diagram discuss the operation of tunnel diode.  Draw its V-I characteristics.

Contd….2.

## Code No:210451				:: 2 ::					OR

7.a)	Write expressions for the electrostatic deflection sensitivity of a CRT and explain.

b)	An electron is released with zero initial velocity from the lower of a pair of horizontal plates which are 3 cm apart.  The accelerating potential between these plates increases from zero, linearly with time at the rate of 10 V/sec.  When the electron is 2.8 cm from the bottom plate, a reverse voltage of 50 V replaces the linearly rising voltage.

(i) with which electrode does the electron collide?

(ii) What is the time of flight?

8.	Write short note on:

a) Life time of charge carriers.

b) PIN diode

c) UJT.

@@@@@