Code No:210854

OR

II B.Tech. I-Semester Supplementary Examinations, November-2003

MATERIAL AND ENERGY BALANCE

(Chemical Engineering)

Time: 3hours								Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Calculate the weight of iron and of steam required to produce 100 kg of hydrogen and the weight of the Fe3O4 formed. What volume will the hydrogen occupy at standard conditions?

3Fe+4H2O→Fe3O4+4H2

b)	An aqueous solution contains 40% Na2CO3 by weight. Express the composition in mole percent and in normality.

2.a) 	Calculate the number of cubic feet of hydrogen sulfide, measured at a temperature of 50C and a pressure of 740 mm Hg, which may be produced from 3.1752 kg of iron sulphide (FeS).

b) 	An automobile tire is inflated to a gage pressure of 2.38 atm at a temperature of 0F. Calculate the maximum temperature to which the tire may be heated without the gage pressure exceeding 3.4 atm.  (Assume that the volume of the tire does not change.)

3.a)	What are critical properties? Write about estimation of critical properties of organic substances and inorganic substances.

b)	 A mixture of ethyl acetate vapor and air has a relative saturation of 50% at 30C and a total pressure of 740 mm Hg. Calculate the analysis of vapor and percentage molar saturation, the vapor pressure of ethyl acetate at 30C is 119 mm Hg.

4. 	Nitrogen is saturated with benzene vapour at a temperature of 30°C and a pressure of

720 mm Hg. Benzene exerts a vapor pressure of 118.5 mm Hg at 30°C. Calculate the composition of the mixture expressed in the following terms:

(a)  Percentage by volume.

(b) Percentage by weight.

(c) kg of benzene per m3 of mixture.

(d) kg of benzene per kg of nitrogen.

(e) kg. mole of benzene per kg mole of nitrogen.

5.a)	 It is desired to make 1000kg of a mixed acid containing 60% H2SO4 , 32% HNO3 , and 8% water by blending (i) a spent acid containing 11.3% HNO3, 44.4% H2SO4 and 44.3% Water (ii) aqueous 90% HNO3  and (iii) aqueous 98%H2SO4 . All percentages are by weight. Calculate the quantities of each of the three acids required for blending

b) 	Explain the terms Bypass and Recycle

(Contd..2)

Code No: 210854				-2-				OR

6.a) 	SO2 is oxidized to SO3. If the conversion is 70% and the air used is 80% excess over the theoretical requirement, calculate the composition of the gases leaving the reactor (on volume basis)

b)	A gas analyzing CO2 5.5%, CO 25%, H2 14% CH4 0.5% and N2 55% by volume is burnt in a furnace with 10% excess air. Calculate the Orsat analysis of the flue gas.

7.a)	How does the heat of vaporization of a substance vary with its temperature and pressure?

b)	The latent heat of vaporization of ethyl alcohol is experimentally found to be 204 cal per gram at its normal boiling point of 78oC. Its critical temperature is 243oC. Estimate the heat of vaporization at a temperature of 180oC.

8.a)  	What is thermoneutrality of salt solutions? Explain with an example

b)	Discuss briefly about

(i) heats of formation of ions

(ii) heats of formation of atoms.

!!!!!