Code No: RR210303

Set No:

1

II B.Tech. I-Semester Regular Examinations November 2003

ELECTRICAL ENGINEERING

( Common to Mechanical Engineering, Chemical Engineering, Production Engineering, Mechatronics and Metallurgy and Material Technology  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Derive an expression for co-efficient of coupling.

b)	Two coils having the inductances of 0.12 milli heneries and 0.072 milli heneries 	and their mutual inductance is 1.15H.   Find coupling co-efficient.

2. a) 	Discuss the phenomena of Electrical resonance in a circuit where a coil is in 	parallel with a capacitor. Why it is called as current magnifying circuit.

b) 	A capacitor is connected in parallel with a coil having L=5.52mH and R=l0Ω., to 	a 100V, 50Hz supply. Calculate the value of the capacitance for which the 	current 	taken from the supply is in phase with the voltage.

3. a) 	Mention and explain the various causes for the failure of the generator to build up.

b)	 The terminal voltage of a generator falls from 250V on open circuit to 238V 	when delivering 60A. It is connected to a load in parallel with a battery having a 	constant emf of 245V and a resistance of 0.1ohm.  Find the current supplied by 	the generator when the total load current is 50A.

4. a)	Derive an expression for the torque developed in N-m and in Kf-m in a dc motor.

b)	Determine the value of torque in Kg-m developed by the armature of a 6-pole wave-wound motor having 492 conductors, 30mwb per pole when the total armature current is 40A.

5. a)	 Explain how do you conduct an open circuit test for a given transformer rating

2KVA, 400/200V experimentally.

b)	A 10KVA, 1- phase transformer has a turn ratio of 300/23. The primary is

connected to a 1500V, 60Hz supply. Find secondary volts on an open circuit

the maximum value of flux.

6. a)	Explain the principle of operation of a 3 phase induction motor.

b)	A 3 phase, 4 pole, delta connected induction motor has a full load slip of 5%.            If the supply frequency is 50Hz., find the full load speed, synchronous speed

and rotor frequency.

(Contd…2)

Code No: RR210303				.. 2..				Set No: 1

7. a)	Derive an expression for the voltage induced in an alternator?

b) 	Find the number of armature conductors in series per phase required for the 	armature of a 3-phase, 50 Hz, 10-pole alternator with 90 slots. The winding is 	star-connected to give a line voltage of 11 KV. The flux per pole is about 0.16wb.

8. a)	Give a summary of four different types of voltmeters commonly used in practice. State whether they can be used for AC or DC circuits. In one case, give a sketch showing the construction with the method of control and damping employed.

b)	A DC voltmeter has a resistance of 28600. When connected in series with an external resistor across a 480V DC supply, the instrument needs 220V.  What is the value of the external resistance?

~ ~ ~ ~

Code No: RR210303

Set No:

2

II B.Tech. I-Semester Regular Examinations November 2003

ELECTRICAL ENGINEERING

( Common to Mechanical Engineering, Chemical Engineering, Production Engineering, Mechatronics and Metallurgy and Material Technology  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.	Two coils having self inductances are 0.9H and 0.4H.their mutual Inductance is 0.3H.  Find the Equivalent Inductance when both are magnetized in (a) same direction (b) opposite direction.  Derive the formula used.

2. a) 	What is meant by dynamic impedance.

b) 	An inductive circuit of resistance 2Ω and inductance 0.01H is connected to a 	250V, 50Hz supply. What capacitance placed in parallel will produce resonance? 	Find the total current taken from the supply and the current in the branch circuit.

3. a) 	Give two applications for each of shunt, series and compound generators.

b) 	The shunt generator delivers full load current of 200A at 240V. The shunt field resistance is 60ohm and full load efficiency is 90%. The stray losses are 800W. Find (i) armature resistance (ii) current at which the maximum efficiency occurs.

4. a)	Explain the classification of dc motors with neat diagrams and corresponding voltage equations for each.

b)	The armature of a dc machine has a resistance of 0.1 Ω and is connected to a 230V supply.  Calculate the back emf when it is running as a motor taking 80A.

5. a)	 Explain with a neat circuit diagram to conduct short circuit test on a given transformer 1KVA ,100/400 volts. Give the ratings of the meters used.

b)	A 3300/230V, 50Hz , 1-phase transformer is to be worked at a maximum flux       	density of 1.2wb/m2 in the core. The effective cross-sectional area of the core is

150cm2 . Calculate the suitable values of primary and secondary turns.

6. a)	Explain the rotating magnetic field developed in an Induction Motor.

b)	A12 pole 3- phase alternator coupled to an engine running at 500rpm. It        supplies an induction motor which has a full load speed of 1440rpm. Find         	  the % slip and the number of poles of the motor.

(Contd…2)

Code No: RR210303.				.. 2..				Set No: 2

7. a)	What is distribution factor? What is its effects? Derive an expression for distribution factor of an alternator?

b) 	In a 60 KV A, 200V, I-phase alternator, the effective armature resistance, and leakage reactance are 0.016 ohm, and 0.07 ohm respectively. Calculate the emf induced in the armature, when the alternator is delivering rated current at a p.f. of i) unity; ii) 0.7 lagging.

8.	If a rectifier type voltmeter has been calibrated to read the rms value of a sinusoidal voltage, by what factor must the scale readings be multiplied when it is used to measure the rms value of i) a square wave voltage ii) a voltage having a form factor of 1.15.

~ ~ ~ ~

Code No: RR210303.

Set No:

3

II B.Tech. I-Semester Regular Examinations November 2003

ELECTRICAL ENGINEERING

( Common to Mechanical Engineering, Chemical Engineering, Production Engineering, Mechatronics and Metallurgy and Material Technology  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Derive an expression for the equivalent Inductance when two coils one connected in parallel.

b)	Two coils of inductances 4 and 6 henries are connected in parallel.  If their mutual inductance is 3 henries, calculate the equivalent Inductance of the combination if  (i) Mutual inductance assists the self-Inductance (ii) Mutual inductance opposes the self Inductance.

2. a) 	Coil is in parallel with R.C series combination. Derive resonant frequency 	expression for this parallel circuit.

b) 	Compute the value of C which results in resonance for the circuit shown when 	f=2500/π Hz.

3. a)	 In a d.c machine the total iron loss is 8KW at its rated speed and excitation. If  the excitation remains the same, but speed is reduced by 25% the total iron loss  is found to be 5 KW. Calculate the hysteresis and eddy current losses at

i) Full speed.	ii) Half the rated speed.

(b) 	Describe in detail the various losses occurring in a d.c. generator.

4. a)	Why is the speed of a shunt motor practically constant?

b)	Draw the characteristics of shunt and series motors.

5. a)	 What useful information will be obtained from open circuit and short circuit tests of a transformer and how they will be used in calculating efficiency and regulation?

b)	 The No load voltage ratio in a single phase 50Hz , core type transformer is 1200/440 . Find the number of turns in each winding if the maximum flux is  0.075wb.

6. a)	How do you classify the induction motors. Give the constructional details of them.

b)	Define slip and give an account of the quantities which vary with slip.

(Contd…2)

Code No: RR210303.				.. 2..				Set No: 3

7. a) 	What are the three voltage drops occurring in an alternator on-load?

b) 	A 550 V, 55 KV A, single phase alternator has an effective resistance of 0.2 ohm. 	A field current of lOA produces an armature current of 200 A on short circuit, and 	an emf of 450 V on open circuit. Calculate i) The synchronous reactance; ii) The 	full- load regulation with power factor 0.8 lagging.

8.a)	Explain the principle of operation of one type of moving iron instrument showing how it is suitable for use on DC and AC systems.

b)	The total resistance of moving iron voltmeter is 1000 and the coil has an 	inductance of 0.765 H. the instrument is calibrated with a full scale deflection 	50V DC. Calculate the percentage error when the instrument is used on

i) 25 Hz supply ii) 250 Hz supply, the applied p.d being 50V in each case.

~ ~ ~ ~

Code No: RR210303.

Set No:

4

II B.Tech. I-Semester Regular Examinations November 2003

ELECTRICAL ENGINEERING

( Common to Mechanical Engineering, Chemical Engineering, Production Engineering, Mechatronics and Metallurgy and Material Technology  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Explain about “Dot convention” in the case of mutually coupled circuits.

b)	Explain briefly “How you can use mutual inductance principle to the transformer.”

2. a) 	How the power factor is improved?

b) 	In a series-parallel circuit the parallel branches A and B are in series with C. The 	impedances are ZA=(4+j3); ZB=(4-j16/3); Zc=(2+j8)ohms. If the current

Ic=(25+j0), draw the complete phasor diagram determining the branch currents 	and voltages and the total voltage. Hence, calculate the complex power for each 	branch and the whole circuit.

3. a)	Draw the characteristics of a compound generator.

b) 	In a long shunt compound generator, the terminal voltage is 230V when the generator delivers 150A. Determine

(i)  	Induced emf.		(ii) 	Total power generated

(iii)	Distribution of this power. Given that shunt field, series field, divertor 	and armature resistances are 92, 0.015, 0.03 and 0.032 ohms respectively

4. a)	With a neat sketch, explain the operation of three point starter.

b)	If the load is removed from a dc series motor in operation, what will happen?

5. a) 	Define the % Regulation of a transformer and explain how do you calculate its 	value in laboratory .

b)	A transformer when tested on full load is found to have copper loss of 1.7%        	and reactance drop of 3.8%. Calculate full load regulation at power factors of

(i) 0.8 lagging      (ii) unity power factor      (iii)  0.707  leading

6.a)	Derive the Torque equation of an Induction motor.

b)	A 3 phase 50Hz, Induction motor is wound for 8 poles. If the full load slip is

2.5% , calculate

(i) Synchronous speed              (ii) Slip speed

(iii) Rotor speed                        (iv) Rotor frequency.

(Contd…2)

Code No: RR210303.				.. 2..				Set No: 4

7. a) 	Discuss the construction, and principle of operation of a 3-phase synchronous generator.

b) 	A 3-phase, star-connected, round-motor synchronous generator, rated at 10 KV A, and 230 V has synchronous reactance of 1.2 ohms per phase, and an armature resistance of 0.5 ohms per phase. Calculate the percentage voltage regulation at full load with (i) 0.8 lagging p.f. (ii) 0.8 leading p.f

8. a)	Describe with the diagram the construction of a repulsion type moving iron instrument with particular reference to the means used for (i) deflection                    (ii) control (iii) damping.

b)	A moving iron voltmeter in which full scale deflection is given by 100V, has a coil of 10000 turns and a resistance 2000. Calculate the number of turns required on the coil of the instrument is converted for use as an ammeter reading 20A full scale deflection.

~ ~ ~ ~