Code No. 210352

OR

II B.Tech.  I-Semester Supplementary Examinations, November-2003

THERMODYNAMICS

(Common to Mechanical Engineering, Production Engineering, Mechatronics)

Time: 3 hours							         Max. Marks:70

Answer any FIVE questions

All question carry equal marks

- - -

1.a)	Explain the following terms clearly.

(i) Thermodynamic equilibrium.

(ii) Thermodynamic properties.

b)	State and explain first law of thermodynamics with examples.  Indicate its limitations.

2.a)	Write a note on scales of temperature.

b)	Air flows steadily at the rate of 22.68 kg/min through on air compressor entering at 6.1 m/s velocity, 1.02 bar pressure and 0.845 m3/kg and leaving at 6.4 m/s, 7.03 bar pressure and 0.163 m3/kg.  The internal energy of the fluid leaving is 88.2 kJ/kg greater than that of the air entering.  Cooling water in the compressor jackets absorbs heat from the air at the rate of 3693 kJ/min.  Compute the shaft work input to the air in kW.

3.a)	Write a note on the following:

(i) Availability and (ii) Irreversibility.

b)	2 kg of air at a pressure of 7 bar occupies a volume of 0.28 m3;  this air is then expanded to a volume of 1.4 m3 according to the law PV1.3= constant.  Find the final temperature, the work done, the heat added or rejected by this air during expansion.

4.a)	State Kelvin-plank and Clausius statements of second law of thermodynamics and show that they yield the same result.

b)	A quantity of gas weighing 0.5 kg expands from a pressure of 8.8 bar absolute to a pressure of 1.1 bar absolute, in such a way that the gas does not receive or reject any heat.  The initial temperature is 200oC.  Find the work done and change in internal energy if Cp=1.008, Cv=0.714.

5.a)	Explain throttling and free expansion processes clearly.

b)	Steam at a pressure of 10 bar and 0.9 dry expands to 1 bar pressure hyperbolically.  Find (i) work done and (ii) change in enthalpy.  Specific heat of steam at constant pressure = 2.0 kJ/kgoC.

6.a)	Write a note on psychrometric chart.

b)	Derive an expression for air standard efficiency of diesel engine in standard form.

7.a)	Explain the working of vapour compression cycle.  Indicate its application.

b)	State and explain Dalton’s law of partial pressures.

8.	Write short notes on the following:

(i) Rankine cycle  (ii) Maxwells relations  (iii) Entropy.

- - -