Code No:RR210104

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

SURVEYING – I

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What are the principles of surveying ?

b)	The plan of an area has shrunk such that a line originally 100m now measures 95mm. If the original scale of the plan was 1cm = 50m; Determine (i) the shrinkage factor (ii) shrunk scale (iii) correct distance corresponding to a measured distance of 980 m (iv) correct area corresponding to a measured area of 10,000 m2 .

2.a)	The magnetic bearing of a line was found to be N 600 30' W in 1956 when the declination on was 50 10' E. Find  its present magnetic bearing if declination is   30 W at present.

b)	What is local attraction? How is it detected and how the observed bearings are corrected for the same?

3.a)	What do you understand by the term plane table survey? When is it recommended

b)	Describe the various components of a plane table used for survey purposes. What are their functions ?

4.a)	An observer at a height of 36m above mean sea level just sees a luminous object on the top of a hill, situated at a distance of 81km  from him. What is the height of the hill above  sea level ?

b)	Starting from a point A of elevation 216.470, levels were taken for a section which extended to a point P. whose reduced level was found to be 218.00. Check levels were carried back along the shortest route from P to A, with the following readings.

| BS | FS | Remarks |
| --- | --- | --- |
| 1.155 |  | P |
| 0.750 | 3.50 |  |
| 1.770 | 2.750 |  |
| 2.450 | 0.370 |  |
| 0.720 | 1.250 |  |
| 1.770 | 3.300 |  |
| 0.710 | 1.865 |  |
|  | 2.250 | A |

Find the error of closure on the starting point.

Contd……..2

Code No:RR210104			::2::					Set.No.1

5.a)	Define contour, contour interval, horizontal equivalent.

b)	Suggest some suitable values of contour interval for different scales of map and type of ground.

c)	Explain the uses of contours.

6.a)	State and explain Simpson’s rule.  Derive an expression for it.

b)	A series of perpendicular offsets were taken from a survey line to a curved boundary.  Determine the area using both Tragezoidal and Simpson’s Rules for the data given below:

| Distance(m) | 0 | 15 | 30 | 45 | 60 | 75 | 90 | 105 | 120 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offset (m) | 3.25 | 505 | 4.2 | 6.65 | 8.70 | 6.30 | 3.20 | 4.50 | 5.65 |

The offsets are taken at internals of 15m.

7.a)	What are the different methods that are generally adopted for measuring   volume. Explain them briefly.

b)     Calculate the volume of earth work by prismoidel formula in a road embankment with the following date.

| Chainage along the centre line | 0 | 100 | 200 | 300 | 400 |
| --- | --- | --- | --- | --- | --- |
| Ground Levels in formation | 201.7 | 202.9 | 202.4 | 204.7 | 206.9 |

Formation level at chainage 0 is 202.3, top width 2.0  ft side slopes are 2 to 1. The longitudinal slope of the embankment is 1 in 100 rising. The ground is assumed to be level across the longitudinal section.

8.a)	Explain with a neat diagram, the working of the Line Ranger.

b)	Describe Amsler polar planimeter with a neat sketch.  Also explain how it is used to measure the area of plan of any shape accurately.

$$$

Code No:RR210104

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

SURVEYING – I

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What are the instruments to be used in chain surveying? Explain in brief

b)	A 20m chain was tested before the commencement of the day’s work and found to be correct. After chaining 940m, the chain was found to be 0.010 m long. At the end of day’s work, after chaining a total distance of 1476m, the chain was found to be 0.15m long. What was  the true distance chained.

2.a)	The true bearing of a tower observed from a station A is 3500 30' and the magnetic bearing of the tower is 20 30'. The back bearing of the line AB when measured with a prismatic compass was found to be 3300 30'. What is the true bearing of the line AB?

b)	Explain the Bow ditch rule for adjusting a compass traverse, with neat sketches.

3.a)	State the three point problem in plane tabling and describe how it is  solved by Bessel’s  method.

b)	Explain with sketches the different methods of locating a point by plane table. Discuss  the relative merits of these methods.

4.a)	Two pegs A and B are fixed 100 m apart. A level is set up near A. Observations on a staff held at A and B gave the following readings A = 1.650 B = 1.665. The level is then  placed near B and observations on a staff held at A and B gave the following; A = 1.590, B = 1.575. State whether the instrument is in adjustment or not. Also determine the correct difference in level between A and B.

b)	A dumpy level was set up exactly midway between two pegs. A and B, 80 m apart. The readings on the staff when held on pegs A and B were 1.865 and 1.780 respectively. The   instrument was then moved and set up at a point C on the line BA produced and 16m from A and the respective staff readings on A and B were 1.620 and 1.550. Calculate the staff B readings on A and B to give a horizontal line of sight.

5.a)	Discuss the different factors based on which the choice of proper contour interval depends.

b)	Explain the different characteristics of contours along with neat sketches.

Contd…….2

Code No:RR210104			::2::					Set.No.2

6. a)	Explain different ways of determining areas by computations based on measurement scaled from a map.

b)	Determine the area in hectares between the line AB and a meandering stream for offsets taken at a regular interval of 20m along the line AB.  Use both Trapezoidal rule and Simpson ‘s rule.

| Point | A |  |  |  |  |  |  |  | B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distance (m) | 0 | 20 | 40 | 60 | 80 | 100 | 120 | 140 | 160 |
| Offset lengths(m) | 23 | 40 | 42 | 30 | 32 | 60 | 10 | 14 | 22 |

7.a)	Derive an expression for the trapezoidal formula for volumes  .

b)	A road embankment is 6.0 m wide at the formation level. The average height of the embankment is 2.4 m at an average gradient of 1 in 50 from the 85 m contour to the 229 m contour. If the side slopes are 2:1, find the length of the road and the volume of the embankment.

8.a)	What is meant by zero circle?  Describe various methods of determining its area.

b)	What is a Box Sextant.  How it is used for the measurement of Horizontal angle.

$$$

Code No:RR210104

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

SURVEYING – I

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)		What are the sources of errors in chaining ? What precautions would you take to guard against them.

b)	A line was measured with a steel tape, which was exactly 30m long at 190C and found to be 1451.43m. The temperature during measurement was 310C find the true length of the line. Take the Co-efficient of expansion of the tape                  per oC=11.7x10-6

2.	The magnetic and true bearings of a line AB were observed as 1560 12' and      1540 05' respectively. Find the values of the magnetic declination if the annual change in the magnetic declination is 2' W, what would be the magnetic and true bearings of the line AB after 3 years since the date of the above observations.

3.a)	Discuss the chief sources of error in plane table surveying and how are they          	eliminated?

b)	Write short notes describing the ‘telescopic alidade’.

4.a)	Define the terms Axis of Telescope, Height of Instrument, GTS Benchmark and datum surface

b)	In carrying out the permanent adjustment of a dumpy level by two peg method, the following observations were made

Instrument at peg E midway between points C and D, 100m apart.

Reading at point C	= 2.000m

Reading at point D	= 3.000m

Instrument at peg F in line of CD such that CF=120m and DF=20m

Reading at point C	= 1.500m

Reading at point D =  2.750m

Check whether instrument needs permanent adjustment or not, and whether the line of sight is inclined upward or downward. What should be the correct reading at C if the instrument is to be adjusted

5.a)	What are the different methods of locating contours under what circumstances these methods are adopted to. Explain direct method of locating contours.

b)	What do you mean by contour gradient? Explain the method of locating contour gradient in detail.

Contd…….2

Code No:RR210104			::2::					Set.No.3

6.a)	The following perpendicular offsets were taken from a chain line to a hedge.

| Chain age(m) | 0 | 15 | 30 | 45 | 60 | 70 | 80 | 100 | 120 | 140 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offsets (m) | 7.60 | 8.5 | 10.7 | 12.8 | 10.6 | 9.5 | 8.3 | 7.9 | 6.4 | 4.4 |

Calculate the area between the survey line, the hedge and the end offsets by Trapezoidal Rule and Simpson’s Rule.

b)	The data of a closed traverse survey is shown below.  Determine the area.

| Line | Latitude (m) | Departure (m) |
| --- | --- | --- |
| AB | -300 | +450 |
| BC | +640 | +110 |
| CD | +100 | -380 |
| DA | -440 | -180 |

7.a)	In a certain railway cutting the width at formation level is 9m. The side of the cutting slope at 1 to 1 and the original ground surface has a side slope of 1 in 8. Determine the volume and cost of the cutting contained in a length of 240m, the average depth of cutting being 1.8m and its cost is Rs 50 per 100 cubic metres.

b)	calculate the quantity of earth work in cubic metres required for a road embankment from the following data. Formation width = 9m; side slope 2 to 1.

Distance(m)	       Height of bank(m)	Side slope of original ground

0			3.0				1 in 10

30		3.6				1 in 8

60		2.7				1 in 12

8.a)	Explain the different parts of a Box Sextant along with a neat sketch. Also explain how it is used for the measurement of vertical angle.

b)	Explain with a neat diagram the construction and working of a prism square.

$$$

Code No:RR210104

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

SURVEYING – I

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)		What is reciprocal  ranging ? Explain in detail.

b)	A 30m tape actually measure 29.985m when it is supported at the two ends only, with the temperature at 300C and a tension of 70N.  The tape weighs 12N and has a cross sectional area of 0.04cm2. If the field temperature is also 300C, what tension should be applied so that the tape measures exactly 30m when supported at the two ends.

2.	A compass was set on the station A and the bearing of AB was  309 0  15  Then the same instrument was shifted to station B and the bearing of BA was found to be 129 0   15. Is there any local attraction at the station A, or at the station B? Can you give a precise answer? State your comment and support it with rational arguments

3.	What is the three-point problem in plane table surveying? How does one solve it? When   does the solution become impossible ?

4.a)	Compare and contrast Rise and Fall Method and Height of Instrument Methods of reduction of levels.

b)	On a level, the angular value of one division of the bubble is 30 seconds and the graduations are 2mm long. Find.

i)  The Radius of Curvature of the tube and

ii) Reading on a staff held 100m away for a disturbance of the bubble 3 divisions from centre towards the observer, the reading with bubble in the centre being 2.540m.

5.a)	Explain tacheometric method of locating contours with a neat sketch. Under what circumstances this method is preferred to.

b)	Explain in detail how the contour plan is used to calculate the capacity of a reservoir.

Contd…….2

Code No:RR210104			::2::					Set.No.4

6.a)	The following offsets were taken from a chain line to a hedge.

| Distance (m) | 0 | 8 | 16 | 24 | 32 | 48 | 64 | 88 | 112 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offset (m) | 3.76 | 4.32 | 5.44 | 4.88 | 3.84 | 3.36 | 3.00 | 2.52 | 1.84 |

Calculate the area in square metres included between the chain line, the hedge and the end offsets by Trapezoidal Rule and Simpson’s Rule.

b)	The following table gives the corrected latitudes and departures (in metres) of the sides of a closed traverse PQRS.  Compute its area by co-ordinates method.

| Side | Latitude |  | Departure |  |
| --- | --- | --- | --- | --- |
|  | N | S | E | W |
| PQ | 128 |  | 9 |  |
| QR | 15 |  | 258 |  |
| RS |  | 143 | 9 |  |
| SP | 0 |  |  | 276 |

7.a)	A railway embankment is 10m wide with side slopes 1 ½ to 1. Assuming the ground to be level in a direction transverse to the centre line, calculate the volume contained in a length of 120m, the centre heights at 20m intervals being in metres 2.2, 3.7, 3.8, 4.0, 3.8, 2.8 and 2.5.

b)	A road embank is 8m wide and 200m in length at the formation level with a side slope of 1.5:1. The embankment has a rising gradient of 1 in 100m. The ground levels at every 50m along the centre line are as follows.

Distance(m)	0		50		100		150		200

R.L.(m)         164.5	          165.2                166.8                167	           167.2

The formation level of zero chainage is 166m. Calculate the volume of earth work.

8.a)	Derive the condition based on which the prism square works.  Also explain how it is tested and adjusted.

b)	In case of Sextant prove that when a ray of light is reflected successively from two mirrors, the angle between the first and last directions of ray is twice the angle between the planes of the two mirrors.  Also list out the optical requirements of the Sextant.

$$$