Code No:RR/NR210205

Set No.

1

II B.Tech. I-Semester Regular Examinations November, 2003

ELECTRO MECHANICS - I

(Electrical and Electronics Engineering)

Time:3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Derive the force in a singly excited relay in the non-linear case.

- In a rectangular electromagnetic relay excited from a voltage source, the current and flux linkages are related as i = λ ( λ  +  2 (1 – x )2 ); x < 1. Find force on the armature as a function of λ.

2.	A 4-pole wave wound armature has 230 conductors and 23 commutator segments.

Give the table of winding connections. Choose retrogressive winding.

3.a)	Explain the effects of armature reaction in a d. c. generator and discuss briefly   the methods to minimize these effects.

b)	With a neat sketch explain the function of commutator in a d.c. machine.

4.a)	What is critical field resistance? How do you calculate the critical field resistance practically.

b)	When a shunt generator fails to build up voltage, then how to rectify the fault.

5.a)	Explain the principle of operation of a DC motor. Derive the equation for the torque developed by a DC motor.

b)	Determine the torque developed when a current of 30A passes through the armature of a motor with lap winding of 310 conductors, 4-Pole, pole shoes 16.2cm long subtending an angle of 600 at the center, bore radius 16.2cm, flux density in air gap 0.7 tesla.

6.a)	A 220V shunt motor takes 60A when running at 800rpm. It has an armature resistance of 0.1. Find the speed and armature current if the magnetic flux is weakened by 20%, contact drop per brush = 1V. Total torque developed remains constant.

b)	A 220V series motor runs at 800 rpm, when taking a current of 15A. The motor has       Ra = 0.3 and Rf = 0.2. Find the resistance to be connected in series with armature if it has to take the same current at the same voltage at 600 rpm. Assume flux is proportional to current.

7.a)	What is the function of no volt release and overload release in a starter? Discuss the operation of these two features in a shunt motor starter.

b)	Explain with neat diagrams construction and working of a series motor starter.

8.a)	How are large series machines tested, explain?

b)	The Hopkinson’s test on two DC shunt machines gave the following results for full load. Line voltage 250V, line current 45A excluding field currents; motor armature current 385A; field current 5A and 4A. Calculate the efficiency of each machine. Armature resistance of each machine is 0.015 .

&&&

Code No:RR/NR210205

Set No.

2

II B.Tech. I-Semester Regular Examinations November, 2003

ELECTRO MECHANICS - I

(Electrical and Electronics Engineering)

Time: 3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Show how mechanical energy output can be determined in the singly excited relay in the non-linear case.

- A doubly excited magnetic field system has coil self and mutual inductances of L11 = L22 = 2, L12 = L21  = cos2θ, where θ is the angle between the axes of the coils. If coil 2 is shorted while coil 1 carries a current of Im = √2 sin 314t, derive expressions for instantaneous and time average torques. If the rotor is allowed to move, at what angle will it come to rest? Plot the average torque variation with respect to θ and explain your result.

2.	Draw the developed armature-winding diagram of a d. c. machine showing commutator segments and position of brushes. The particulars of the windings are given below: Type = simplex retrogressive wave winding; no. of slots = 21; no. of poles = 4;  no. of conductors per slot = 2; no. of commutation segments = 21;  coil span  = 5 slot pitches.

3.a) 	Explain the purpose of laminating the armature core of a d. c. machine.

b)	The diameter of a commutator ring of a lap wound d. c. motor is 25 cms. The brush width is 1.8 cm and width of mica insulation is 0.2 cm.  If the speed of the motor is 200 rpm, determine the time of commutation.

4.a)	What is critical speed? How do you calculate the critical speed in the laboratory?

b)	What are the conditions to build up of emf in a shunt generator?

5.a)	How does a DC motor automatically adjust input to match the mechanical load on the motor?

b)	Explain the armature reaction in DC shunt motors, indicating also a few remedies to its adverse effects.

6.a)	What are the different methods of speed control of DC motor? Explain.

b)	A 220V DC shunt motor draws a no load armature current of 2.5A when running at 1400rpm. Determine its speed when taking an armature current of 60A if armature reaction weakens the flux by 3%.

7.a)	Explain Swinburne’s test of finding the performance of a DC machine working as a motor. What are the advantages and disadvantages?

b)	Write a note on Brake Test on D.C. Motor.

Contd..2

Code No:RR/NR210205			-2-				Set No.2

8.a)	What are the advantages of the conventional efficiency determination over direct loading? And why direct loading is relatively impossible for large machines?

b)	Two 10 HP motors have each a full load efficiency of 0.83 the running light losses of motor A are 530 W, and of motor B are 420 W. Find the half load efficiency of each motor.

&&&

Code No:RR/NR210205

Set No.

3

II B.Tech. I-Semester Regular Examinations November, 2003

ELECTRO MECHANICS - I

(Electrical and Electronics Engineering)

Time:3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Show how mechanical energy output can be determined in the doubly excited system in the non-linear case.

b)	A doubly excited magnetic field system has coil self and mutual inductances of L11 = L22 = 2, L12 = L21  = cos2θ, where θ is the angle between the axes of the coils. If coil 2 is shorted while coil 1 carries a current of i = Im sin ωt, derive expressions for instantaneous and time average torques. Find time average torque when θ = π/4 and Im = √2 sin 314t.

2.		Determine a suitable winding scheme for a 2-circuit, 4-pole winding of  a  d.c

machine. The armature slots are 17 and the commutator bars are 51. Give the       winding table and use retrogressive type winding.

3.a)	When are dummy coils used and which type of D.C. armature winding these will occur?

- Calculate the ampere turns for each commutating pole of an 8-pole generator with 107 slots, each containing 1000 ampere conductors. The interpole air-gap is

1.2 cm. The flux density in the air gap is to be 0.32 T. Neglect iron parts and leakage.

4.a)	How do you classify the compound generators.

b)	What are the various characteristics of compound generators?

5.	A 4-pole 250V DC shunt motor has lap connected 960 conductors. The flux per pole is 20mWb. Determine the torque developed by the armature and the useful torque in Nm when current drawn by the motor is 32A. The armature resistance is 0.1 and shunt field resistance is 125. The rotational losses of the machine amount to 825W. Derive the formula used.

6..a)	Derive the standard torque equation of a D.C. motor from first principles.

b)	A 200V DC shunt motor with a constant main field drives a load the torque of which varies as the square of the speed. When running at 600 rpm it takes 30A.  Find the speed at which it will run and the current it will draw if a 20 resistor is connected in series with armature. Neglect motor losses.

Cont..2

Code No:RR/NR210205				-2-			Set No.3

7.a)	Why is a starter necessary for a dc motor? What is the difference between 3 point and 4 point starters?

b)	Calculate the values of resistance elements for a 6-stud starter. The maximum current at starting is not to exceed 20A for a 200V shunt motor. The armature resistance is 0.5. What is the value of minimum current?

8.a)	What are the various methods of finding inertia of a motor? Explain any one method to calculate inertia experimentally?

b)	A retardation test is carried out on a 1000 rpm DC machine. The time taken for the speed to fall from 1030 rpm to 970 rpm is i) 36 seconds with no excitation.   ii) 15 seconds with full excitation and iii) 9 seconds with full excitation and armature supplying an extra load of 10A at 219 V.

&&&

Code No:RR/NR210205

Set No.

4

II B.Tech. I-Semester Regular Examinations November, 2003

ELECTRO MECHANICS - I

(Electrical and Electronics Engineering)

Time:3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.	Describe the principle of energy conversion. From the consideration of various energies included, develop the model of an electromechanical energy conversion device.

2.	Give lay out (winding table) of a simplex lap progressive winding used for a 44 slot, 4-pole d. c. armature with 44 commutator segments.

3.	A 4-pole d. c. shunt generator supply a current of 143 A. It has 492 conductors on   the armature lap connected while delivering full load, the brushes are given in actual lead of 100.  Calculate the magnetizing ampere turns per pole. The field winding is shunt connected and takes 10 A. Find the number of extra shunt field turns necessary to neutralize demagnetization.

4.	What is parallel operation? How do you connect the two shunt generators in parallel?

5.a)	In every dc generator, motor action occurs and in every dc motor a generator action occurs. Explain.

- A 250V dc shunt motor takes 41A at full load. Find the back emf on full load      if   Ra = 0.1 and Rf = 250.

6.a)	What are the various methods of speed control of dc series motors?

A 480V, 20KW shunt motor took 2.5A when running light. Taking  Ra = 0.6 and Rf  = 800 and brush drop of 2V, find the full load efficiency

7.a)	What is the effect of excitation, speed and load on the losses of a DC machine?

b)	A 230V, DC shunt motor is taking 5A when running light (i.e at no load). The armature resistance (including brushes) is 0.2 and field circuit resistance is 115. For an input current of 72A, calculate the shaft output and efficiency. Also calculate the armature current at which the efficiency is maximum.

8.a)	How can you conduct the retardation test on D.C. Shunt Motors.

b)	A fields test on two mechanically coupled DC series motors (with their field   windings connected in series)gave the following data; motor : armature current      - 50A, armature voltage – 500V, drop across field winding – 38V, generator : armature current  - 38A, armature voltage – 400V, drop across field winding        – 36V, resistance of each armature is 0.2  , calculate the efficiency of each machine at this load.

&&&