Code No:NR211802

Set No.

1

II B.Tech. I-Semester Supplementary Examinations, November-2003.

FURNACE TECHNOLOGY AND PYROMETRY

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define the steady state heat transfer.

b)	Explain the role of heat transfer by convection and radiation in metallurgical furnaces.

c)	Derive an equation for conduction through homogeneous plane wall.

2.a)	Derive the following equation for conduction under un-steady state

∂t∂τ=α∂2t∂x2

b)	A furnace wall consists of fire brick (K=12x10-1 w/m/oC), insulating brick (K=1x10-1 w/m/oC) and red brick (K=18x10-1 w/m/oC) with thickness of 0.1 m of each.  The inside and outside temperatures are 1200 and 200oC.  Find heat loss through this composite wall and temperature of interfaces.

3.a)	Derive Newton-Rikman law for a fluid in contact with a solid boundary.

b)	Describe types of heat exchangers based on temperature distribution.

c)	A quantity of 20x103 kg/hr of water is to be heated in a counter flow heat exchanger from 30oC to 90oC.  Hot gases used for heating water enter the exchanger at 300oC and leave 200oC.  What is the overall heat transfer co-efficient, if the total surface area is 3000 sq.m.

4.a)	What is a black body?  Explain Planck’s law of monochromatic radiation of a black body.

b)	Two parallel infinitely large plates are temperatures of 1500 and 600oC with emissivities of 0.8 and 0.5.  Calculate radiant interchange.

c)	Explain luminous flame.

5.a)	Describe characteristic features of open hearth furnace with a neat diagram.

b)	Explain electric resistance tube furnace.

Contd….2

Code No:NR211802					-2-			Set No:1

6.a)	What is Seebeck effect?

b)	What are the required properties of thermo-couples?

c)	Explain use of potentiometer for measuring emf of a thermo-couple.

d)	A milli-voltmeter with a  resistance of 15 ohms is calibrated to read correctly when couple and lead resistance equal 3 ohms.  If it increases to 4 ohms.  Determine % error in readings of milli-voltmeter.

7.a)	What is Wein’s law of monochromatic radiation?

b)	Explain principle, construction and use of F&F optical pyrometer (Wedge type).

c)	What is the effect of reflection on optical pyrometer readings.

8.a)	Explain platinum temperatures.

b)	Describe principle and use of Fery metal spiral radiation pyrometer and mention the advantages of radiation pyrometers.

c)	An optical pyrometer sighted upon inside of a hallow metal wedge read as 1500oC.  When focused upon outside, the apparent temperature was read as 1350oC.  What is the emissivity of the metal.

- - -

Code No:NR211802

Set No.

2

II B.Tech. I-Semester Supplementary Examinations, November-2003.

FURNACE TECHNOLOGY AND PYROMETRY

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define steady state heat transfer.

b)	Compare natural convection with forced-convection.

c)	Derive an equation for conduction through composite cylindrical wall.

2.a)	Derive the following equation under un-steady state conduction.

∂t∂τ=α∂2t∂x2

b)	An annealing oven is constructed of brick (K=1.6x10-1 w/m/oC).  The inside dimensions are 3x2x1m.  Wall thickness is 0.1m.  The inner and outer surface temperatures are 800 and 100oC.  Determine heat loss per day.

3.a)	Why is convective heat transfer complicated?

b)	Describe types of heat exchangers based on temperature distribution.

c)	Two fluids in a heat exchanger are separated by a plane steel wall of 0.01m thick (K=400x10-1 w/m/oC).  The film co-efficients are 6 and 3 kw/m2/oC.  Determine rate of heat exchange for a mean temperature difference of 120oC.

4.a)	Define emissivity and explain economic thickness of insulation.

b)	Calculate net radiant interchange between very large parallel planes at temperatures 900 and 600oC.  The emissivities of hot and cold planes are              0.9 and 0.7.

c)	Explain heat transfer through air spaces.

5.a)	Describe characteristic features of Blast furnace and explain its developments.

b)	Explain heat balance of a furnace.

6.a)	What is the principle involved in thermo-electric pyrometer?

b)	Explain thermo-pile and its use.

c)	Explain preparation and calibration of thermo-couples.

d)	A milli-voltmeter with a resistance of 25 ohms is calibrated to read correctly when couple and lead resistance equal 3 ohms.  If it increases to 5 ohms.  Determine % error in readings of milli-voltmeter.

Contd…..2

Code No:NR211802				-2-				Set No:2

7.a)	What is the principle involved in optical pyrometers?

b)	Explain principle, construction and use of pyro-optical pyrometer.

c)	What is the effect of absorption on optical pyrometer readings?

8.a)	What is callendar’s correction?

b)	Explain the principle and use of pyro-pyrometer and mention the draw backs of radiation pyrometers.

c)	A dis-appearing optical pyrometer requires currents of 349 and 456 mA for brightness match at 800 and 1200oC.  What current should be required at 1000 and 1300oC assuming scale law of

1T=AI23+B

- - -

Code No:NR211802

Set No.

3

II B.Tech. I-Semester Supplementary Examinations, November-2003.

FURNACE TECHNOLOGY AND PYROMETRY

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What is the importance of heat transfer to Metallurgical tings?

b)	Compare heat transfer by radiation with conduction and convection.

c)	Derive an equation for conduction through a plane homogeneous wall whose K is given by K= +t.

2.a)	Explain graphical solution of un-steady state conduction.

b)	The temperature distribution across a wall of 1m thick at certain instant of time is given by t=900-300x-50x2 when t is in oC and x in m.  Determine (i) the rate of heat entering, leaving and stored (ii) the rate of change of temperature at x=0.2 and 0.8m.

Given that 	e=2250 kg/m3.

Cp=2.5x10-1 WH/kg/oC

K=0.8 W/moC.

3.a)	What is the significance of Grashaff number and Naisselt number in convection.

b)	What is the general basis of dimensional analysis used in connection?

c)	In a heat exchanger, water enters at 90oC and leaves at 50oC.  Another fluid enters at 30oC and leaves at 60oC determine log mean temperature difference for          (i) counter flow and (ii) parallel flow.

4.a)	What is emissivity?  Derive Kirchhoff’s law of radiation.

b)	Determine net radiation between the floor and roof of a furnace each 3x3m located 2m apart.  Emissivities of roof and floor are 0.7 and 0.9.  The floor and roof temperatures are 1400 and 900oC.

c)	Explain non-luminous flame.

5.a)	Describe characteristic features of electric arc furnace with a neat diagram.

b)	Explain heat losses in furnaces.

Contd…..2

Code No:NR211802				-2-				Set No:3

6.a)	What is Thomson’s effect?

b)	Explain thermo-electric power of thermo-couples.

c)	Explain cold junction corrections of thermo-couples.

d) 	The hot and cold junctions of a thermo-couple remain at constant temperatures.  If the resistance of couple and leads changes from 2 to 3 ohms, the emf across milli-voltmeter drops by 0.2%.  Determine the resistance of the milli-volt meter.

7.a)	What is Planck’s law of monochromatic radiation?

b)	Explain the principle, construction and use of dis-appearing filament optical pyrometer (Morse type).

c)	What is the effect of distance on optical pyrometer readings?

8.a)	Explain resistance measurement by wheat stone bridge.

b)	Explain principle and use of fixed focus radiation pyrometer (Foster pyrometer).

c)	In F and F optical pyrometer, a temperature of 1000oC requires a setting of 41o and 1400oC is balanced at 72o.  What position should be required for a temperature of 1200oC.

- - -

Code No:NR211802

Set No.

4

II B.Tech. I-Semester Supplementary Examinations, November-2003.

FURNACE TECHNOLOGY AND PYROMETRY

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What is the importance of heat transfer in Metallurgical furnaces?

b)	Compare conduction with convection.

c)	Derive an equation for conduction through homogeneous cylindrical wall.

2.a)	Explain graphical solution of un-steady state conduction.

b)	A furnace operating at 1400oC has wall of two layers 0.2m fire brick               (K=21x10-1 w/m/oC) and insulation 0.1m thickness (K=2.5x10-1 w/m/oC).  Outside wall temperature is 200oC.  Estimate the rate of heat loss and maximum temperature in the insulation.

3.a)	What is the significance of Reynolds number and Prandl number in convection.

b)	What are the main advantages of dimensional analysis in convection?

c)	Calculate the overall heat transfer co-efficient based on outer area for a condenser tube (K=600x10-1 w/m/oC) having outside and inside diameters of 0.03 and 0.02m.  Steam side and water side film co-efficients are 6 and 5 kw/m2/oC.

4.a)	What is white body?  Explain Stefan-Boltzmann’s law of total radiation.

b)	Two equal parallel discs of 1m dia are located 0.5m apart directly opposite each other.  The temperatures of the discs are 800 and 500oC.  Determine net radiant heat transfer.  F12=F21=0.38.

c)	What combined co-efficient of convection and radiation?

5.a)	Describe characteristic features of Cupola and explain its developments.

b)	Describe induction furnace.  Mention its advantages and draw backs.

6.a)	What is Peltier effect?

b)	Compare noble metal thermo-couples with base metal thermo-couples.

c)	Explain use of milli-voltmeter for measuring emf of a thermocouple.

d)	The resistance of thermo-couple and its leads is 3 ohms.  The hot junction remaining constant, the emf of the couple is read successively by a potentiometer and milli-voltmeter as 25 and 24.2 mV.  Calculate the resistance of the             milli-voltmeter.

Contd…..2

Code No:NR211802				-2-				Set No:4

7.a)	What are the reasons for using red filters in optical pyrometers over others?

b)	Compare Leeds and Northrup optical pyrometer with Fisher and Fery optical pyrometer.

c)	What is the effect of emissivity on optical pyrometer readings?

8.a)	Explain resistance measurement by potentiometer.

b)	Describe principle and use of Fery radiation pyrometer.

c)	A certain furnace has an apparent temperature of 1100oC.  When measured by an optical pyrometer sighted upon the furnace through a glass window.  If an additional window similar to the first is interposed, the apparent temperature is found to be 1060oC.  What is the true temperature of the furnace?

- - -