Code No:NR-210201

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

PRIME MOVERS AND PUMPS

(Electrical and Electronics Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain various types of draft tubes.

b)	Explain the working of a Kaplan turbine through a neat sketch.

2.a)	With a neat sketch explain the working principle of reciprocating pump.

b)	What are the component parts of reciprocating pump? Explain.

3.a)	Explain the different types of losses and efficiencies of centrifugal pump.

b)	What is Manometric efficiency of centrifugal pump?

4.a)	What are the essential differences between Carnot and Rankine steam engine cycles?

b)	 In an Ideal engine operating on the Carnot cycle, 20% of the heat input is converted into work. The efficiency gets doubled when the temperature of sink is reduced by 60 oC.  Estimate source and sink temperature.

5.a)	What are the two general categories of combustion engines. How do they differ in principle?

b)	What is meant by ‘valve timing diagram’? What useful information does it give?

6.a)	Define a boiler. What are the elements of a boiler and mention the application of it.

b)	How boilers are classified ? Give example of each classification.

7.a)	What is the principle of operation of simple impulse steam turbines?

b)	Show a diagrammatic view of a simple impulse steam turbine and explain its constructional features.

8.a)	Compare the working processes in reciprocating I.C engines and gas turbines.

b)	Explain the working principle of constant volume gas turbine.

$$$

Code No:NR-210201

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

PRIME MOVERS AND PUMPS

(Electrical and Electronics Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the performance and characteristic curves of Francis turbine.

b)	Explain the method of selection of turbines.

2.a)	Compare reciprocating pump and Centrifugal pump?

b) 	What is the maximum suction lift for centrifugal pump and why?

3.a)	Explain heads and efficiencies of a centrifugal pump.

b)	What is priming?  Explain its necessity?

4.a)	What is a cycle? What is the difference between an ideal and actual cycle?

b)	An air standard diesel cycle has a compression ratio of 14. The pressure at the beginning of compression stroke is 1 bar and the temperature is 27o C. If the maximum temperature of the cycle is 2500 o C.  Determine the efficiency of the engine working on this cycle.

5.a)	What is an internal combustion engine? How are they classified?

b)	 Describe the operating principle of a 4-stroke petrol engine. Draw the indicator

diagram.

6.a)	What is the difference between forced circulation and natural circulation  boilers ? Give examples of each type?

b)	Explain with the help of a neat sketch, the construction and working of Cornish 	boiler

7.a)	What is the difference between impulse turbine and reaction turbine?

b) 	Explain the working of a single stage impulse turbine.

8.a)	Discuss the relative advantages and disadvantages of reciprocating I.C engines and gas turbines.

b) 	With the aid of a schematic diagram, explain the working of a simple closed cycle

gas turbine.

$$$

Code No:NR-210201

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

PRIME MOVERS AND PUMPS

(Electrical and Electronics Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What do you mean by gross head and net head of a turbine?  Explain the different types of the efficiencies of a turbine.

b)	The following data is given for a Kaplan Turbine.  Determine the operating speed, unit power, unit speed and unit discharge.  Power = 15 MW, Head = 10 m.

2.a)	What is a pump?

b) 	What are its uses?

c) 	Write the different classification of pumps?

3.a)	Explain various efficiencies of a centrifugal pump.

b)	Sketch a centrifugal pump and explain the functions of its component parts.

4.a)	With the help of p- and T-s diagrams.  Explain the Otto cycle and derive its efficiency.

b)	The efficiency of an Otto cycle is 60 % and γ=1.5. What is the compression ratio?

5.a)	Classify I.C.engines with their applications.

b)	An engine develops 50 kW Brake Power. If its mechanical efficiency is 80%, what is the frictional power?

6.a)	Explain with the help of a neat sketch, the construction and working of Cochran boiler

b)	Discuss briefly the difference between high pressure and low pressure boilers.

7.a)	What is compounding? Describe various ways of compounding steam turbines.

b)	   List the advantages of a steam turbine over the conventional steam engine.

8.a)		Compare the advantages and disadvantages of gas turbines and steam turbines.

b) 	List the assumptions made in the analysis of ideal gas turbine cycle.

$$$

Code No:NR-210201

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

PRIME MOVERS AND PUMPS

(Electrical and Electronics Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the working principles and functions of different components of Kaplan turbine with neat sketch.

b)	A turbine develops 800 kW under a head of 25 m at 135 rpm.  Calculate the specific speed of the turbine and state the type of turbine.

2.a)	What is a reciprocating pump? Elaborate.

b)	A single acting reciprocating pump has a plunger diameter of 250mm and stroke of 350mm. If the speed of the pump is 60 rpm and it delivers 0.2m3/ sec of water. Find the theoretical discharge, coefficient of discharge and the slip.

3.a)	Why is the pump called centrifugal pump? How is the centrifugal action present inside the casing?

b)	What are the different types of the casing in centrifugal pump?

4.a)	Explain the cycle based on which a steam power plant works. Represent the cycle   	on T-S diagram.

b)  	Derive the efficiency of that cycle.

5.a)	What are the various applications of I.C.Engines

b)	Describe the working of two-stroke petrol engine.

6.a)	Describe with a neat diagram, the construction and working of a Babcock and Wilcox water tube boiler.

b)	What are the advantages of water tube boilers over fire tube boilers.

7.	Classify steam turbines according to

- the principle of action.

- the arrangement of pressure drop

- the direction of flow

- the number of cylinders.

8.	Show the schematic diagram of a gas turbine cycle with inter-cooled compression and explain how it is useful in increasing the specific work output.

$$$