Code No. 210251

OR

II B.Tech. I-Semester Supplementary Examinations, November-2003

# ELECTROMAGNETIC FIELDS

(Electrical and Electronics Engineering)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Derive an expression for the Electric field at a point on y-axis due to infinitely long thin filament whose line charge density is pL c/m when it is situated along z-axis.

b)	An infinitely long uniform line charge is located at y=2, z=4.  If L = 10nc/m, find E at the origin.

2.a)	Derive an expression for the capacitance of overhead lines.

b)	State and prove Gauss’s law. Obtain point form of Gauss’s law.

3.a)	Define dipole moment and polarization. Derive expression for the field due to dipole.

b)	Derive expression for the energy stored in Electrostatic field.

4.a)	State Boundary conditions at the conductor and dielectric interfaces.

b)	Find magnetic field at the center of a square loop of side 2m.

5.a)	Write Lorentz force equation. Obtain expression for the force between two current elements.

b)	Find the internal inductance per unit length of a cylindrical conductor of radius 2 meters.

6.a)	Explain the concept retarded potentials in detail.

b)	If the vector magnetic potential is given by A = (y cos kx) ax + (y + e2x)az find magnetic field at (0, 1, 2).

7.a)	State Maxwell’s equation in both differential and Integral form and explain them.

b)	Derive Wave equations for conducting medium.

c)	For silver,  = 3.0mho/m.  At what frequency will the depth of penetration be 1 mm?

8.	Write short notes on:

a) Poynting theorem

b) Ampere’s circuital law

c) Equation of continuity.

@@@@@