Code No: 210253

OR

II B.Tech. I-Semester Supplementary Examinations, November-2003

OPTIMIZATION TECHNIQUES

(Electrical and Electronics Engineering)

Time: 3hours								Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Give the statement of an optimization problem.  Explain design vector and desgin constraints.

b)	A rocket is designed to travel in outer space from one fuel depot to another which are separated by a distance of 10s.  The thrust of the rocket can be changed instantaneously after traveling a  distance of s.  If the maximum thrust that can be developed between the stages i and i+1 is Ci (i=1,2,3,……10), how should the thrust  of the rocket be controlled in order to traverse the total distance in minimum time?  Assume that the path of the rocket is a straight line and no external forces act on the rocket.

2.a)	Define “rth differential of f” and explain Tailor’s series expansion of  a multivariable function.

b)	If f(x) has an extreme point (maximum or minimum) at X=X* and if the first partial derivatives of f(x) exist at X*, then prove ∂f∂x1(X*)=∂f∂x2(X*)=……..=∂f∂xn(X*)=0.

3.	Find the dimensions of a cylindrical tin (with top and bottom) made up of sheet metal to maximize its volume such that the total surface area is equal to Ao=24.

4.	Minimize f(x)=12 x12+x22+x32 subject to

g1(x)=x1-x2=0

g2(x)=	x1 +  x2 +  x3 -1=0

by (i) direct substitution	 (ii) constrained variation,  and

(iii) Lagrange multiplier method.

5.a)	Minimize z = 4x + 5y

subject to   x + y  10

2x + 5y  35

x,y  0

Contd.2

Code No:210253				-2-				OR

- An animal feed company must produce 200 kg of a mixture consisting of ingredients x1 and x2  daily.   x1 costs Rs.3/kg and x2, Rs. 8/kg.  No more than 80kg of x1 can be used and atleast 60 kg of x2 must be used.  Formulate a linear programming model to minimize the cost.

6.	Solve the following transportation problem for minimization (unit costs are given in rupees)

Destination

| Origin | D1 | D2 | D3 | D4 | D5 | Supply |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | 4 | 3 | 1 | 2 | 6 | 40 |
| 02 | 5 | 2 | 3 | 4 | 5 | 30 |
| 03 | 3 | 5 | 6 | 3 | 2 | 20 |
| 04 | 2 | 4 | 4 | 5 | 3 | 10 |
| Demand | 30 | 30 | 15 | 20 | 5 |  |

7.	Explain in brief with examples.

(a)  North West corner rule

(b)  Vogel’s approximation method.

8.	Apply technique of Dinamic Programming  for the linear programming problem given below:

Maximize  z = 4x + 5y

Subject to  z  4; y  6

3x + 2y  18; x, y  0

&&&