Code No. 210351

OR

II B.Tech I-Semester Supplementary Examinations, November 2003

FLUID MECHANICS

(Common to Mechanical Engineering, Production Engineering and Mechatronics)

Time: 3 hours							Max. Marks: 70

Answer any Five questions

All question carry equal marks

---

1. a)	State the Newton’s Law of viscosity.  Differentiate between Newtonian and Non-Newtonian fluids with examples.

b)	A rectangular plate of 3 m wide and 5 m depth is immersed vertically in water.  Top edge of the plate is at a depth of 2 m from the free surface.  Determine the total pressure force and the centre of pressure.

2. a)	The velocity of a 3-D flow field is given by V= (y2 + z2) i + (x2 + z2) j + (x2 + y2)k

check whether the flow is possible or not.  Determine the velocity at a point       (1, 2, 3).

b)	Derive the continuity equation for a 3-D fluid flow.

3. a)	Briefly describe the growth of the boundary layer over a flat plate.

b)	Determine the displacement thickness of a boundary layer for the given velocity profile as υV = 2yδ − yδ2

4. a)	Derive the Bernoulli’s equation along a stream line.

b)	50 lit/sec of water flows in a horizontal pipe which is 200 m long and the centre of which is 3 m above datum line.  The pipe tapers from 30 cm diameter  to

20 cm.  If the pressure at one end of the pipe is 105 N/m2, calculate the pressure at the other end.  Neglect losses.

5. a)	A venturimeter with inlet area 100 cm2 and throat 25 cm2 is employed to measure discharge of carbon dioxide supplied at 2 bar and 290 K.  The U-tube upright differential manometer connected between inlet and throat shows 20 cm deflection of the water column.  Estimate the mass flow rate.

b)	Determine the sonic velocity in air at STP conditions.  How does it change with the altitude in the standard atmosphere?

6. a)	A glass tube of 4 mm diameter is dipped in mercury and in water.  The surface tension in mercury and water are 0.51 N/m and 0.0736 N/m respectively.  The angle of contact for mercury and water are 130o and 0o respectively.  Determine the capillary rise or fall in water and mercury.

b)	A triangular plate of 2 m base and 4 m high is vertically immersed in water.  The apex of the triangle is 3 m below the free surface.  Determine the hydrostatic force and centre of pressure.

Contd….2

Code No. 210351				.2.					OR

7. a)	Show that  = 1.73 y – x represents uniform flow with a velocity of 2 m/s at an angle of 30o to the X-axis.

b)	Define stream line, stream function, path line and streak line.

8. a)	Define momentum thickness and obtain the expression for the same.

b)	Pressures on the suction side and discharge side of a pump are read by the pressure gauges as

(i)	Vaccum of 25 cm of mercury and

(ii)	140 kN/m2 respectively.

The gauge on the suction side is placed 60 cm below the centre line of the pump.  Whereas the gauge on the discharge side is placed 17 cm above the centre line.  If the diameters of pipes on suction and discharge sides are 15 cm and 10 cm respectively and quantity of water to be pumped is 30 lit/ sec, calculate the power required for the pump.

###