Code No.210452

OR

II B.Tech. I-Semester Supplementary Examinations, November-2003

NETWORK THEORY

(Common to Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Bio-Medical Engineering and Electronics and Control Engineering)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain Theveniens theorem with example.

b)	Find the Norton equivalent for the network

c)	Find current is i1 the given circuit.

2.a)	State and explain Tellegen’s theorem.

b)	Verify Tellegen’s theorem for the given network.

(Contd…2)

Code No.210452				-2-				OR

c)	Determine the current through capacitor C1 by the principle of superposition in 	figure.

3.a)	Define resonance. Draw the impedance variation with frequency of series and 	parallel RLC circuits and explain.

b)	In a series RLC circuit with R = 100Ω, L = 0.5 H and C = 40  F, calculate the 	resonant, lower and upper half power frequencies.

4.a)	Define transient and steady state condition. In the series RL circuit the switch is 	closed in position 1 at t = 0 and at t = 500  sec, the switch is moved to position 2. 	Obtain the equation for current.

b)	In the network of the figure, the switch k is in position a for a long period of time. 	At t = 0, the switch is moved from a to b. Find 2(t). Assume that initial current in 	2H inductor is zero.

(Contd…3)

Code No.210452				-3-				OR

5.a)	Explain the principle of compensation theorem and give its advantage.

b)	In the network the resistance R is changed from 4Ω to 2 Ω.

Determine the compensation source VC and calculate the current through 8Ω 	resistor.

6.a)	Obtain the relationship between Z and h parameters.

b)	For the following network find Z11(S) G12(S), Y22(S), h21(S) and Z21(S).

7.a)	Explain necessary and sufficient conditions for a function to be positive real 	function.

b)	Synthesise the function in Foster first form.

Y(S)=(S+5)(S+7)(S+9)(S+6)(S+8)S.

8.a)	Discuss briefly image impedance and reactance matching.

b)	Define self inductance, mutual inductance and coefficient of coupling.

c)	For the given network, M12 = 0. Formulate the loop equations for this network 	using Kirchhoffes voltage law.

***     ***     ***