Code No: 210551

OR

II B.Tech., I-Semester Supplementary Examinations, November-2003

ELECTRONICS DEVICES AND CIRCUITS

(Computer Science and Engineering)

Time: 3hours								Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

----

1.a)	For an open circuit pn junction diode derive the expression for contact potential.

b)	Explain Hall effect.

2.a)	Explain the construction and operation of a Schottky diode.

b)	A zener diode rated 10V, 32 ma can be considered ideal.  i.e., Rz=0.  Calculate the range of RL and I2 for VL to be maintained constant.  What is the maximum wattage consumed by the diode.

3.a)	What are the transistor parameters that vary with temperature?  Explain.

b)	A P-channel FET has VP=4V and IDSS=12ma.  For the figure determine RD and RS so that ID=4 ma and VDS=6V.  VDD is 12 V.

Contd..2

Code No:210551				-2-				OR

4.a)	Explain why is it important to have low dark current for avalanche photodiode.

b)	An electron moving with initial velocity of 106 m/s enters an uniform magnetic field at an angle of 300 with it.  Calculate the magnetic flux  density required in order that the radius of the helical path be 1 m.  Also,  Calculate the time taken by the electron for one revolution  and pitch of the helix.

5.a)	Derive an expression for stability factor for a fixed bias circuit.

b)	Calculate the ripple voltage of FWR with 120f capacitor connected to a load of 60 ma.  Frequency 50 Hz.  Calculate (i) the dc voltage of the peak voltage is 60 V.

(ii) Ripple factor.

6.a)	Derive the equations for voltage gain, current gain input impedance and output impedance for BJT using low frequency h-parametric CE model.

b)	Discuss the effect of emitter bypass capacitor on low frequency response.

7.a)	Derive an expression for the acceleration attained by an electron kept in electric field of intensity E.

b)	Compare EMOSFET and DMOSFET

8.	Write notes on:

(a)  Photo transistor

(b) Tunnel diode

(c) Regulation characteristics in a power supply.

&&&