Code No. 210552

OR

II B.Tech. I-Semester Supplementary Examinations, November 2003

PRINCIPLES OF ELECTRICAL ENGINEERING AND NETWORKS

(Computer Science and Engineering)

Time: 3 hours							Max. Marks: 70

Answer any Five questions

All question carry equal marks

---

1.a)	Derive an expression for determination of RMS value of sine wave.

b)	A resultant current wave is made up of two components: direct current of 10A and sinusoidal alternating current of 50 Hz, with peak value of 10A.

(i)  Calculate the average value of current of the resultant wave over one complete cycle.

(ii)   Determine the rms value of current of the resultant wave.

2.a)	With suitable diagram, explain line and phase quantities in delta  connected system.

b)	Three inductive coils, each with a resistance of 15 ohm.  and  in inductance of 0.03H are connected in star to 3-phase 400V, 50Hz supply. Calculate (i) Phase current and line current and (ii) total power absorbed.

3.a)	Explain the constructional details of DC machines.

b)	Develop a general expression for the resistance of a direct current armature  winding with z-conductors each of the length L (including  overhang ) and cross section S arranged in a pairs of circuit and P pole pairs. The resistivity of the winding materials is P.

4.a)	Derive the torque equation of a dc motor.

b)	A shunt generator delivers 40 kw  at 240V when running at 450 rpm. The armature and field resistances are 0.03 ohm.  And 60 ohm respectively. Calculate the speed of the machine running as a shunt motor and taking 40 kw input at 240V. Allow 1V per brush for contact drop.

5.a)	Derive an equivalent circuit for a single phases transformer.

b)	A 6600/400 V , 50 Hz , single phase core type transformer has a net cross-sectional area of the core of 428 cm2. The maximum flux density in the core is 1.5 tesla. Calculate the number of tutns in a primary and secondary windings.

6.a)	Explain with the help of diagrams how a rotating magnetic field is produced in the air gap of a 3-phases induction motor.

b)	The voltage applied to the stator of a 3-phase, 6-pole induction motor has a frequency of 50 Hz. The frequency of the emf induced in the rotor is 2 Hz. What is the slip and at what speed is the motor running.

Contd…2.

Code No. 210552				-2-				OR

7.a)	State and explain the superposition theorem.

b)	Calculate the current in the 2 ohm. resistor of given below diagram using thevenin’s theorem.

8.	Write short notes on the following:

(a)    DC motors starter

(b)    Efficiency and regulation of single phase transformers

(c)    Maximum power transfer theorem

-x-.