Code No:211853

OR

II B.Tech. I-Semester Supplementary Examinations, November-2003.

FURNACE TECHNOLOGY AND PYROMETRY

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What is the importance of heat transfer to Metallurgical tings?

b)	Compare heat transfer by radiation with conduction and convection.

c)	Derive an equation for conduction through a plane homogeneous wall whose K is given by K= +t.

2.a)	Explain graphical solution of un-steady state conduction.

b)	The temperature distribution across a wall of 1m thick at certain instant of time is given by t=900-300x-50x2 when t is in oC and x in m.  Determine (i) the rate of heat entering, leaving and stored (ii) the rate of change of temperature at x=0.2 and 0.8m.

Given that 	e=2250 kg/m3.

Cp=2.5x10-1 WH/kg/oC

K=0.8 W/moC.

3.a)	What is the significance of Grashaff number and Naisselt number in convection.

b)	What is the general basis of dimensional analysis used in connection?

c)	In a heat exchanger, water enters at 90oC and leaves at 50oC.  Another fluid enters at 30oC and leaves at 60oC determine log mean temperature difference for          (i) counter flow and (ii) parallel flow.

4.a)	What is emissivity?  Derive Kirchhoff’s law of radiation.

b)	Determine net radiation between the floor and roof of a furnace each 3x3m located 2m apart.  Emissivities of roof and floor are 0.7 and 0.9.  The floor and roof temperatures are 1400 and 900oC.

c)	Explain non-luminous flame.

5.a)	Describe characteristic features of electric arc furnace with a neat diagram.

b)	Explain heat losses in furnaces.

Contd…..2

Code No:211853				-2-				OR

6.a)	What is Thomson’s effect?

b)	Explain thermo-electric power of thermo-couples.

c)	Explain cold junction corrections of thermo-couples.

d) 	The hot and cold junctions of a thermo-couple remain at constant temperatures.  If the resistance of couple and leads changes from 2 to 3 ohms, the emf across milli-voltmeter drops by 0.2%.  Determine the resistance of the milli-volt meter.

7.a)	What is Planck’s law of monochromatic radiation?

b)	Explain the principle, construction and use of dis-appearing filament optical pyrometer (Morse type).

c)	What is the effect of distance on optical pyrometer readings?

8.a)	Explain resistance measurement by wheat stone bridge.

b)	Explain principle and use of fixed focus radiation pyrometer (Foster pyrometer).

c)	In F and F optical pyrometer, a temperature of 1000oC requires a setting of 41o and 1400oC is balanced at 72o.  What position should be required for a temperature of 1200oC.

- - -