Code No: RR-210201

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

HYDRAULICS AND HYDRAULIC MACHINERY

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Define mass density, weight density, specific volume and specific gravity.

b)	A 30 cm diameter shaft revolves in a guide bearing of 60 cm long at 500 rpm.  If the oil film bearing is 0.13 mm and viscosity of oil is 0.05 Ns/m2, find the power absorbed.

2. a)	Briefly explain the classification of flows.

b)	What type of acceleration is to be expected if

- Streamlines are parallel and equidistant

- Streamlines are straight and converging

- Streamlines are curved but equispaced

(iv)	Streamlines are curved and converging

3.	A pump is installed in a pipeline, 5cm diameter carrying oil of specific gravity 0.80.  It returns oil to a 10cm diameter pipe at the same elevation with a pressure increase of 15 KN/m2.  The quantity of oil flowing in the pipeline is 0.1 m3/s.  The motor driving the pump delivers 3 KW to the pump shaft.  Calculate the loss of energy in the pump.

4. a)   	Explain pipes in series and parallel

b)	A rough pipe is of diameter 8cm.  The velocity at a point 3cm from wall is 30% more than the velocity at a point 1cm from pipe wall.  Determine the average height of the roughness.

5.	Explain the working principle of a Pitot Static Tube with a neat sketch?

6.	Using impulse-momentum, derive an expression for the force exerted by a moving jet of fluid on a stationary curved vane.

7.	(i) Derive an expression for the specific speed of a turbine

(ii) Explain similarity laws.

8.	Discuss the conditions under which ‘Cavitation’ and ‘negative slip’ occur and state why air vessels are used in reciprocating pumps.

###

Code No: RR-210201

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

HYDRAULICS AND HYDRAULIC MACHINERY

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	How does vapour pressure affect fluid flow?

b)	Find the torque required to rotate a shaft of diameter 50 mm at 1400 rpm concentrically with a sleeve 50.16 mm in diameter and 10 m long filled with oil.  Kinematic viscosity of oil is 0.94 x 10-4m2/s.  Specific gravity of oil is 0.85.

2. a)	Derive the equation for a streamline.

b)	Differentiate between

(i) Uniform flow and non-uniform flow   (ii) Steady flow and unsteady flow

(iii) Laminar flow and turbulent flow  (iv) Rotational flow and irrotational flow

3.	Water flows at the rate of 0.70 m3/s through a pipe transition, whose diameter changes from 100cm at inlet to 50cm at the outlet.  If the pressure intensity at the centerline of 100cm section is 9810 N/m2, what force will be required to produce the change in momentum of water as it passes through the transition?

4. a)   	Explain hydraulically smooth and rough turbulent flows

b)	A smooth pipe of diameter 80mm and 800m long carries water at the rate of 0.5m3/minute. Calculate the loss of head, wall shear stress and centerline velocity.  Assume kinematic viscosity of water as 0.015 stokes. Assume f = 0.0791(Re)14.

5.	Water flows in a 250mm pipe. Two pitot tubes are installed in the pipe, one on the centerline and the other 50mm from the centerline. If the velocities at the two points are 3m/s and 2m/s respectively calculate the reading on the differential mercury manometer connected to the two tubes.

6.	Derive equations for the force of impact of jet (a) on a normal flat vane moving in the direction of jet (b). On a series of normal flat vanes mounted on a wheel. The vane velocity is less than jet velocity in both cases.

7.	(i) What are the different unit quantities? Explain them and give their uses.

(ii)Explain the different characteristic curves for a hydraulic turbine.

8.	What is an indicator diagram of a reciprocating pump? Sketch the theoretical indicator diagram for a single acting-reciprocating pump not fitted with an air vessel. With the help of the diagram explain clearly the effect of acceleration and friction on both suction and delivery strokes.

###

Code No: RR-210201

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

HYDRAULICS AND HYDRAULIC MACHINERY

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Define absolute pressure and gauge pressure.  Derive the relationship between them.

b)	A skater weighing 800 N skates at 54 kmph on ice at 0oC.  The average skating area supporting him is 10 cm2 and the effective dynamic coefficient of friction between the skates and the ice is 0.02.  If there is actually a thin film of water between the skates and the ice, determine its average thickness.  Assume   μ of water at 0oC as 0.001 Ns/m2.

2. a)	Derive the continuity equation for one-dimensional flow.

b)	Distinguish between stream line, path line and streak line.

3. a)  	Water is pumped at the rate of 0.5 m3/s through a 30cm diameter pipe upto a hill top.  Hill top has an elevation of 50m, and the diameter of the pipe line reduces to 20cm.  If the pump maintains a pressure of 9810 N/m2 at hill top, what is the pressure at the foot of the hill?  What is the power required to pump the water.

b)	A pipe of 300mm diameter conveying 0.3 m3/s of water has a right angled bend in a horizontal plane.  Find the resultant force exerted on the bend if the pressure at inlet and outlet of the bend are 25 N/cm2 and 23 N/cm2.

4. a)   	Derive an expression to compute loss of head for sudden expansion

b)	Determine the wall shearing stress in a pipe of diameter 100mm which carries water.  The velocity at the pipe centre and 30mm, from the pipe centre are 2 m/s and 1.5 m/s respectively.  The flow in pipe is assumed to be turbulent.

5.	A pitot-static tube placed in the center of a 300mm pipe line, has one orifice pointing upstream and other perpendicular to it. If the pressure difference between the two orifices is 60mm of water when the discharge through the pipe is 1500 litres per minute, calculate the coefficient of pitot tube. Take mean velocity in the pipe to be 0.85 of the central velocity.

6.	A horizontal jet of water strikes a flat vane inclined to the jet at an angle . Obtain the components of the force of impact of jet in the direction of jet and normal to it if (a) the vane is stationary and (b) the vane moves in the direction of the jet with a certain velocity less than the jet velocity.

Contd….2

Code No. RR210201					.2.			Set No. 3

7.	a) What are similarities? Explain dynamic similarity

b) Define specific speed. Give its range for different turbines.

c) Draw velocity triangles for Pelton turbine at inlet and outlet. Derive the

expression for power developed.

8. a) 	Explain the term ‘slip’ and ‘cavitation’ with reference to reciprocating pumps

b)	Explain the functions of air vessels in a reciprocating pump.

###

Code No: RR-210201

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

HYDRAULICS AND HYDRAULIC MACHINERY

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Define specific weight, viscosity, surface tension and vapour pressure and mention their SI units.

b)	Space between two parallel plates kept 3 mm part is filled with an oil of viscosity 0.2 Ns/m2.  Find the shear stress at the lower plate when the upper plate is moving with a velocity of 1.5 m/s.

2. a) 	Classify the flows based on time and space criteria for the following cases.

(i)	A constant diameter pipe carrying constant discharge

(ii)	A uniform pipe bend carrying constant discharge

(iii)	A constant diameter pipe carrying varied discharge

(iv)	A converging pipe carrying constant discharge

(v)	A converging pipe carrying varied discharge

b)	Stream line, streak line and path line are identical in steady flows.  Explain.

3. 	A nozzle of diameter 20mm is fitted to a pipe of diameter 40mm.  Find the force exerted by the nozzle on the water which is flowing through the pipe at the rate of 1.2 m3/min.

4. a)   	Derive an expression to compute loss of head for sudden contraction

b)	Water is flowing through a rough pipe of diameter 500mm and length 4000m at the rate of 0.5 m3/s.  Find the power required to maintain this flow.  Take the average height of roughness as 0.40mm.

5.	A pitot tube is mounted on an airplane to indicate the relative speed of the plane. What differential pressure intensity will the instrument register when the plane is traveling at a speed of 220km/hr in a wind blowing at 40km/hr against the direction of motion of the plane? Take specific weight of air as 11.9N/m3.

6.	A jet of water moves smoothly over the surface of a curved vane. Analyse the forces acting on the vane and determine the resultant force in magnitude and direction. Assume shackles flow at entry and exit.

7.	A quarter scale turbine model is tested under a head of 12m. The full scale turbine is to work under a head of 30 m and to run at 428 rpm. Find the speed of the model.  If the model develops 100 kW and uses 1100 /s at this speed, what power will be obtained from full scale turbine assuming that the efficiency is 3% better than that of the model?

Contd…2

Code No. RR210201					.2.			Set No. 4

8. 	Explain the working principles of single acting and double acting reciprocating pumps with neat sketches.

###